SA WG2 Meeting #119	S2-170871

13-17 February 2017, Dubrovnik, Croatia	(was S2-17xxxx)

Source:	Sony

Title:	Fast Connection Set-up solution

Document for:	Approval

Agenda Item:	6.10

Work Item / Release:	FS-REAR/Rel-15

Abstract of the contribution: This solution proses a solution to speed up the PC5 connection set-up by utilizing end-to-end AS security.

# Discussion

According to 23.303

Observation 1: Remote UE can initiate Direct Communication if the Remote UE has knowledge about the Layer-2 ID

Observation 2: In ProSe R13 it is required to set-up a secure link after the Direct Communication Request have been sent to the Relay UE.

For enhancing the connection setup, we would like to find ways to optimize this procedure. In ProSe R13 the Remote UE and Relay UE have a PDCP connection and this can be used for AS encryption of the PC5-u. For Layer-2 relay, we have the PDCP connection between the eRemote UE and the eNodeB that will provide AS security protection for all links between the eRemote UE and the eNodeB.

Observation 3: There is an AS security relation between the eRemote UE and the eNodeB. This security relation can be used to protect the data sent over PC5.

Proposal 1: Utilize the AS security protection provided on PCDP level between eRemote UE and eNodeB to provide a secure PC5 link.

According to TR33.899 study in SA3, SA3 is looking into how to authenticate remote UE via relay UE, even though this study has not concluded it could be valuable to see if any solutions in TR33.899 can be valid for REAR.

The procedure above focus on the mutual authentication UE-Network, however it would be interesting to use some of the concepts. We see that the Remote UE generates a Remote UE Indicator (RUE_indicator). We see that the Relay UE can identify and verify the message from the Remote UE by looking at the RUE_indicator. If the RUE_indicator is valid the Relay will forward the message to the network.

Observation 4: The RUE_Indicator can be used to validate that the message is coming from an authenticated and authorized Remote UE.

Proposal 2: Use the SA3 concept of RUE_indicator to verify any messages send by the eRemote UE to the eRelay is coming from an authenticated and authorized Remote UE.

# Conclusion

The following observation and proposals shall be considered.

Observation 1: Remote UE can initiate Direct Communication if the Remote UE has knowledge about the Layer-2 ID

Observation 2: In ProSe R13 it is required to setup a secure link after the Direct Communication Request have been sent to the Relay UE.

Observation 3: There is an AS security relation between the eRemote UE and the eNodeB. This security relation can be used to protect the data sent over PC5.

Proposal 1: Utilize the AS security protection provided on PCDP level between eRemote UE and eNodeB to provide a secure PC5 link.

Observation 4: The RUE_Indicator can be used to validate that the message is coming from an authenticated and authorized Remote UE.

Proposal 2: Use the SA3 concept of RUE_indicator to verify any messages send by the eRemote UE to the eRelay is coming from an authenticated and authorized Remote UE.

# Proposal

We propose to update TR 23.733 with a solution reflecting above observations and proposals.

* * * Start of changes * * * *

## 6.y	Solutions to key issue 3

### 6.y.x	Solution X: Fast Connectionn setup by optimizing eRemote UE security procedures

#### 6.y.x.1	Description

#### This solution proposes a way to optimize the connection setup between the eRemote UE connection and the network in order to facilitate fast connection setup. In TS 23.303 clause 5.4.5 the procedure is described how to set up One-to-one ProSe Direct Communication. It involve two steps. First, UE-1 sends a Direct Communication Request message to UE-2. The message then triggers step two, the mutual authentication and security association between UE-1 and UE-2.

#### This solution for Layer-2 relay proposes to utilize the fact that for traffic using indirect 3GPP communication between the eRemote UE and eNodeB, the traffic is security protected by legacy AS security procedures. This protection can be used to provide a secure PC5 link. The optimization proposed for setting up One-to-one communication between an eRemote UE and an eRelay UE, is to omit the step 2 in TS23.303 clause 5.4.5, since the PC5 is secured by the AS security between eRemote and eNodeB.

#### Figure 6.y.x.1-1: eRemote UE initiated Connection Resume procedure

- The eRemote UE and eRelay UE have performed initial attach to the network and performed Service Authorization for ProSe Discovery, ProSe Direct Communication and indirect 3GPP Communication. In case the eRemote UE and eRelay UE have prior association (e.g. are aware of each other’s Layer2-IDs), then the eRemote can directly communicate with the eRelay UE. If the eRelay’s Layer-2 ID is unknown to the eRemote UE, then the eRemote UE can acquire Layer-2 ID by e.g. execute a discovery procedure.

- eRemote UE generates an eRemote UE RRC Resume Request message including an Remote UE indicator (RUE_Indicator) that indicates the message is from a eRemote UE. The eRemote UE RRC Resume Request message includes information needed in the eNodeB to access the stored AS context see TS 36.300. The eRemote send the message to the eRelay UE over PC5.

NOTE1: Similar to the RRC Resume message specified in 23.401 clause 5.3.5A, The eRemote RRC Resume Request message will be security protected using the stored AS context in the eRemote UE.

#### Editor’s note: It is FFS how the RUE_Indicator is generated. Alignment with SA3is needed.

- The eRemote UE send the eRemote UE RRC Resume Request message to the eRelay UE

#### Upon receiving the message from the eRemote UE, the eRelay UE identifies and verifies the RUE indicator and forwards the eRemote UE RRC Resume Request message to the eNodeB.

Editor’s note: It is FFS how the RUE_Indicator is verified. Alignment with SA3is needed.

- eNodeB and eRemote UE resumes the RRC Connection (e.g. using a RRC Connection Resume procedure according to 23.401 clause 5.3.5A. The PC5 will be security protected by the AS encryption (on PDCP layer) between eNB and eRemote.

#### The eNodeB notifies the MME that the UE's RRC connection is resumed in the S1-AP UE Context Resume Request message, which includes an RRC resume cause. The MME enters the ECM-CONNECTED state. The MME identifies that the UE returns at the eNodeB for which MME has stored data related to the S1AP association, UE Context and bearer context including the DL TEID(s), necessary to resume the connection.

#### MME acknowledges the connection resumption in S1-AP UE Context Resume Response message.

#### The uplink data from the UE can now be forwarded by eRelay to the eNodeB and further forwarded by the eNodeB to the PDN GW.

#### 6.1.1.2	Impact analysis

SA3 enhancements needed to enable the generation and verification of a Remote UE Indicator (RUE_Indicator)

#### 6.1.1.3	Evaluation

* * * End of changes * * * *