SA WG2 Meeting #119	S2-171055

13 - 17 February 2017, Dubrovnik, Croatia	(revision of S2-17xxxx)

Source:	CATT

Title:	TS 23.501: Update to UE reachablity management

Document for:	Discussion / Approval

Agenda Item:	6.5.7

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to add registration area management into TS 23.501.

- Discussion

Update to MO only mode

For MO only mode UE, it will not be tracked by the network, thus no registration area needs to be allocated. However, the network may decide to apply mobility restriction to this UE, in this case, the AMF needs to provide an allowed area or a non-allowed area to the UE. As we agreed that the allowed/non-allowed area is carried in a TA list, the MO only UE can also receive a TA list which is not for tracking purpose. Therefore, we clarify that:

“When the AMF applies MO only mode to a UE, no registration area, i.e. a TA list, needs to be allocated for paging purpose, but if mobility restriction needs to be executed by the UE, a TA list indicating an allowed/non-allowed area shall be assigned.”

Clarification on UE reachability in CM-CONNECTED

As RAN level paging will be introduced into 5GS, when it is activated, even for a UE in CM-CONNECTED state, the AMF can not determine whether it is reachable. Hence we add a new clause to clarify it.

2.	Proposal

According to the discussion above, we would like to propose to add the following texts in TS 23.501

* * * * Start of 1st Change * * * *

5.4.1	UE reachability in CM-IDLE

5.4.1.1	General

The UE and the AMF negotiate UE reachability characteristics in CM-IDLE state during registration and registration update procedures.

There are two UE reachability categories in CM-IDLE negotiation:

1.	UE reachability allowing Mobile Terminated data while the UE is CM-IDLE mode.

-	The UE location is known by the network on a Tracking Area List granularity

-	Paging procedures apply to this category.

-	Mobile originating and mobile terminated data apply in this category for both CM-CONNECTED and CM-IDLE mode.

2.	Mobile Originated only (MO only) mode

-	Mobile originated data applies in this category for both CM-CONNECTED and CM-IDLE mode.

-	Mobile terminated data is not supported when the UE is in CM-IDLE mode, it is only supported when the UE is in CM-CONNECTED mode.

Editor's note:	The final name of MO only mode is FFS.

* * * * Next Change * * * *

5.4.1.3	Mobile Originated only (MO only) mode

Editor's note:	The final name of MO only mode is FFS.

A UE may indicate preference for MO only mode during registration and/or registration update. The AMF, based on subscription, local configuration based on UE indicated preferences, UE subscription information and network policies, or any combination of them, determines whether MO only mode is applied for the UE and indicates it to the UE during registration signalling.

The UE and core network re-initiates or exits the MO only mode at subsequent registration signalling. MO only mode needs to be applied explicitly in every registration and registration update.

When the AMF applies MO only mode to a UE, no registration area, i.e. a TA list, needs to be allocated for paging purpose, but if mobility restriction needs to be executed by the UE, a TA list indicating an allowed/non-allowed area shall be assigned.

When the AMF applies MO only mode to a UE, the AMF considers the UE always unreachable while in CM-IDLE. The UE in MO only mode is only reachable for mobile terminated data or signalling when the UE is in CM-CONNECTED mode for the PDU sessions that are resumed. The CN rejects any request for downlink data delivery for an MO only UE in idle mode. The CN also does not allow downlink transport over NAS for SMS, location services, etc.

A UE in MO only mode performs a registration update upon the expiry of a periodic registration timer.

A UE in MO only mode does not listen to paging while in CM-IDLE. A UE in MO only mode may stop any access stratum procedures in CM-IDLE, until the UE initiates CM-IDLE to CM-CONNECTED mode procedures due to one of the following triggers:

-	A change in the UE (e.g. change in configuration) requires an update its registration with the network.

-	Periodic registration timer expires

-	MO data pending

-	MO signalling pending (e.g. SM procedure initiated)

-	Moving out of the allowed/non-allowed area assigned by the AMF

* * * * Next Change * * * *

5.4.2	UE reachability in CM-CONNECTED

For a UE in CM-CONNECTED state, the AMF knows the UE location is on a serving (R)AN node granularity, and considers the UE is reachable for signalling or data transfer.

Editor's note:	When RAN level paging is activated, the UE reachability is further determined by the RAN node.

* * * * End of Changes * * * *