SA WG2 Meeting #119	S2-171078

13 - 17 Feb 2017   Dubrovnik, Croatia	(revision of S2-17xxxx)

Source:	Samsung

Title:	description for UE preference

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Propose to capture UE preference description in the procedure.

# Discussion

Analysis on TR conclusion

1.	Based on UE indicated preferences, UE subscription data and network policies, or any combination of them, the NG CN determines whether MO only mode is applied for the UE and indicates it to the UE during registration signalling. The UE and core network re-initiates (or exits) the MO only mode at subsequent registration signalling.

(…)

5.	If the UE needs to be de-registered at the end of CN-Connected mode, the UE can provide the preference e.g.:

-	CN-IDLE mode is required, or CN-IDLE mode is not required.

Also in the TS 23.501 v0.1.1,

### 5.4.1	UE reachability in CM-IDLE

#### 5.4.1.1	General

The UE and the AMF negotiate UE reachability characteristics in CM-IDLE state during registration and registration update procedures.

Based on this high-level description captured in the TS 23.501, it is proposed to address UE indicated preference in the procedure section.

Proposal 1: It is proposed to capture UE indicated preference in the general section of registration procedure.

# Proposal

Add the following to the TS 23.502.

*************** Start of changes *********************

## 4.2	Connection, Registration and Mobility Management procedures

Editor's note:	Should include mobility management related procedures.

### 4.2.1	General

### 4.2.2	Registration procedures

#### 4.2.2.1	General

#### 4.2.2.2	Registration

Editor's note:	Procedure equivalent to Attach procedure and TAU (for all type of triggers) procedure. Procedure includes aspects required to support network slicing, and policy control. Procedure may include e.g. authentication/authorisation, RRM, Capability handling, DRX aspects. Network sharing is considered as part of the procedure description.

##### 4.2.2.2.1	General

A UE needs to register with the network to get authorized to receive services, to enable mobility tracking and to enable reachability. The Registration procedure is used e.g. when the UE needs to initially register to the 5G system, upon mobility procedure when the UE changes to a new Tracking area (TA) in idle mode and when the UE performs a periodic update (due to a predefined time period of inactivity), etc.

Editor's note:	It is FFS whether to add a reference to TS 23.501 for a complete list of triggers for initiating the registration procedure.

Editor's note:	Whether the procedure is used in CM-CONNECTED state is FFS.

Editor's note:	The procedure description based on the NF services will be documented once the applicable components are identified.

Editor's note:	Further details due to network slicing is FFS.Editor's note:	Aspects related to dual registration in 3GPP and non-3GPP access is FFS.

Editor's note:	Additions of a new clause for redirection due to network slicing is FFS.

Editor's note:	The identities needs to be aligned with the agreed identities.

Editor's note:	Enhancements of the normative wording (i.e. adding shall/should/may) is FFS.

During the initial registration the Permanent Equipment Identifier is obtained from the UE. The AMF operator may check the PEI with an EIR. The AMF passes the PEI (IMEISV) to the UDM, to the SMF and the PCF.

A UE can indicate UE preference for mobility management support during the registration procedure. The UE preference for mobility management can include one or more of following information:

-	Paging Off

-	CM-IDLE Off

Editor’s note: Above list is not exhausted and can be further updated to reflect normative agreement.

If Paging Off is requested by the UE, the AMF determines whether Mobile On-demands mode is applied to the UE. If the AMF allows the UE to use Mobile On-demands mode, the AMF includes Paging Off indication in the registration accept. If Paging Off is not provided from the UE, by default, the UE and the AMF consider paging is supported.

If CM-IDLE Off is requested by the UE, the AMF determines whether implicit deregistration at the end of CM-CONNECTED mode is applied to the UE. If the AMF allows the UE to perform implicit deregistration at the end of CM-CONNECTED mode, the AMF includes CM-IDLE Off indication in the registration accept. If CM-IDLE Off is not provided from the UE, by default, the UE and the AMF consider CM-IDLE state is supported.

*************** End of changes *********************