SA WG2 Meeting #119	S2-171527

13 - 17 February 2017, Dubrovnik, Croatia	(revision of S2-171495)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, Samsung, Huawei

Title:	23.501 § 8.3: User plane protocol stack between the UE and the DN

Document for:	Approval

Agenda Item:	6.5.3

Work Item / Release:	5GS_Ph1  / Rel-15

Abstract of the contribution: This contribution proposes the protocol stack between the UE and the DN.

# Introduction

Void

Changes:

-	UDP/IP: These are the backbone network protocols.

NOTE3:  Co-location of the UL CL or Branching Point with a PDU session anchor is a deployment option.

To avoid P-CR clash, corresponding modifications to § 8.3.x User Plane for untrusted non 3GPP Access are done in updates of Tdoc 0893 (E//)

# Proposal

It is proposed to update TS 23.501 “System Architecture for 5G System”.

* * * Start of changes* * * * all text new

## 8.3	User Plane Protocol stacks

.

#### 8.3.1	User Plane Protocol stack for a PDU session

This clause illustrates the protocol stack for the User plane transport related with a PDU session.

Figure 8.3.1-1: User Plane Protocol Stack

Legend:

-	PDU layer: This layer corresponds to the PDU carried between the UE and the DN over the PDU session. When the PDU session Type is IPV6, it corresponds to IPv6 packets ; When the PDU session Type is Ethernet, it corresponds to Ethernet frames ; etc….

-	5G Encapsulation: This layer supports multiplexing traffic of different PDU sessions (possibly corresponding to different PDU session Types) over N3 (i.e. between AN and 5G Core) or over N9 (i.e. between different UPF of the 5G Core). It provides encapsulation on a per PDU session level. This layer carries also the marking associated with a QoS Flow defined in clause 5.7

Editor’s notes: The details of encapsulation header should be updated according to the protocol chose/defined in stage 3.

-	AN protocol stack: This set of protocols/layers depends on the AN

-	When the AN is a 3GPP RAN, these protocols/layers are defined by the 3GPP RAN

Editor’s notes: The details of RAN protocol stacks depend on the decision of RAN WG and will be updated later.

- When the AN is an Untrusted non 3GPP access to 5GC the 5G AN interfaces with the 5GC at a N3IWF defined in clause 4.3.2 and the AN protocol stack is defined in clause 8.3.3

-	UDP/IP: These are the backbone network protocols.

NOTE1:  	The number of UPF in the data path is not constrained by 3GPP specifications: there may be in the data path of a PDU session 0, 1 or multiple UPF that do not support a PDU session anchor functionality for this PDU session.. For the IP type PDU session, the UPF acting as PDU session anchor is the IP anchor point of the IP address/prefix allocated to the UE.

NOTE 2: 	The “non PDU anchor session” UPF depicted in the Figure 8.3.1-1 is optional

NOTE3:  The N9 interface may be intra-PLMN or inter PLMN (in case of Home Routed deployment).

In case there is an UL CL (Uplink Classifier) or a Branching Point (both defined in clause 5.6.4) in the data path of a PDU session, the UL CL or Branching Point acts as the non PDU session anchor UPF of Figure 8.3.1-1. In that case there are multiple N9 interfaces branching out of the UL CL / Branching Point each leading to different PDU session anchors.

NOTE4:  Co-location of the UL CL or Branching Point with a PDU session anchor is a deployment option.

* * * End of Changes * * *