SA WG2 Meeting #120	S2-171696

27-31 March 2017, Busan, Korea	(was S2-17xxxx)

Source:	AT&T, Telecom Italia, Huawei

Title:	NWDA Function Definition

Document for:	Approval

Agenda Item:	6.5.6

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution propose modification of existing NWDA related text to create a section on NWDA

# 1	Introduction

Currently NWDA is not defined anywhere but mentioned as part of description of an interface. This contribution proposes modification of existing NWDA related text for clean-up and to create a new subsection similar to a subsection of 6.2 of TS 23.203

.

# 2	Proposed Updates

********************************************************************************

Start 1st Change

********************************************************************************

### A.2.2.6	N23 reference point

The N23 reference point resides between the Network Data Analytics (NWDA) and the PCF. The N23 reference point, enables the PCF to subscribe to and be notified on network status analytics e.g. congestion information for a specific slice.

Editor's note:	The actual parameter notified to the PCF is FFS.

Editor's note:	Applicability of slicing in PCF architecture is FFS.

********************************************************************************

End 1st Change

********************************************************************************

********************************************************************************

Start 2nd Change

********************************************************************************

### A.2.1.1	Network Data Analytics (NWDA)

A.2.1.1.1 General

NWDA represents operator managed network analytics logical function. It provides slice specific network data analytics to the PCF. The subscription to NWDA by PCF  is on a network slice level and the NWDA is not required to be aware of the current subscribers using the slice. NWDA can however collect long term UE/subscriber specific Analytics. Since the PCF is aware of UEs/subscribers using the slice it can request such information from NWDA if needed. NWDA notifies / publishes analytic information to the PCF(s) that are subscribed to it. PCF uses that data in its policy decisions. The PCF, the slice and NWDA in question are always in the serving network.

NOTE:  NWDA functionality beyond its support for N23 is out of scope of 3GPP.

********************************************************************************

End 2nd Change

********************************************************************************