SA WG2 Meeting #120	S2-172173

27 – 31 March 2017, Busan, Korea	(revision of S2-17xxxx)

Source:	ETRI

Title:	TS 23.502: P-CR to add N2 Release procedure for Non-3GPP

Document for:	Approval

Agenda Item:	6.5.10

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution proposes N2 Release procedure for non-3GPP access in TS 23.502.

Introduction

It is proposed to include the section 4.12.x N2 release procedure via untrusted non-3GPP access in TS 23.502 to be aligned with N2 release procedure for 3GPP access in in the clause 4.8.x.

Proposal

It is proposed to add the following texts into TS 23.502.

* * * Start of changes* * * *

4.12.x	N2 release procedure via Untrusted non-3GPP access

This procedure is used to release the logical N2 signalling connection (over N2) and all N3 connections (in N3) for a UE via untrusted non-3GPP access. This procedure will move the UE from CM-CONNECTED to CM-IDLE in both the UE and AMF, and all UE related context information is deleted in the N3IWF. When the N2 signalling connection is lost, e.g. due to loss of the signalling transport or because of an N3IWF or AMF failure, the N2 release procedure is performed locally by the N3IWF and by the AMF. When the N2 release procedure is performed locally by the N3IWF or by the AMF, each node performs locally its actions as described in the procedure flow below without using or relying on any of the signalling shown directly between N3IWF and AMF.

The initiation of N2 Release procedure is either:

-	N3IWF-initiated with cause e.g. Rekeying of IKE SA failure, UE unreachable, Administration reason, User inactivity, etc.; or

-	AMF initiated with cause e.g. Administration reason, etc.

Both N3IWF-initiated and AMF-initiated N2 release procedures are shown in Figure 4.12.x-1.

Figure 4.12.x-1: N2 Release Procedure via Untrusted non-3GPP access

1.	The UE has already connected to the 5G network as CM-CONNECTED and may already establish one or multiple PDU sessions.

2a.	The N3IWF may release the IKEv2 tunnel locally if the N3IWF detects the UE is not alive based on the liveness check mechanism as specified in RFC 7296 [x].

2b.	If the N3IWF detects a need to release the UE's signalling connection and all access resources for the UE, the N3IWF sends an N2 UE Context Release Request (Cause) message to the AMF. Cause indicates the reason for the release.

NOTE 1:	Step 2 is only performed when the N3IWF-initiated N2 release procedure is considered. Step 2 is not performed and the procedure starts with Step 3 when the AMF-initiated N2 release procedure is considered.

3. 	This step is part of N2 release procedure in the Figure 4.8.x, Step x to y. If the UE has no activated PDU session, then this the step is not executed.

4.	The AMF releases N2 by sending the N2 UE Context Release Command (Cause) message to the N3IWF that triggers the N3IWF to release all UE related context.

5a.	If the IKEv2 tunnel is not released in step 2, the N3IWF sends INFORMATIONAL Request (Delete payload) message to the UE. Delete payload is included to indicate the release of the IKE SA and any child IPSec SA(s) if existing.

Editor's note: It is FFS whether an IKE SA for liveness check may be maintained in case that Dead Peer Detection (DPD) as specified in RFC 7295 [x] is performed to track UE reachability while the UE is in CM-IDLE.

5b.	The UE sends an empty INFORMATIONAL Response message to acknowledge the release of the IKE SA as described in RFC 7296 [y].

6.	The N3IWF confirms the N2 Release by returning an N2 UE Context Release Complete message to the AMF. With this, the signalling connection between the AMF and the N3IWF for that UE is released.

* * * End of Changes * * *