SA WG2 Meeting #120	S2-171838

26 - 31 March 2017, Busan, Korea	(revision of S2-17xxxx)

Source:	China Mobile

Title:	Clarifications for 5GS to EPC handover procedure using Nx interface

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_Ph1/ Rel15

Abstract of the contribution: This contribution proposes the following changes to TS23.502.

- The Nx based 5GS to 4G handover procedure is only applicable to PDU sessions with SSC Mode 1.

- It is up to the AMF function to allocate the EPS Bearer IDs for any UE served by it.

- The PGW-C+SMF conducts the EPS QoS parameters mapping and allocates TFT filters with the PCC rules obtained from PCRF+PCF.

# 1	Discussion

Section 4.11.1.1 of TS 23.502 describes Nx interface based 5GS to 4G handover procedure. During the handover procedure, the assigned user plane anchor is to be kept unchanged during the lifetime of the PDU session regardless of the access technology the UE is using to provide seamless session continuity. Similarly, for PDU session of SSC mode 1, the UPF acting as PDU session anchor at the establishment of the PDU session is maintained during the lifetime of the PDU session. However, for PDU session of SSC mode2 and 3, a new user plane anchor will be established taking place of the original user plane anchor, so the service continuity is more likely to rely on the implementation of application layer. Therefore, it is proposed that the SSC mode 2 and 3 are not applicable to the 5GS to 4G handover procedure using Nx interface in this release.

The AMF network function has the overall view towards any individual UE which is served by it. So In comparison with other network function, the AMF is more suitable to allocate the EPS Bearer IDs for any UE. During PDU session establishment and GBR QoS flow establishment, EPS Bearer IDs for default bearers and dedicated bearers are allocated by the AMF respectively, non-GBR flows are mapped to default EPS bearer and the GBR flows are mapped to dedicated bearers in EPC. The EPS Bearer Ids are to be provided to the UE and PGW-C+SMF through the AMF.

If PCC is deployed, the PCR+PCRF provides the PCC rules to the PGW-C+SMF which is proposed to conduct EPS QoS parameters mapping and allocate TFT filters during the course of the PDU session establishment and GBR QoS flow establishment, otherwise the PGW-C+SMF will conduct EPS QoS parameters mapping and allocate TFT filters based on the local policies.

# 2	 Proposal

It is proposed to adopt the following in TS 23.502.

* * * * Begin First Change * * * *

4.11.1	Handover procedures for single-registration mode

4.11.1.1	5GS to EPS handover using Nx interface

Nx interface is used to provide seamless session continuity for single registration mode. Figure 4.11.1.1-1 describes the handover procedure from 5GS to EPS when Nx is supported. In this release, this flow is only applicable to PDU session with SSC Mode 1.

Figure 4.11.1.1-1: 5GS to EPS handover for single-registration mode with Nx interface

The procedure involves a handover to EPC and setup of default EPS bearer and dedicated bearers for GBR QoS flows in EPC in steps 1-16 and re-activation, if required, of dedicated EPS bearers for non-GBR QoS flows in step 17.

UE has one or more ongoing PDU sessions each including one or more QoS flows. During PDU session establishment and GBR QoS flow establishment, PGW-C+SMF performs EPS QoS mappings and allocate TFT with the PCC rules obtained from the PCR+PCRF if PCC is deployed, otherwise EPS QoS mappings and TFT allocation are executed by the PGW-C+SMF locally, and EPS Bearer IDs are allocated by the serving AMF such that non-GBR flows map to default EPS bearer (which is allocated an EPS bearer ID) and EPS Bearer IDs are allocated for the GBR flows that are mapped to dedicated bearers in EPC. The EPS Bearer Id and mapped QoS parameters for these bearers are also provided to the UE and PGW-C+SMF. The mapped EPS QoS parameters may be provided to PGW-C+SMF by the PCF+PCRF, if PCC is deployed.

Editor's note:	How TFT is created/allocated is FFS.

1.	5G RAN decides that the UE should be handed over to the E-UTRAN. The 5G RAN sends a Handover Required (Target eNB ID, Source to Target Transparent Container) message to the AMF.

2.	The AMF determines from the 'Target eNB Identifier' IE that the type of handover is Handover to E-UTRAN. The AMF requests the PGW-C+SMF to provide SM Context that also includes the mapped EPS Bearer Contexts. This step is performed with all PGW-C+SMFs allocated to the UE.

Editor's note:	This step should be aligned with intra 5GC inter-AMF handover.

NOTE:	In roaming scenario, the UE's SM EPS Contexts are obtained from the V-SMF.

3.	The AMF selects an MME and sends a Relocation Request (Target E-UTRAN Node ID, Source to Target Transparent Container, mapped MM and SM EPS UE Context (default and dedicated GBR bearers)) message. The SGW address and TEID for both the control-plane or EPS bearers in the message are such that target MME selects a new SGW.

4.	The MME selects the Serving GW and sends a Create Session Request message for each PDN connection to the Serving GW.

5.	The Serving GW allocates its local resources and returns them in a Create Session Response message to the MME.

6.	The MME requests the target eNodeB to establish the bearer(s) by sending the message Handover Request message. This message also contains a list of EPS Bearer IDs that need to be setup.

7.	The target eNB allocates the requested resources and returns the applicable parameters to the target MME in the message Handover Request Acknowledge (Target to Source Transparent Container, EPS Bearers setup list, EPS Bearers failed to setup list).

8.	If the MME decides that indirect forwarding applies, it sends a Create Indirect Data Forwarding Tunnel Request message (Target eNB Address, TEID(s) for DL data forwarding) to the Serving GW. The Serving GW returns a Create Indirect Data Forwarding Tunnel Response (Cause, Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding) message to the target MME.

9.	The MME sends the message Relocation Response (Cause, List of Set Up RABs, EPS Bearers setup list, MME Tunnel Endpoint Identifier for Control Plane, RAN Cause, MME Address for control plane, Target to Source Transparent Container, Address(es) and TEID(s) for Data Forwarding).

10.	If indirect forwarding applies, the AMF forwards to the PGW-C+SMF the information related to data forwarding to the SGW. The PGW-C+SMF returns a Create Indirect Data Forwarding Tunnel Response.

11.	The AMF sends the Handover Command to the source 5G RAN. The source 5G RAN commands the UE to handover to the target access network by sending the HO Command. This message includes a transparent container including radio aspect parameters that the target eNB has set-up in the preparation phase. The UE correlates the ongoing QoS flows with the indicated EPS Bearer IDs to be setup in the HO command. UE locally deletes the QoS flows that do not have an EPS bearer ID assigned.

12.	When the UE has successfully accessed the target eNodeB, the target eNodeB informs the target MME by sending the message Handover Notify.

13.	The target MME informs the Serving GW that the MME is responsible for all the bearers the UE have established by sending the Modify Bearer Request message for each PDN connection.

The target MME releases the non-accepted EPS Bearer contexts by triggering the Bearer Context deactivation procedure. If the Serving GW receives a DL packet for a non-accepted bearer, the Serving GW drops the DL packet and does not send a Downlink Data Notification to the SGSN.

14.	The Serving GW informs the PGW-C+SMF of the relocation by sending the Modify Bearer Request message for each PDN connection. The PGW locally deletes the QoS flows that do not have an EPS bearer ID assigned. Due to the "match all" filter in the default QoS flow, the PGW maps the IP flows of the deleted QoS flows to the default QoS flow.

15.	The PGW-C+SMF acknowledges the Modify Bearer Request. At this stage the user plane path is established for the default bearer and the dedicated GBR bearers between the UE, target eNodeB, Serving GW and the PGW+SMF.

16.	The Serving GW acknowledges the user plane switch to the MME via the message Modify Bearer Response.

17.	The PGW-C+SMF initiates dedicated bearer activation procedure for non-GBR QoS flows by mapping the parameters of the non-GBR flows to EPC QoS parameters. This setup may be triggered by the PCRF+PCF which may also provide the mapped QoS parameters, if PCC is deployed. This procedure is specified in TS 23.401 [x], clause 5.4.1.

* * * * End First Change * * * *