3GPP TSG SA WG2 Meeting #120	S2-172068

27 - 31 March 2017, Busan	(revision of S2-17xxxx)

Source:	Huawei, HiSilicon

Title:	TS 23.502: Providing S-NSSAI to the RAN and UE during PDU session establishment

Document for:	Discussion and approval

Agenda Item:	6.5.3

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution updates the session establishment procedure return S-NSSAI to RAN and UE.

# Discussion

## 1.1 Providing S-NSSAI to UE

If network slice selection policy (NSSP) includes a default rule which matches all applications to an S-NSSAI (i.e. a default S-NSSAI), then when UE requests a new PDU session for an application, there is always an S-NSSAI which is associated with the application or a default S-NSSAI included in request.

However, if NSSP doesn’t include a default rule which matches all applications to a default S-NSSAI, for some types of applications, there may be no S-NSSAI included in the PDU session establishment request from UE. In this case, the CN need to provide the default S-NSSAI associated with the established session to the UE. Then the UE can routes the user data of other applications which are also associated with the S-NSSAI of this PDU established session.

Proposal 1: The PDU session establishment procedure should include providing the S-NSSAI associated with the session in the N1 SM information.

## 1.2 Providing S-NSSAI to RAN

The approved S2-170603 “Response to LS on Network slicing and QoS for New Radio”, it is stated the following context about Slice ID:

On Slice ID:

Q6 (to SA2): How can the RAN receive an identifier that unequivocally identifies the network slice a UE needs to access?

Ans6: On what slice info the RAN may use, see Ans7 below.

Q7 (to SA2): How is such identifier defined? In RAN3 it was discussed that the identifier can be either provided by the CN or it could be provided by the UE.

Ans7: The NSSAI is a set of S-NSSAI.  The S-NSSAI pinpoints a single slice and is composed by a Slice/Service type information and an optional Slice differentiator information. See section 8.1 or TR 23.799 for more information. Whether a single value which is a representation of a collection of the S-NSSAIs could also be used as NSSAI is to be assessed in normative work.

The UE can provide the NSSAI in RRC connection establishment signalling. The 5G CN may also provide the RAN with the NSSAI that identifies the set of slices the UE is authorized to use. How and when is not yet decided. Whether NSSAI in RAN and NAS are exactly the same, is to be determined during normative phase.

For RAN to perform the slice specific handling for the session, RAN need to know which network slice the PDU session running in. It proposed to provide the S-NSSAI to the RAN from the CN through N2 message during PDU session establishment.

Proposal 2: The S-NSSAI associated with the session should be provided by the CN to the AN in the N2 SM information.

Additionally, the IPv4 address or IPv6 prefix is missed in the N1 SM message in the PDU session establishment accept message.

Proposal 3: The IPv4 address or IPv6 prefix allocated to the UE should be included in the N1 SM message in the PDU session establishment accept message.

# Proposal

The following changes are proposed to clauses 4.3.2 of TS 23.502.

* * * Begin of 1st Changes * * *

### 4.3.2	PDU Session establishment

#### 4.3.2.1	General

For the PDU session establishment procedure, two type of PDU session establishments exist:

-	UE initiated PDU session establishment procedure.

-	Network initiated PDU session establishment procedure. In this case the network sends the device trigger message to the application(s) on the UE side. The trigger payload included in Device Trigger Request message contains the information on which application on the UE side is expected to trigger the PDU Session establishment request. Based on that information, the application(s) on the UE side trigger the PDU session establishment procedure. For more detail refer to clause 5.2.6.

If the UE is simultaneously registered to a non-3GPP access via a N3IWF located in a PLMN different from the PLMN of the 3GPP access, the functional entities in the following procedure are located in the PLMN of the 3GPP access for non-roaming and LBO scenarios.

#### 4.3.2.2	UE requested PDU Session Establishment

Editor's note:	Additional details to support Ethernet and Unstructured PDU types in the procedure is FFS.

Editor's note:	Additional details to support UL Classifier or IPv6 multi-homing are FFS. This version of the procedure assumes a single UPF serves the PDU session.

Editor's note:	Additional details to support PDU sessions for emergency services are FFS.

Editor's note:	Further detailing of the messages over N2 is FFS and is to be coordinated with RAN WG3.

Editor's note:	Message naming over N11 may need further discussion as well as the need for message from AMF to SMF in step15 in X.Y.1. If step 15 is removed the AMF action described in step 15 would be carried out in step 9.

Editor's note:	It is FFS how the procedure will be updated once the relevant NF service procedures have been developed.

Editor's note:	Impact to the procedure due to mobility between 3GPP and non-3GPP access is FFS.

Editor's note:	The terminology for the PCF interactions below may need to be revisited and aligned with policy description in e.g. TS 23.501 [2].

##### 4.3.2.2.1	Non-roaming and Roaming with Local Breakout

In case of roaming, the AMF determines if a PDU session is to be established in LBO or Home Routing. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

The procedure assumes that the UE has already registered on the AMF thus the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI (Optional), DNN, PDU Session ID, N1 SM information).

In order to establish a new PDU session, the UE generates a new PDU Session ID.

The UE initiates the UE Requested PDU Session establishment procedure by the transmission of a NAS message containing a PDU Session Establishment Request within the N1 SM information. The PDU Session Establishment Request may include a PDU Type, SSC mode, Protocol Configuration Options.

The NAS message sent by the UE is encapsulated by the AN in a N2 message that should include User location information and Access Technology Type Information.

The SM information may contain SM PDU DN Request Container containing information for the PDU session authorization by the external DN.

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on the PDU Session ID that is not used for any existing PDU Session(s) of the UE. If the N1 SM information contains the requested S-NSSAI, the AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2. Otherwise, the AMF first determines a default S-NSSAI for the requested PDU session according to the UE subscription, then selects an SMF as described in TS 23.501 [2], clause 6.3.2.

3.	From AMF to SMF:  SM Request (Subscriber Permanent ID, DNN, S-NSSAI, PDU Session ID, AMF ID, N1 SM information, User location information, Access Technology Type).

The AMF ID uniquely identifies the AMF serving the UE. The N1 SM information contains the PDU Session Establishment Request received from the UE.

4a.	SMF to UDM: Subscription Data Request (Subscriber Permanent ID, DNN).

If the SMF has not yet retrieved the SM-related subscription data for the UE related with the DNN, the SMF requests this subscription data.

4b.	UDM to SMF: Subscription Data Response.

Subscription data includes the authorized PDU type(s), authorized SSC mode(s), Default QoS profile.

Editor's note:	Further detailing of SM-related subscription data is FFS.

The SMF checks whether the UE request is compliant with the user subscription and with local policies. If that is not the case the SMF rejects the UE request via NAS SM signalling (including a relevant SM rejection cause) relayed by the AMF, the SMF indicates to the AMF that the PDU session ID is to be considered as released and the rest of the procedure is skipped.

5.	SMF to DN via UPF

If the SMF needs to authorize/authenticate the establishment of the PDU session as described in clause 5.6.6 of TS 23.501 [2], the SMF selects an UPF as described in TS 23.501 [2] clause 6.3.3 and triggers the PDU session establishment authentication/authorization as described in section 4.3.2.X.

If the PDU session establishment authentication/authorization fails, the SMF terminates the PDU session establishment procedure and indicates a rejection to the UE.

6a.	If dynamic PCC is deployed, the SMF performs PCF selection.

6b.	The SMF may initiate PDU-CAN Session Establishment towards the PCF to get the default PCC Rules for the PDU Session.

Editor's note:	The purpose of step 5 is to receive PCC rules before selecting UPF. If PCC rules are not needed as input for UPF selection, step 5 can be skipped.

7.	The SMF selects an SSC mode for the PDU Session. If step 5 is not performed, the SMF also selects an UPF as described in TS 23.501 [2] clause 6.3.3. In case of PDU Type IPv4 or IPv6, the SMF allocates an IP address/prefix for the PDU Session as described in TS 23.501 [2] clause 5.8.1.

8.	If dynamic PCC is deployed and the PDU-CAN Session Establishment was not done in step 5, the SMF initiate PDU-CAN Session Establishment towards the PCF to get the default PCC Rules for the PDU Session. Otherwise, if dynamic PCC is deployed and PDU Type is IPv4 or IPv6, SMF initiates PDU-CAN Session Modification and provides the allocated UE IP address/prefix to the PCF.

9.	If step 5 was not performed, the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF:

9a.	The SMF sends an N4 Session Establishment/Modification Request to the UPF and provides Packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to UPF in this step.

9b.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If CN Tunnel Info is allocated by the UPF, the CN Tunnel Info is provided to SMF in this step.

10.	SMF to AMF:  SM Request Ack (N2 SM information (PDU Session ID, S-NSSAI, QoS Profile, CN Tunnel Info), N1 SM information (PDU Session Establishment Accept (Authorized QoS Rule, SSC mode, S-NSSAI, the allocated IP address/prefix))) to the AMF.

The N2 SM information carries information that the AMF shall provide to the (R)AN.

The CN Tunnel Info corresponds to the Core Network address of the N3 tunnel corresponding to the PDU session.

The QoS Profile provides the AN with the mapping between QoS parameters and QoS Flow Identifiers. This is further described in TS 23.501 [2] clause 5.7.

The PDU Session ID may be used by AN signalling with the UE to indicate to the UE the association between AN resources and a PDU session for the UE.

The N1 SM information contains the PDU Session Establishment Accept that the AMF shall provide to the UE.

Multiple Authorized QoS Rules may be included in the PDU Session Establishment Accept within the N1 SM information and in the N2 SM information.

The SM Request Ack further contains information allowing the AMF to know which UE is the target of the SMF request as well to determine which access towards the UE to use.

NOTE:	The access information is to deal with the case where a UE is simultaneously connected over 3GPP and Non 3GPP access.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to know which UE is the target of the SMF request as well to determine which access towards the UE to use.

11.	AMF to (R)AN: N2 PDU Session Request (N2 SM information, , PDU Session Establishment Accept).

The AMF sends the PDU Session Establishment Accept and the N2 SM information received from the SMF within the N2 PDU Session Request to the (R)AN.

12.	(R)AN to UE: The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE establishing the necessary RAN resources related to the Authorized QoS Rules for the PDU Session request received in step 10.

(R)AN also allocates (R)AN tunnel information for the PDU Session. (R)AN forwards the NAS message (PDU Session Establishment Accept) provided in step 10 to the UE. (R)AN shall only provide the NAS message to the UE if the necessary RAN resources are established and the allocation of (R)AN tunnel information are successful.

13.	(R)AN to AMF: N2 PDU Session Request Ack ((R)AN Tunnel Info).

The (R)AN Tunnel Info corresponds to the Access Network address of the N3 tunnel corresponding to the PDU session.

14.	AMF to SMF: SM Request (N2 SM information).

The AMF forwards the N2 SM information received from (R)AN to the SMF.

Editor's note:	It is FFS if steps are needed to indicate from the UE to the core network that the UE has successfully established the PDU Session, or whether it is sufficient with a successful establishment in (R)AN indicated in step 12. For example, it is FFS if the UE shall send a NAS PDU Session Establishment Complete message to indicate that the UE has successfully established the PDU Session.

15a.	If the N4 session for this PDU Session was not established already, the SMF initiates an N4 Session Establishment procedure with the UPF. Otherwise, the SMF initiates an N4 Session Modification procedure with the UPF. The SMF provides AN Tunnel Info and CN Tunnel Info. The CN Tunnel Info only needs to be provided if the SMF selected CN Tunnel Info in step 8.

15b.	The UPF provides a N4 Session Establishment/Modification Response to the SMF.

16.	After this step, the AMF forwards relevant events to the SMF, e.g. at handover where the (R)AN Tunnel Info changes or the AMF is relocated.

Editor's note:	It is FFS if the SMF has to explicitly subscribe to these events or whether the subscription is implicit.

17.	SMF to UE, via UPF: In case of PDU Type IPv6, the SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the UPF.

During the lifetime of the PDU session, the AMF stores an association of the PDU session ID and the SMF ID.

* * * End of 1st Changes * * *