SA WG2 Meeting #121	S2-174074

29 May– 2 Jun 2017, Hangzhou, China	(was S2-173802)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, ATT, Huawei, Vodafone

Title:	TS 23.502: PCF services identification

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes PCF services and service operations

# 1	Background

During the last SA2, following was agreed:

- NF services are derived from the information flows that describe end to end functionality, where applicable (see 3GPP TS 23.502 [3], Annex B drafting rules).

- Complete end to end message flows can be described by a sequence of NF service invocations.

NOTE:	While the use of NF services is not restricted to the identified consumers, no specific effort will be made in this release of the specification, to develop the description of NF services beyond what is necessary to support their use in information flows.

It was also agreed that:

“Each of the NF services offered by a Network Function shall be self-contained, reusable and use management schemes independently of other NF services offered by the same Network Function (e.g. for scaling, healing, etc).”

“NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service operation: An elementary unit a NF service is composed of. “

It is proposed that we summarize the NF services in a table with service description, input/output parameters, other relevant information and have a column to determine whether the given call flow is referring to NF service operation or NF service.

# 2	Criteria

We consider the following set of criteria for defining the services:

- each NF service operates on its own set of context(s)

- services operations that operate on a common context data shall be defined within a single service. Any direct access of a context owned by a NF service are only made by the service operations of that NF service.

- One non-service based interface shall be tied to at most one NF service in an NF.

# 3	Considerations for identifying PCF services and service operations

The services exhibited by PCF should mainly include features that are essential for other Network functions would need as described in e2e call flows in section 4.

To start with, the list of functions that UDM performs need to be considered. From 3GPP TS 23.501 section 6.2.4 the following are the list of PCF functions:

The Policy function (PCF) includes the following functionality: - Supports unified policy framework to govern network behaviour. - Provides policy rules to Control Plane function(s) to enforce them. - Implements a Front End to access subscription information relevant for policy decisions in a User Data Repository (UDR).

It is proposed to update TS 23.502 to include the following services for PCF

If this is agreeable, then it is proposed to update the call flows in section 5 accordingly (i.e. remove the ones that are not relevant as per e2e call flows in section 4).

* * * * Start Change * * *.*

### 5.2.5	PCF Services

Table 5.2.5: NF Services provided by PCF

| NF service | Description | NF service operations | Known Consumer (s) | Mechanisms | Reference |
| --- | --- | --- | --- | --- | --- |
| Npcf_PolicyControl | Provides access control, network selection and mobility management related policies to the AMF, UE Route Selection Policy to the UE via the AMF, and session related policies to the SMF | PolicyCreate | AMF, SMF | Request-Response | 4.2.2.2.2 (step 16 and 17), 4.3.2.2.1 (step 6b), 4.3.3.2 (step 1b, step 2 and step 9), 4.3.2.2.2 (step 9b) |
|  |  | PolicyUpdate | AMF, SMF | FFS | 4.2.3.2 (step 10), 4.2.3.2 (step 9), 4.3.2.2.1 (step 8), 4.3.2.2.2 (step 11), 4.3.3.2 (step 1b and step 2 and step 9), 4.3.4.3 (step 1b) |
|  |  | PolicyDelete | AMF, SMF | Request-Response | 4.2.2.2.2 (step 20 and 21), 4.3.4.2 (step 3), 4.3.4.3 (step 3) |
| Npcf Event Notification | Enables NF consumer to subscribe/unsubscribe the notification of UE mobility events, session events, and policy stats of the UE(s). | EventSubscribe | AF, NEF | Subscribe-Notify | FFS |
|  |  | EventUnsubscribe | AF, NEF | Subscribe-Notify | FFS |
|  |  | EventNotify | AF, NEF | Subscribe-Notify | FFS |
| Npcf_Policy Authorization | Authorises an AF request and creates policies as requested by the authorised AF for the PDU-CAN session which the AF session is bound to. | AFAuthorize | AF, NEF | Request-Response | FFS |

Editor's note:   Whether common or separate services should be defined for UE and mobility management policies and Session policy management is FFS.

Editor's note:   Whether common or separate PolicyUpdate operations for Npcf_PolicyControl service should be defined due to various triggers from AMF/SMF and from other NFs (e.g. UDR, NWDS, AF, NEF, OCS, etc) and what are the corresponding Operation Semantics are FFS. Whether the name of the operation should be change is FFS.

Editor’s note: The reference to Clause 4 system procedures for which the NF service operations are applied are work on progress and will be further updated.

Editor’s note: The reference to procedures defined in TS 23.501 for which the NF service operations are applied is FFS.

#### 5.2.5.1	Npcf_PolicyControl service

#### Editor’s note: whether the service operations are documented in textual form or table form is TBD.

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| PolicyCreate | Provides the requested policy to the NF Consumer | UE Permanent ID, Requested Policy Container | The requested policy rule of the UE or PDU-CAN session |
| PolicyUpdate | Provides updated policy to the NF Consumer. | UE Permanent ID, Requested Policy Container. | The requested policy rule of the UE or PDU-CAN session |
| PolicyDelete | Provides means for the NF Consumer to delete the context of UE related policy or PDU-CAN session related policy. | UE Permanent ID, Delete Type. | Success or Failure |

Service description: This service provides access control, network selection and mobility management related policies to the AMF, UE Route Selection Policy to the UE via the AMF, and session related policies to the SMF, which includes the following functionalities:

- Create policies based on the request from AMF during UE registration;

- Create policies based on the request from SMF during PDU-CAN session establishment;

#### Notify AMF/SMF of the updated policies which are subscribed

#### Delete policies for an UE or a PDU-CAN session

##### 5.2.5.1.1	Npcf_PolicyControl_PolicyCreate Service Operation

##### 5.2.5.1.1.1 Description

Service operation name: Npcf_PolicyControl_PolicyCreate

Description: Provides the requested policy to the NF Consumer.

Known NF Consumers: AMF/SMF.

Inputs, Required: UE Permanent ID, Requested Policy Container.

Inputs, Optional: None.

Outputs, Required: The requested policy rule of the UE or PDU-CAN session.

Outputs, Optional: None.

See sub-clause 4.2.2.2.2 (step 16 and 17) for the detail usage of this service operation for AMF. In step 16, the AMF requests the PCF to apply operator policies for the UE; in step 17, the PCF acknowledges AMF with requested policy.

See sub-clause 4.3.2.2.1 (step 6b), sub-clause 4.3.2.2.2 (step 9b), and sub-clause 4.3.3.2 (step 1b, step 2 and step 9) for the detail usage of this service operation for SMF. In those steps, the SMF requests default PCC Rules for the PDU Session from PCF/H-PCF.

##### 5.2.5.1.2	Npcf_PolicyControl_PolicyUpdate Service Operation

##### 5.2.5.1.2.1	Description

Service operation name: Npcf_PolicyControl_PolicyUpdate

Description: Provides updated policy to the NF Consumer.

Known NF Consumers: AMF/SMF.

Inputs, Required: None.

Inputs, Optional: None.

Outputs, Required: The updated policy rules of the UE or PDU-CAN session.

Outputs, Optional: None

See sub-clause 4.2.3.2 (step 10), sub-clause 4.2.3.2 (step 9), sub-clause 4.3.2.2.1 (step 8), sub-clause 4.3.2.2.2 (step 11), sub-clause 4.3.3.2 (step 1b, step 2 and step 9) and sub-clause 4.3.4.3 (step 1b) for the detail usage of this service operation for SMF. In those steps, the updated PCC Rules for the PDU Session are provided to the SMF by the PCF/H-PCF.

##### 5.2.5.1.3	Npcf_PolicyControl_PolicyDelete Service Operation

##### 5.2.5.1.3.1 Description

Service operation name: Npcf_PolicyControl_Delete

Description: Provides means for the NF Consumer to delete the context of UE related policy or PDU-CAN session related policy.

Known NF Consumers: AMF/SMF.

Inputs, Required: UE Permanent ID, Delete Type.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

See sub-clause 4.2.2.2.2 (step 20 and 21) for the detail usage of this service operation for AMF. In step 20, the old AMF requests to delete the UE context in the PCF; in step 21, the PCF deletes the UE context and acknowledges the old AMF.

See sub-clause 4.3.4.2 (step 3) and sub-clause 4.3.4.3 (step 3) for the detail usage of this service operation for SMF. In those steps, the SMF initiates the PDU-CAN session termination procedure, and PCF deletes the session contexts.

* * * * Next Change * * **

#### 5.2.5.3	Npcf_PolicyAuthorization

Editor’s note: whether the service operations are documented in textual form or table form is TBD.

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| EventSubscribe | Authorize the request, and optionally determines and installs the policy according to the information provided by the NF Consumer | The UE Identity (e.g. UE IP address, MSISDN), the requested policy container | Success or Failure |

Service description: This service is to authorise an AF request and to create policies as requested by the authorised AF for the PDU-CAN session to which the AF session is bound.

##### 5.2.5.3.1	Npcf_Policy Authorization_Authorise Service Operation

##### 5.2.5.3.1.1	Description

#### Service operation name: Npcf_PolicyControl_Authorize

Description: Authorize the request, and optionally determines and installs the policy according to the information provided by the NF Consumer .

Known NF Consumers: AF/NEF.

Inputs, Required: The UE Identity (e.g. UE IP address, MSISDN), the requested policy container.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

* * * * Next Change * * **

#### 5.2.5.2	Npcf_EventNotification Service

Editor’s note: whether the service operations are documented in textual form or table form is TBD.

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| EventSubscribe | Enable NF consumers to explicitly subscribe the notification of UE mobility events (e.g. registration status change, connection status change, location change, and etc.), session events (e.g. usage report, credit report, and etc.) and policy states of the UE(s). | SUPI or External ID, List of events (including the event type and event object), Reporting options (e.g. periodic reporting, event based reporting) | Success or Failure |
| EventUnsubscribe | Enable NF consumers to explicitly unsubscribe the notification of UE mobility events (e.g. registration status change, connection status change, location change, and etc.), session events (e.g. usage report, credit report, and etc.) and policy states of the UE(s). | SUPI or External ID, List of events (including the event type and event object), Reporting options (e.g. periodic reporting, event based reporting) | Success or Failure |
| EventNotify | Notify NF consumers of the subscribed events, such as UE mobility events (e.g. registration status change, connection status change, location change, and etc.), session events (e.g. usage report, credit report, and etc.) and policy states of the UE(s). | The list of event type and its object, Notify ID | The event information for the subscribed event type and event object |

Service description: This service is provided by the PCF for the NF consumer to subscribe/unsubscribe the notification of UE mobility events (e.g. registration status change, connection status change, location change, and etc.), session events (e.g. usage report, credit report, and etc.) and policy stats of the UE(s).

##### 5.2.5.2.1	Npcf Event Notification_EventSubscribe Service Operation

##### 5.2.5.2.1.1  Description

Service operation name: Npcf_EventNotification_EventSubscribe

Description: provided by the PCF for NF consumers to explicitly subscribe the notification of UE mobility events (e.g. registration status change, connection status change, location change, and etc.), session events (e.g. usage report, credit report, and etc.) and policy states of the UE(s).

Known NF Consumers: AF/NEF.

Inputs, Required: SUPI or External ID, List of events (including the event type and event object), Reporting options (e.g. periodic reporting, event based reporting)

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

##### 5.2.5.2.2	Npcf Event Notification_EventUnsubscribe Service Operation

##### 5.2.5.2.2.1  Description

Service operation name: Npcf Event Notification_EventUnsubscribe

Description: provided by the PCF for NF consumers to explicitly unsubscribe the notification of UE mobility events (e.g. registration status change, connection status change, location change, and etc.), session events (e.g. usage report, credit report, and etc.) and policy states of the UE(s).

Known NF Consumers: AF/NEF.

Inputs, Required: SUPI or External ID, List of events (including the event type and event object), Reporting options (e.g. periodic reporting, event based reporting)

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

##### 5.2.5.2.3	Npcf Event Notification_EventNotify Service Operation

##### 5.2.5.2.3.1	Description

Service or service operation name: Npcf Event Notification_EventNotify

Description: provided by the PCF to notify NF consumers of the subscribed events, such as UE mobility events (e.g. registration status change, connection status change, location change, and etc.), session events (e.g. usage report, credit report, and etc.) and policy states of the UE(s).

Known NF Consumers: AF/NEF.

Inputs, Required: The list of event type and its object, Notify ID.

Inputs, Optional: None.

Outputs, Required: The event information for the subscribed event type and event object.

Outputs, Optional: None.

* * * * End Change * * *.*