SA WG2 Meeting #122 Bis	S2-175550

21 - 25 August 2017, Sophia Antipolis, France

Source:	Huawei

Title:	Conflict handling between handover procedure and QoS flow procedure in 5GC aligning with EPC

Document for:	Approval

Agenda Item:	6.5.7.3

Work Item / Release:	5G-Ph1 / Rel-15

Abstract of the contribution: Discusses the need that 5GC should align handover conflict handling methods with EPC.

- Discussion

In EPS, there is a conflict handling mechanism between handover procedure (including X2-HO, S1-HO, Inter-RAT HO) and NAS signaling or EPS bearer procedure. For HO has been known by MME scenario and “message crossing” scenario, the methods are specifically described in clause 5.5.1.1.1, clause5.5.1.2.1 and 5.5.2.0, TS 23.401.

To avoid unpredictable behaviour during handovers leading to inconsistent states in the UE and the Network leading to lost sessions, 5GS should remain this feature aligning with EPS.

Different with EPS, MM and SM are separated and SM procedures are managed by SMF in 5GS, in addition, SMF will not be relocated during HO procedure in R15, so the alignment feature in 5GS can be simplified. Take the N2-based HO for an example, the feature may be presented as follows: (Xn-HO or inter-RAT HO is similar)

If the SMF receives a rejection to the QoS flow procedure (e.g. QoS flow establishment/modification/release; etc.) from the NG RAN with an indication that an N2-based handover is in progress, the SMF shall reattempt the same QoS flow procedure when either the handover is complete or the handover is deemed to have failed.

In order to minimise the number of procedures rejected by NG RAN, the SMF should pause QoS flow procedures (e.g. QoS flow establishment/ modification/ release; etc.) while a N2-based handover is ongoing, and continue to initiate the same QoS flow procedure when either the handover procedure has complete or the handover is deemed to have failed.

2	Proposal

It is proposed to Update TS 23.502 as follows.

######################### TEXT PROPOSAL FOR TS 23.502 ###########################

4.9.1	Handover procedures in 3GPP access

Editor's note:	(S2-167180):

1)	Intra NG RAN node,

2)	Inter NG RAN node with Xn interface,

3)	Intra AMF, Intra SMF, Inter NG RAN node without Xn interface,

4)	Intra AMF, Inter SMF, Inter NG RAN node without Xn interface,

5)	Inter AMF, Inter and Intra SMF, Inter NG RAN node without Xn interface.

"NG RAN node" naming discussed in RAN, and AMF to be changed when new name agreed.

If the SMF receives a rejection to the QoS flow procedure (e.g. QoS flow establishment/modification/release; etc.) from the NG RAN with an indication that handover procedure is in progress, the SMF shall reattempt the same QoS flow procedure when either the handover is complete or the handover is deemed to have failed. This feature is applied to Xn based inter NG RAN handover as defined in clause 4.9.1.1 and Inter NG-RAN node handover without Xn interface as defined in clause 4.9.1.2.

If handover is ongoing (i.e. from the time that a PDU Handover Request has been received until either the Handover procedure has succeeded (Handover Complete) or failed (Handover Failure)), in order to minimize the number of procedures rejected by NG RAN, the SMF should pause QoS flow procedures (e.g. QoS flow establishment/ modification/ release; etc.), and continue to initiate the same QoS flow procedure when either the handover procedure has complete or the handover is deemed to have failed. This feature is applied to Inter NG-RAN node handover without Xn interface as defined in clause 4.9.1.2.

########################NEXT CHANGE IN TS 23.502 ###########################

4.11.1.1	5GS to EPS handover using Nx interface

Nx interface is used to provide seamless session continuity for single registration mode. Figure 4.11.1.1-1 describes the handover procedure from 5GS to EPS when Nx is supported.

If the PGW-C+SMF receives a rejection for a QoS flow procedure with an indication that the request has been temporarily rejected due to 5GS to EPS handover procedure in progress, the PGW-C+SMF behaves as specified in clause 4.9.1.

If 5GS to EPS handover procedure is ongoing, in order to minimize the number of procedures rejected by NG RAN, the PGW-C+SMF should behave as specified in clause 4.9.1.

Editor's note:	It is FFS if this flow is applicable to PDU session with SSC Mode 2 and 3.

# Figure 4.11.1.1-1: 5GS to EPS handover for single-registration mode with Nx interface

The procedure involves a handover to EPC and setup of default EPS bearer and dedicated bearers for GBR QoS flows in EPC in steps 1-16 and re-activation, if required, of dedicated EPS bearers for non-GBR QoS flows in step 17.

UE has one or more ongoing PDU sessions each including one or more QoS flows. During PDU session establishment and GBR QoS flow establishment, EPS QoS mappings and EPS Bearer IDs are allocated such that non-GBR flows map to default EPS bearer (which is allocated an EPS bearer ID) and EPS Bearer IDs are allocated for the GBR flows that are mapped to dedicated bearers in EPC. The EPS Bearer Id and mapped QoS parameters for these bearer is also provided to the UE and PGW-C+SMF. The mapped EPS QoS parameters may be provided to PGW-C+SMF by the PCF+PCRF, if PCC is deployed.

Editor's note:	It is FFS whether the 5G RAN needs to be aware of the EPS bearer ID.

Editor's note:	The details of QoS parameters mapping from 5GC to EPC and EPS bearer ID allocation is FFS.

Editor's note:	How TFT is created/allocated is FFS.

1.	5G RAN decides that the UE should be handed over to the E-UTRAN. The 5G RAN sends a Handover Required (Target eNB ID, Source to Target Transparent Container) message to the AMF.

2.	The AMF determines from the 'Target eNB Identifier' IE that the type of handover is Handover to E-UTRAN. The AMF requests the PGW-C+SMF to provide SM Context that also includes the mapped EPS Bearer Contexts. This step is performed with all PGW-C+SMFs allocated to the UE.

Editor's note:	This step should be aligned with intra 5GC inter-AMF handover.

NOTE:	In roaming scenario, the UE's SM EPS Contexts are obtained from the V-SMF.

3.	The AMF selects an MME and sends a Relocation Request (Target E-UTRAN Node ID, Source to Target Transparent Container, mapped MM and SM EPS UE Context (default and dedicated GBR bearers)) message. The SGW address and TEID for both the control-plane or EPS bearers in the message are such that target MME selects a new SGW.

4.	The MME selects the Serving GW and sends a Create Session Request message for each PDN connection to the Serving GW.

5.	The Serving GW allocates its local resources and returns them in a Create Session Response message to the MME.

6.	The MME requests the target eNodeB to establish the bearer(s) by sending the message Handover Request message. This message also contains a list of EPS Bearer IDs that need to be setup.

7.	The target eNB allocates the requested resources and returns the applicable parameters to the target MME in the message Handover Request Acknowledge (Target to Source Transparent Container, EPS Bearers setup list, EPS Bearers failed to setup list).

8.	If the MME decides that indirect forwarding applies, it sends a Create Indirect Data Forwarding Tunnel Request message (Target eNB Address, TEID(s) for DL data forwarding) to the Serving GW. The Serving GW returns a Create Indirect Data Forwarding Tunnel Response (Cause, Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding) message to the target MME.

9.	The MME sends the message Relocation Response (Cause, List of Set Up RABs, EPS Bearers setup list, MME Tunnel Endpoint Identifier for Control Plane, RAN Cause, MME Address for Control Plane, Target to Source Transparent Container, Address(es) and TEID(s) for Data Forwarding).

10.	If indirect forwarding applies, the AMF forwards to the PGW-C+SMF the information related to data forwarding to the SGW. The PGW-C+SMF returns a Create Indirect Data Forwarding Tunnel Response.

11.	The AMF sends the Handover Command to the source 5G RAN. The source 5G RAN commands the UE to handover to the target access network by sending the HO Command. This message includes a transparent container including radio aspect parameters that the target eNB has set-up in the preparation phase. The UE correlates the ongoing QoS flows with the indicated EPS Bearer IDs to be setup in the HO command. UE locally deletes the QoS flows that do not have an EPS bearer ID assigned.12.	When the UE has successfully accessed the target eNodeB, the target eNodeB informs the target MME by sending the message Handover Notify.

13.	The target MME informs the Serving GW that the MME is responsible for all the bearers the UE have established by sending the Modify Bearer Request message for each PDN connection.

The target MME releases the non-accepted EPS Bearer contexts by triggering the Bearer Context deactivation procedure. If the Serving GW receives a DL packet for a non-accepted bearer, the Serving GW drops the DL packet and does not send a Downlink Data Notification to the SGSN.

14.	The Serving GW informs the PGW-C+SMF of the relocation by sending the Modify Bearer Request message for each PDN connection. The PGW locally deletes the QoS flows that do not have an EPS bearer ID assigned. Due to the "match all" filter in the default QoS flow, the PGW maps the IP flows of the deleted QoS flows to the default QoS flow.

15.	The PGW-C+SMF acknowledges the Modify Bearer Request. At this stage the User Plane path is established for the default bearer and the dedicated GBR bearers between the UE, target eNodeB, Serving GW and the PGW+SMF.

16.	The Serving GW acknowledges the User Plane switch to the MME via the message Modify Bearer Response.

17.	The PGW-C+SMF initiates dedicated bearer activation procedure for non-GBR QoS flows by mapping the parameters of the non-GBR flows to EPC QoS parameters. This setup may be triggered by the PCRF+PCF which may also provide the mapped QoS parameters, if PCC is deployed. This procedure is specified in TS 23.401 [x], clause 5.4.1.

########################END of CHANGE IN TS 23.502 ###########################