SA WG2 Meeting #122BIS	S2-175665

21- 25 Aug 2017, Sophia Antipolis, France	(revision of S2-17xxxx)

Source:	Huawei, HiSilicon, China mobile

Title:	TS 23.502: Update the registration procedure of SMS over NAS

Document for:	Approval

Agenda Item:	6.5.8.1

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution updates the registration procedure of SMS over NAS in TS 23.502.

Introduction

The contribution proposes to update the following changes,

- Registration of SMS service should be part of common registration procedure. Now it looks like a separated procedure. The contribution updates registration procedure to align with common registration procedure, and also replace the P2P messages with the relevant service operations.

- Supporting AMF relocation during Registration procedure to align with the following agreement in TS 23.501.

NOTE 4: When serving AMF is relocated for a given UE, SMS function for a given UE does not need to change. Thus the source AMF includes SMSF identifier as part of UE context transfer during AMF relocation to target AMF.

Proposal

It is proposed to update TS 23.502 as follows:

* * * Begin of Changes * * *

4.13.3.1	Registration procedures for SMS over NAS

Figure 4.13.3.1-1: Registration procedure supporting SMS over NAS

1.	During initial registration in 5GS defined in Figure 4.2.2.2.2-1, to enable SMS over NAS transporting, the UE includes an "SMS supported" indication in Registration Request in step 1-3 indicating the UE's capability for SMS over NAS transport. The "SMS supported" indication indicates whether the UE supports SMS delivery over NAS via 3GPP access, or via non3GPP access, or via both the 3GPP and non-3GPP access.

2.  Step 4 to step 13 of the Registration procedure in Figure 4.2.2.2.2-1 are performed. When AMF relocation happens during the registration, the old AMF transfers SMSF’s address to the new AMF as part of UE context in step 5 of Figure 4.2.2.2.2-1.

3.	The AMF invokes the Nudm_UEContextManagement_Registration service operation towards the UDM, and gets subscription data from the UDM as described in step 14a and 14b of Figure 4.2.2.2.2-1. If the "SMS supported" indication is included in the Registration Request, the AMF checks SMS subscription from the UDM for the UE on whether the SMS service is allowed to the UE. If yes and the UE context doesn’t include an available SMSF of the serving PLMN, the AMF discovers and selects an SMSF to serve the UE.

The SMSF discovery is based on the following methods:

-	SMSF's address preconfigured in the AMF (i.e., SMSF FQDN); or

-	SMSF's address received from the UDM; or

-	The AMF invokes Nnrf_NF management_Discovery service operation from NRF to discover the SMSF address as described in clause 5.2.7.1.2.

For roaming scenario, the AMF discovers and selects an SMSF in VPLMN.

4. Step 15 to step 20 of the Registration procedure in Figure 4.2.2.2.2-1 are performed.

5a.	The AMF includes the "SMS supported" indication to the UE in the Registration Accept message of step 21 of Figure 4.2.2.2.2-1 if the AMF has selected an SMSF; or

5b.	The AMF includes the "SMS supported" indication to the UE in the Registration Accept message of step 21 of Figure 4.2.2.2.2-1 only after step 8 in which the AMF has received a positive indication from the selected SMSF.

The "SMS supported" indication in the Registration Accept message (sent in step 5a or 5b) indicates to the UE whether the network allows the SMS message delivery over NAS via 3GPP access or via both the 3GPP and non-3GPP access.

6.	The AMF invokes Nsmsf_SMService_Activate service operation from the SMSF. The invocation includes AMF address and SUPI. AMF uses the SMSF address derived from step 3.

7.	If the UE context already exists in the SMSF, the SMSF shall replace the old AMF address with the new AMF address. Otherwise, the SMSF retrieves the SMS related subscription in step 7a by invoking Nudm_Subscriber_Data_Get service operation from the UDM. SMSF also creates an UE context to store the SMS subscription information and the AMF address that is serving this UE. The SMSF then invokes Nudm_UE_Context_Management_Registration service operation to store the SMSF address in the UDM for short message terminating routing.

8.	The SMSF responds back to the AMF with Nsmsf_SMService_Activate service operation response message. The AMF stores the SMSF address received as part of the UE context.

4.13.3.2	De-Registration procedures for SMS over NAS

If UE indicates to AMF that it no longer wants to send and receive SMS over NAS (e.g., not including "SMS supported" indication in subsequent TAU) or AMF considers that UE is no longer reachable (i.e., UE is detached from 5GS), AMF uses Namf_EventExposure_Notify service operation to notify SMSF that UE is currently deregistered from SMS over NAS in 5GS and subsequently deletes the SMSF address. The SMSF shall invoke Nudm_UE_Context_Management_Deregistration service operation from UDM to trigger UDM to delete SMSF address of the UE. The SMSF also removes the contexts of the UE.

* * * End of Changes * * *