SA WG2 Meeting #122bis	S2-175739

21 - 25 August 2017, Sofia-Antipolis, France	(revision of S2-174571, 5156)

Source:	NTT DOCOMO

Title:	No-N26 Interworking Procedures

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Captures the No-N26 interworking procedures based on agreed text in 23.501-120.

# 1. Introduction

# 2.	Updates to 23.502

### **************  START CHANGES **********************

## 4.11	System interworking procedures with EPS

Editor's note:	Including interworking procedures with EPS.

### 4.11.1	Interworking procedures with N26 interface

### 4.11.1.1	General

#### 4.11.1.2	Handover procedures

##### 4.11.1.2.1	5GS to EPS handover using N26 interface

Nx interface is used to provide seamless session continuity for single registration mode. Figure 4.11.1.1-1 describes the handover procedure from 5GS to EPS when Nx is supported.

...

### **************  NEXT CHANGE **********************

##### 4.11.1.1.2	EPS to 5GS handover using N26 interface

###### 4.11.1.1.2.1	General

N26 interface is used to provide seamless session continuity for single registration mode. Figure 4.11.1.2.1-1 describes and Figure 4.11.1.x.3-1 the handover procedure from EPS to 5GS when N26 is supported.

..

### **************  NEXT CHANGE **********************

#### 4.11.1.2	Idle Mode procedures

##### 4.11.1.2.1	5GS to EPS Idle mode mobility

Editor's Note: Will contain 5GS to EPS Idle mode mobility procedure

##### 4.11.1.2.2	EPS to 5GS Idle mode mobility

Editor's Note: Will contain 5GS to EPS Idle mode mobility procedure

### **************  NEXT CHANGE **********************

### 4.11.2	Interworking procedures without N26 interface

### 4.11.2.1 	General

#### 4.11.2.2	5GS to EPS Mobility

The following procedure is used by UEs in single-registration mode on mobility from EPS to 5GS. The UE may have been either in idle or connected mode in 5GS. If the UE was in connected mode, it goes into idle mode and then performs this procedure.

Figure 4.11.2.2-1. Mobility procedure from 5GS to EPS without N26 interface.

UEs operating in single-registration mode starts the procedure from Step 1.

UEs operating in dual-registration mode starts the procedure from Step 5.

0.	UE is registered in 5GS.

1.	Step 1 as in clause 5.3.3.1 in TS 23.401[x].

2.	Step 2 as in clause 5.3.3.1 in TS 23.401[x] with the following modifications:

The UE shall provide a 4G-GUTI mapped from the 5G-GUTI. The GUTI type also indicates to the MME that the GUTI is mapped from 5G-GUTI (see TS 23.401[x] clause 4.3.19.1).

3.	Step 3 as in clause 5.3.3.1 in TS 23.401[x].

4.	If the MME determined that the old node is an AMF based on UE's 4G-GUTI and the MME is configured to support 5GS-EPS interworking without N26 procedure, the MME sends a TAU Reject message with a "Handover PDN Connection Setup Support" indication to the UE.

5. Step 1 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401[x] with the following modification:

The UE provides 4G-GUTI mapped from 5G-GUTI.

6.	Step 2 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401[x] with the following modification:

If the UE provided a 4G-GUTI mapped from 5G-GUTI in Step 1 and the MME is configured to support 5GS-EPS interworking without N26 procedure, the MME does not perform Step 3, Identity request to old MME/SGSN/AMF in clause 5.3.2.1 in TS 23.401[x].

7.	Steps 4-7 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401[x].

8. Step 8 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401[x], with the following modifications:

If the MME determined that the old node is an AMF based on UE's 4G-GUTI and the MME is configured to support 5GS-EPS interworking without N26 procedure, the MME does not include the "initial attach" indication to the HSS+UDM. The HSS+UDM does not send cancel location to the old AMF.

9.	Step 11 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401[x].

10.Steps 12-24 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401[x].

11. Step 25 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401[x] with the following modification:

Additional trigger for this step: The UE provided a 4G-GUTI mapped from 5G-GUTI in Step 5 and the MME is configured to support 5GS-EPS interworking without N26 procedure.

12. Step 26 as in in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401[x].

13. If the UE had setup PDU Sessions in 5GS which it wants to transfer to EPS and maintain the same IP address/prefix, the UE performs the UE requested PDN Connectivity Procedure as specified in TS 23.401[x] clause 5.10.2 and sets the Request Type to "handover" in Step 1 of the procedure.

UEs in single-registration mode perform this step immediately after the completing of E-UTRAN Initial Attach procedure. UEs in dual-registration mode may perform this step any time after the completing of E-UTRAN Initial Attach procedure. Also, UEs in dual-registration mode may perform this step only for a subset of PDU sessions.

#### 4.11.2.3	EPS to 5GS Mobility

The following procedure is used by UEs in single-registration mode on mobility from 5GS to EPS. The UE may have been either in idle or connected mode in EPC. If UE in single-registration mode was in connected mode, it goes into idle mode and then performs this procedure.

This procedure is also used by UEs in dual-registration mode to perform registration in 5GS when the UE is also registered in EPC. The procedure is the General Registration procedure as captured in clause 4.2.2. Difference from that procedure are captured below.

Figure 4.11.2.3-1. Mobility procedure from EPS to 5GS without N26 interface.

0.	The UE is attached in EPC.

1.	Step 1 in clause 4.2.2.2.2 with the following clarifications:

The UE shall set the Registration type to “mobility registration update”. The UE shall provide a UE Temporary ID mapped from the 4G-GUTI (see clause 5.17.2.2: 5G-GUTI mapped from 4G-GUTI).

2.	Step 2 as in clause 4.2.2.2.

3. Step 3 as in clause 4.2.2.2. with the following modifications:

If the Registration type is "mobility registration update" and the UE provided a 5G-GUTI mapped from 4G-GUTI in Step 1, and the AMF is configured to support 5GS-EPS interworking procedure without N26 interface, the AMF treats this registration request as "initial Registration". If the UE provided PDU Session Status information in Step 1, the AMF does not attempt to synchronize the status of PDU session.

If the UE has provided a 5G-GUTI mapped from 4G-GUTI in Step 1 and the AMF is configured to support 5GS-EPS interworking procedure without N26 interface, the AMF does not perform Steps 4 and 5 in clause 4.2.2.2.

4.	Steps 6-13 as in clause 4.2.2.2.

5.	Step 14 as in clause 4.2.2.2. with the following modifications:

If the AMF determined that the old node is an MME based on UE's 5G-GUTI and the AMF is configured to support 5GS-EPS interworking without N26 procedure, the AMF does not include the "initial Registration" indication to the UDM. The UDM does not send cancel location to the old MME.

6.	Steps 15-21 as in clause 4.2.2.2.

7.	Step 22 as in clause 4.2.2.2 with the following modifications:

If the Registration type in Step 1 is "initial Registration" and the AMF determined that the old node is an MME based on the GUTI provided in Step 1 and the AMF is configured to support 5GS-EPS interworking procedure without N26 interface, the AMF includes a "Dual registration support" indicator to the UE.

If the Registration type in Step 1 is "mobility registration update " and the UE provided a 5G-GUTI mapped from 4G-GUTI in Step 1, and the AMF is configured to support 5GS-EPS no-N26 interworking procedure, the AMF includes a "Handover PDU Session Setup with EPC Support" indication to the UE. If the UE had provided PDU Session Status information in Step 1, the AMF sets the PDU Session Status to not synced.

8.	Step 23 as in clause 4.2.2.2.

9.	If the UE had setup PDN Connections in EPC which it wants to transfer to 5GS and maintain the same IP address/prefix, the UE performs the UE requested PDU Session Establishment Procedure as in clause 4.3.2.2 and sets the Request Type to "Existing PDU Sessions" in Step 1 of the procedure.

UEs in single-registration mode performs this step immediately after the Step 8. UEs in dual-registration mode may perform this step any time after Step 8. Also, UEs in dual-registration mode may perform this step only for a subset of PDU sessions.

### **************  NEXT CHANGE    **********************

##### 4.3.2.2.1	Non-roaming and Roaming with Local Breakout

This clause specifies PDU Session establishment in the non-roaming and roaming with local breakout cases. The procedure is used to establish a new PDU session as well as to hand over an existing PDU Session between 3GPP access and non-3GPP access.

In case of roaming, the AMF determines if a PDU session is to be established in LBO or Home Routing. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

The procedure assumes that the UE has already registered on the AMF thus the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI, DNN, PDU Session ID, Request type, N1 SM information).

In order to establish a new PDU session, the UE generates a new PDU Session ID.

The UE initiates the UE Requested PDU Session establishment procedure by the transmission of a NAS message containing a PDU Session Establishment Request within the N1 SM information. The PDU Session Establishment Request may include a PDU Type, SSC mode, Protocol Configuration Options.

The Request Type indicates "Initial request" if the PDU Session Establishment is a request to establish a new PDU Session and indicates "Existing PDU Session" if the request refers to an existing PDU Session between 3GPP access and non-3GPP access or to an existing PDN Connection in EPC.

The NAS message sent by the UE is encapsulated by the AN in a N2 message towards the AMF that should include User location information and Access Technology Type Information.

The N1 SM information may contain SM PDU DN Request Container containing information for the PDU session authorization by the external DN.

The AMF receives from the AN the NAS SM message (built in step 1) together with User Location Information (e.g. Cell Id in case of the RAN).

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on that Request Type indicates "initial request" and that the PDU Session ID that is not used for any existing PDU Session(s) of the UE. If the NAS message does not contain an S-NSSAI, the AMF may determine a default S-NSSAI for the requested PDU session according to the UE subscription. The AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2. The AMF stores an association of the PDU session ID and the SMF ID.

The case where the Request Type indicates "Existing PDU Session", and either the AMF does not recognize the PDU Session ID or the subscription context from UDM does not contain an SMF ID corresponding to the DNN constitutes an error case.

3.	From AMF to SMF: Nsmf_PDUSession_CreateSMRequest (Subscriber Permanent ID, DNN, S-NSSAI, PDU Session ID, AMF ID, N1 SM information (PDU Session Establishment Request), User location information, Access Technology Type, PEI, Request Type).

The AMF ID uniquely identifies the AMF serving the UE. The AMF forwards the PDU Session ID together with the N1 SM information containing the PDU Session Establishment Request received from the UE. The PEI (Permanent Equipment Identifier) is defined in TS 23.501 [2] clause 5.9.3.

4.	SMF invokes: Nudm_SubscriberData_GetRequest  (Subscriber Permanent ID, DNN).

If the Request Type in step in step 3 indicates "Existing PDU Session" the SMF determines that the request is due to handover between 3GPP access and non-3GPP access. or due to handover from EPS. The SMF identifies the existing PDU Session based on the PDU Session ID.

If the SMF has not yet retrieved the SM-related subscription data for the UE related with the DNN, the SMF requests this subscription data.	UDM responds to SMF: with Nudm_SubscriberData_GetResponse.

Subscription data includes the authorized PDU type(s), authorized SSC mode(s), Default QoS profile.

Editor's note:	Further detailing of SM-related subscription data is FFS.

The SMF checks whether the UE request is compliant with the user subscription and with local policies. If that is not the case the SMF rejects the UE request via NAS SM signalling by responding to the AMF with Nsmf_PDUSession_CreateSMResponse including a relevant SM rejection cause. The SMF indicates to the AMF that the PDU session ID is to be considered as released and the rest of the procedure is skipped.

5.	SMF to DN via UPF

If the SMF needs perform secondary authorization/authentication during the establishment of the PDU session by a DN-AAA server as described in clause 5.6.6 of TS 23.501 [2], the SMF selects an UPF as described in TS 23.501 [2] clause 6.3.3 and triggers the PDU session establishment authentication/authorization as described in clause 4.3.2.3.

If the PDU session establishment authentication/authorization fails, the SMF terminates the PDU session establishment procedure and indicates a rejection to the UE.

6a.	If dynamic PCC is deployed, the SMF performs PCF selection.

6b.	The SMF may invoke Npcf_PolicyControl_PolicyCreateRequest including PDU Session ID to establish a PDU CAN session and get the default PCC Rules for the PDU Session. If the Request Type in step 3 indicates "Existing PDU Session", the PCF initiates a PDU-CAN Session Modification instead, by invoking the Npcf_PolicyControl_PolicyUpdateNotification service.

Editor's note:	The purpose of step 5 is to receive PCC rules before selecting UPF. If PCC rules are not needed as input for UPF selection, step 5 can be skipped.

7.	If the Request Type in step 3 indicates "Initial request", the SMF selects an SSC mode for the PDU Session. If step 5 is not performed, the SMF also selects an UPF as described in TS 23.501 [2] clause 6.3.3. In case of PDU Type IPv4 or IPv6, the SMF allocates an IP address/prefix for the PDU Session as described in TS 23.501 [2] clause 5.8.1. For Unstructured PDU Type the SMF may allocate an IPv6 prefix for the PDU session and N6 point-to-point tunnelling (based on UDP/IPv6) as described in TS 23.501 [2] clause 5.6.9.4.

If the Request Type in Step 3 is "Existing PDU Session", the SMF maintains the same IP address/prefix that has already been allocated to the UE in the source network.

8a.	SMF invokes the Npcf_PolicyControl_PolicyCreateRequest: If dynamic PCC is deployed and the PDU-CAN Session Establishment has not yet been done, the SMF invokes PCF service to establish PDU-CAN session to get the default PCC Rules for the PDU Session.

8b. SMF provides EventExposure_Notify service operation: Otherwise, if Request Type is "initial request" and dynamic PCC is deployed and PDU Type is IPv4 or IPv6, SMF notifies the PCF with the allocated UE IP address/prefix.

9.	If Request Type indicates "initial request" and step 5 was not performed, the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF:

9a.	The SMF sends an N4 Session Establishment/Modification Request to the UPF and provides Packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to UPF in this step.

9b.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If CN Tunnel Info is allocated by the UPF, the CN Tunnel Info is provided to SMF in this step.

If the Request Type indicates "Existing PDU Session", and the SMF creates CN tunnel information, then this step is skipped. Otherwise, this step is performed to obtain the CN tunnel information from the UPF using the N4 Session Modification Procedure.

10.	SMF to AMF: Nsmf_PDUSession_CreateSM Response (Cause, N2 SM information (PDU Session ID, QoS Profile(s), CN Tunnel Info, S-NSSAI), N1 SM information (PDU Session Establishment Accept (Authorized QoS Rule, SSC mode, S-NSSAI, allocated IPv4 address))).

Editor's note:	It is FFS how the AMF and SMF associate the N11 transactions with a corresponding UE context.

The N2 SM information carries information that the AMF shall forward to the (R)AN which includes:

-	The CN Tunnel Info corresponds to the Core Network address of the N3 tunnel corresponding to the PDU session.

-	The QoS Profile provides the (R)AN with the mapping between QoS parameters and QoS Flow Identifiers. Multiple QoS profiles can be provided to the (R)AN. This is further described in TS 23.501 [2] clause 5.7.

-	The PDU Session ID may be used by AN signalling with the UE to indicate to the UE the association between AN resources and a PDU session for the UE.

S-NSSAI is corresponding to the PDU session.

The N1 SM information contains the PDU Session Establishment Accept that the AMF shall provide to the UE.

Multiple Authorized QoS Rules may be included in the PDU Session Establishment Accept within the N1 SM information and in the N2 SM information.

The SM Response further contains PDU Session ID and information allowing the AMF to know which the target UE as well to determine which access towards the UE to use.

NOTE:	The access information is to deal with the case where a UE is simultaneously connected over 3GPP and Non 3GPP access.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to determine which access towards the UE to use.

11.	AMF to (R)AN: N2 PDU Session Request (N2 SM information, NAS message (PDU Session ID, PDU Session Establishment Accept)).

The AMF sends the NAS message containing PDU Session ID and PDU Session Establishment Accept targeted to the UE and the N2 SM information received from the SMF within the N2 PDU Session Request to the (R)AN.

12.	(R)AN to UE: The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE establishing the necessary RAN resources related to the Authorized QoS Rules for the PDU Session request received in step 10.

(R)AN also allocates (R)AN N3 tunnel information for the PDU Session. In case of Dual Connectivity, the Master RAN node may assign some (zero or more) QFIs to be setup to a Master RAN node and others to Secondary RAN node. The RAN tunnel information includes a tunnel endpoint for each involved RAN node, and the QFIs assigned to each tunnel endpoint. A QFI can be assigned to either the Master RAN node or the Secondary RAN node and not to both.

(R)AN forwards the NAS message (PDU Session ID, N1 SM information (PDU Session Establishment Accept)) provided in step 10 to the UE. (R)AN shall only provide the NAS message to the UE if the necessary RAN resources are established and the allocation of (R)AN tunnel information are successful.

13.	(R)AN to AMF: N2 PDU Session Response (PDU Session ID, Cause, N2 SM information (PDU Session ID, (R)AN Tunnel Info, List of accepted/rejected QoS profile(s))).

The (R)AN Tunnel Info corresponds to the Access Network address of the N3 tunnel corresponding to the PDU session.

14.	AMF to SMF: Nsmf_PDUSession_ UpdateSMContext Request (N2 SM information, Request Type).

The AMF forwards the N2 SM information received from (R)AN to the SMF.

Editor's note:	It is FFS if steps are needed to indicate from the UE to the core network that the UE has successfully established the PDU Session, or whether it is sufficient with a successful establishment in (R)AN indicated in step 12. For example, it is FFS if the UE shall send a NAS PDU Session Establishment Complete message to indicate that the UE has successfully established the PDU Session.

15a.	If the N4 session for this PDU Session was not established already, the SMF initiates an N4 Session Establishment procedure with the UPF. Otherwise, the SMF initiates an N4 Session Modification procedure with the UPF. The SMF provides AN Tunnel Info and CN Tunnel Info. The CN Tunnel Info only needs to be provided if the SMF selected CN Tunnel Info in step 8.

NOTE:	If the PDU Session Establishment Request was due to mobility between 3GPP and non-3GPP access or mobility from EPC, the downlink data path is switched towards the target access in this step.

15b.	The UPF provides a N4 Session Establishment/Modification Response to the SMF.

16.	SMF to AMF: Nsmf_PDUSession_ UpdateSMContext Response (Cause).

After this step, the AMF forwards relevant events to the SMF, e.g. at handover where the (R)AN Tunnel Info changes or the AMF is relocated.

Editor's note:	It is FFS if the SMF has to explicitly subscribe to these events or whether the subscription is implicit.

17.	SMF to UE, via UPF: In case of PDU Type IPv6, the SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the UPF.

18.	If the PDU Session Establishment request is due to a handover between 3GPP access and non-3GPP access, i.e. the Request type is set to "existing PDU Session" the SMF executes the steps X-Y specified in clause 4.3.4 in order to release the user-plane over the source access (3GPP or non-3GPP access).

Editor's note:	The exact reference to steps of network initiated PDU session release procedure will be defined when the procedure is available.

If the PDU Session Establishment request is due to a handover from EPC, the SMF (SMF+PGW-C) shall initiate the PDN GW Initiated PDN Disconnection procedure in EPC as defined in clause 5.10.3 of TS 23.401[x]

19.	If the SMF identity is not included in step 4b by the UDM in the DNN subscription context, the SMF invokes the Nudm_UEContextManagement_Registration service operation  including the SMF address and the DNN. The UDM stores the SMF identity, address and the associated DNN.

If during the procedure the PDU session establishment is not successful, the SMF informs the AMF. Once the AMF associates an SMF with a PDU session ID, the SMF is automatically subscribed to notifications of incoming N1 signalling associated with this PDU session ID. These notifications automatically provide any User Location information and Access Type that the AMF has received from the (R)AN in association with N1 signalling.

### **************  NEXT CHANGE    **********************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: " System Architecture for the 5G System; Stage  2".

[3]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[4]	IETF RFC 5998: "An Extension for EAP-Only Authentication in IKEv2".

[5]	IETF RFC 4282: "The Network Access Identifier".

[6]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[7]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[8]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[x]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

### **************  END CHANGES    **********************