SA WG2 Meeting #122bis	S2- 175787

21 - 25 August 2017, Sophia Antipolis, France	(revision of S2-17xxxx)

Source:	LG Electronics, SK Telecom

Title:	TS 23.502: Applicability of handover between 3GPP and N3GPP

Document for:	Approval

Agenda Item:	6.5.10 Non-3GPP access specific functionality and flows

Work Item / Release:	5G_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to store LBO/HR/non-roaming information in the UDM when a PDU session is established. Based on that information AMF can determine whether the handover request can be accepted or not.

Discussion

Following figures show Home-routed roaming architecture for non-3GPP access in TS 23.501.

Figure 4.2.8.2.5-1: Home-routed Roaming architecture for 5G Core Network with non-3GPP access - N3IWF in the different VPLMN from the 3GPP access

Figure 4.2.8.2.5-2: Home-routed Roaming architecture for 5G Core Network with non-3GPP access - N3IWF in HPLMN and different PLMN in 3GPP access

The handover between 3GPP and non-3GPP is triggered by the UE and the UE can decide to handover a PDU session based on radio conditions and traffic steering rules. In order to support handover, the network shall select the same SMF and UPF when the UE requested to handover a PDU session. In the previous meeting, this feature is supported by using “Existing PDU Session” indication. The AMF selects the same SMF if Request Type is set to “Existing PDU Session”.

Let’s consider the following scenario (As shown in Figure 1).

1. The UE is roaming in VPLMN1.

2. The UE established PDU session (PDU Session ID #1) via 3GPP access and it is established as LBO.

2. The UE established PDU session (PDU Session ID #2) via 3GPP access and it is established as HR.

3. The UE tries to handover PDU sessions to non-3GPP access.

4. The UE selects N3IWK in HPLMN based on N3IWK selection procedure.

5. The UE successfully registered to N3IWK via AMF in HPLMN.

6. The UE tries to handover PDU Session ID #1 over N3GPP access.

7. The AMF shall reject the UE request because the PDU session is established as LBO in VPLMN1

8. The UE tries to handover PDU Session ID #2 over N3GPP access.

9. The AMF shall accept the UE request.

10. The UE handovers PDU Session ID #2 successfully.

Figure 1 Example scenario

In step 7, the AMF shall reject the UE request because the AMF cannot select the same SMF. However, how the AMF knows whether the request should be rejected or not is unclear. The AMF cannot reject the request just because the SMF is located in VPLMN. For example, in roaming scenario, if the UE connects N3GPP access via N3IWK in VPLMN, the UE can establish a PDU session in N3GPP access with home routed roaming architecture (Please see above Figure 4.2.8.2.5-1 from TS 23.501). After that the UE may handover the PDU session via 3GPP access. The AMF in VPLMN shall not reject the request because the PDU session can be handed over to 3GPP access with home routed roaming architecture. Also the AMF should know whether the request is handover or not to decide handover applicability.

Proposal 1: It is proposed to use “handover indication” to the AMF when the UE sends PDU session establishment request for handover of a PDU session.

Possible solution is storing roaming mode (i.e. home-routed, local breakout, non-roaming) in the UDM when the PDU session is established. Based on that information AMF can determine whether the handover request can be accepted or not.

Proposal 2: It is proposed to store LBO/HR/non-roaming information in the UDM when a PDU session is established. Based on that information AMF can decide whether the handover request can be accepted or not.

Proposal

Based on the above discussion the following changes to TS 23.502 are proposed.

* * * * Start of 1st Change * * * *

4.3.2.2.1	Non-roaming and Roaming with Local Breakout

This clause specifies PDU Session establishment in the non-roaming and roaming with local breakout cases. The procedure is used to establish a new PDU session as well as to hand over an existing PDU Session between 3GPP access and non-3GPP access.

In case of roaming, the AMF determines if a PDU session is to be established in LBO or Home Routing. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

The procedure assumes that the UE has already registered on the AMF thus the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI, DNN, PDU Session ID, Request type, N1 SM information).

In order to establish a new PDU session, the UE generates a new PDU Session ID.

The UE initiates the UE Requested PDU Session establishment procedure by the transmission of a NAS message containing a PDU Session Establishment Request within the N1 SM information. The PDU Session Establishment Request may include a PDU Type, SSC mode, Protocol Configuration Options.

The Request Type indicates "Initial request" if the PDU Session Establishment is a request to establish a new PDU Session and indicates "Handover" if the request refers to handover of an existing PDU Session between 3GPP access and non-3GPP access.

The NAS message sent by the UE is encapsulated by the AN in a N2 message towards the AMF that should include User location information and Access Technology Type Information.

The N1 SM information may contain SM PDU DN Request Container containing information for the PDU session authorization by the external DN.

The AMF receives from the AN the NAS SM message (built in step 1) together with User Location Information (e.g. Cell Id in case of the RAN).

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on that Request Type indicates "initial request" and that the PDU Session ID that is not used for any existing PDU Session(s) of the UE. If the NAS message does not contain an S-NSSAI, the AMF may determine a default S-NSSAI for the requested PDU session according to the UE subscription. The AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2. The AMF stores an association of the PDU session ID and the SMF ID.

If the UE included “Handover” indication, the AMF decides whether the PDU session can be handed over to other access based on the UE context in the AMF or PDU session context from the UDM. If the PDU session cannot be handed over to the other access, the AMF rejects UE request.

The case where the Request Type indicates "Handover", and either the AMF does not recognize the PDU Session ID or the subscription context from UDM does not contain an SMF ID corresponding to the DNN constitutes an error case.

3.	From AMF to SMF: Nsmf_PDUSession_CreateSMRequest (Subscriber Permanent ID, DNN, S-NSSAI, PDU Session ID, AMF ID, N1 SM information (PDU Session Establishment Request), User location information, Access Technology Type, PEI).

The AMF ID uniquely identifies the AMF serving the UE. The AMF forwards the PDU Session ID together with the N1 SM information containing the PDU Session Establishment Request received from the UE. The PEI (Permanent Equipment Identifier) is defined in TS 23.501 [2] clause 5.9.3.

4.	SMF invokes: Nudm_SubscriberData_GetRequest (Subscriber Permanent ID, DNN).

If the Request Type in step in step 3 indicates "Handover" the SMF determines that the request is due to handover between 3GPP access and non-3GPP access. The SMF identifies the existing PDU Session based on the PDU Session ID.

If the SMF has not yet retrieved the SM-related subscription data for the UE related with the DNN, the SMF requests this subscription data.

UDM responds to SMF: with Nudm_SubscriberData_GetResponse.

Subscription data includes the authorized PDU type(s), authorized SSC mode(s), Default QoS profile.

Editor's note:	Further detailing of SM-related subscription data is FFS.

The SMF checks whether the UE request is compliant with the user subscription and with local policies. If that is not the case the SMF rejects the UE request via NAS SM signalling by responding to the AMF with Nsmf_PDUSession_CreateSMResponse including a relevant SM rejection cause. The SMF indicates to the AMF that the PDU session ID is to be considered as released and the rest of the procedure is skipped.

5.	SMF to DN via UPF

If the SMF needs perform secondary authorization/authentication during the establishment of the PDU session by a DN-AAA server as described in clause 5.6.6 of TS 23.501 [2], the SMF selects an UPF as described in TS 23.501 [2] clause 6.3.3 and triggers the PDU session establishment authentication/authorization as described in clause 4.3.2.3.

If the PDU session establishment authentication/authorization fails, the SMF terminates the PDU session establishment procedure and indicates a rejection to the UE.

6a.	If dynamic PCC is deployed, the SMF performs PCF selection.

6b.	The SMF may invoke Npcf_PolicyControl_PolicyCreateRequest including PDU Session ID to establish a PDU CAN session and get the default PCC Rules for the PDU Session. If the Request Type in step 3 indicates "Handover", the PCF initiates a PDU-CAN Session Modification instead, by invoking the Npcf_PolicyControl_PolicyUpdateNotification service.

Editor's note:	The purpose of step 5 is to receive PCC rules before selecting UPF. If PCC rules are not needed as input for UPF selection, step 5 can be skipped.

7.	If the Request Type in step 3 indicates "Initial request", the SMF selects an SSC mode for the PDU Session. If step 5 is not performed, the SMF also selects an UPF as described in TS 23.501 [2] clause 6.3.3. In case of PDU Type IPv4 or IPv6, the SMF allocates an IP address/prefix for the PDU Session as described in TS 23.501 [2] clause 5.8.1. For Unstructured PDU Type the SMF may allocate an IPv6 prefix for the PDU session and N6 point-to-point tunnelling (based on UDP/IPv6) as described in TS 23.501 [2] clause 5.6.9.4.

8a.	SMF invokes the Npcf_PolicyControl_PolicyCreateRequest: If dynamic PCC is deployed and the PDU-CAN Session Establishment has not yet been done, the SMF invokes PCF service to establish PDU-CAN session to get the default PCC Rules for the PDU Session.

8b. SMF provides EventExposure_Notify service operation: Otherwise, if Request Type is "initial request" and dynamic PCC is deployed and PDU Type is IPv4 or IPv6, SMF notifies the PCF with the allocated UE IP address/prefix.

9.	If Request Type indicates "initial request" and step 5 was not performed, the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF:

9a.	The SMF sends an N4 Session Establishment/Modification Request to the UPF and provides Packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to UPF in this step.

9b.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If CN Tunnel Info is allocated by the UPF, the CN Tunnel Info is provided to SMF in this step.

10.	SMF to AMF: Nsmf_PDUSession_CreateSM Response (Cause, N2 SM information (PDU Session ID, QoS Profile(s), CN Tunnel Info, S-NSSAI), N1 SM information (PDU Session Establishment Accept (Authorized QoS Rule, SSC mode, S-NSSAI, allocated IPv4 address))).

Editor's note:	It is FFS how the AMF and SMF associate the N11 transactions with a corresponding UE context.

The N2 SM information carries information that the AMF shall forward to the (R)AN which includes:

-	The CN Tunnel Info corresponds to the Core Network address of the N3 tunnel corresponding to the PDU session.

-	The QoS Profile provides the (R)AN with the mapping between QoS parameters and QoS Flow Identifiers. Multiple QoS profiles can be provided to the (R)AN. This is further described in TS 23.501 [2] clause 5.7.

-	The PDU Session ID may be used by AN signalling with the UE to indicate to the UE the association between AN resources and a PDU session for the UE.

S-NSSAI is corresponding to the PDU session.

The N1 SM information contains the PDU Session Establishment Accept that the AMF shall provide to the UE.

Multiple Authorized QoS Rules may be included in the PDU Session Establishment Accept within the N1 SM information and in the N2 SM information.

The SM Response further contains PDU Session ID and information allowing the AMF to know which the target UE as well to determine which access towards the UE to use.

NOTE 1:	The access information is to deal with the case where a UE is simultaneously connected over 3GPP and Non 3GPP access.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to determine which access towards the UE to use.

11.	AMF to (R)AN: N2 PDU Session Request (N2 SM information, NAS message (PDU Session ID, PDU Session Establishment Accept)).

The AMF sends the NAS message containing PDU Session ID and PDU Session Establishment Accept targeted to the UE and the N2 SM information received from the SMF within the N2 PDU Session Request to the (R)AN.

12.	(R)AN to UE: The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE establishing the necessary RAN resources related to the Authorized QoS Rules for the PDU Session request received in step 10.

(R)AN also allocates (R)AN N3 tunnel information for the PDU Session. In case of Dual Connectivity, the Master RAN node may assign some (zero or more) QFIs to be setup to a Master RAN node and others to Secondary RAN node. The RAN tunnel information includes a tunnel endpoint for each involved RAN node, and the QFIs assigned to each tunnel endpoint. A QFI can be assigned to either the Master RAN node or the Secondary RAN node and not to both.

(R)AN forwards the NAS message (PDU Session ID, N1 SM information (PDU Session Establishment Accept)) provided in step 10 to the UE. (R)AN shall only provide the NAS message to the UE if the necessary RAN resources are established and the allocation of (R)AN tunnel information are successful.

13.	(R)AN to AMF: N2 PDU Session Response (PDU Session ID, Cause, N2 SM information (PDU Session ID, (R)AN Tunnel Info, List of accepted/rejected QoS profile(s))).

The (R)AN Tunnel Info corresponds to the Access Network address of the N3 tunnel corresponding to the PDU session.

14.	AMF to SMF: Nsmf_PDUSession_ UpdateSMContext Request (N2 SM information).

The AMF forwards the N2 SM information received from (R)AN to the SMF.

Editor's note:	It is FFS if steps are needed to indicate from the UE to the core network that the UE has successfully established the PDU Session, or whether it is sufficient with a successful establishment in (R)AN indicated in step 12. For example, it is FFS if the UE shall send a NAS PDU Session Establishment Complete message to indicate that the UE has successfully established the PDU Session.

15a.	If the N4 session for this PDU Session was not established already, the SMF initiates an N4 Session Establishment procedure with the UPF. Otherwise, the SMF initiates an N4 Session Modification procedure with the UPF. The SMF provides AN Tunnel Info and CN Tunnel Info. The CN Tunnel Info only needs to be provided if the SMF selected CN Tunnel Info in step 8.

NOTE 2:	If the PDU Session Establishment Request was due to mobility between 3GPP and non-3GPP access, the downlink data path is switched towards the target access in this step.

15b.	The UPF provides a N4 Session Establishment/Modification Response to the SMF.

16.	SMF to AMF: Nsmf_PDUSession_ UpdateSMContext Response (Cause).

After this step, the AMF forwards relevant events to the SMF, e.g. at handover where the (R)AN Tunnel Info changes or the AMF is relocated.

Editor's note:	It is FFS if the SMF has to explicitly subscribe to these events or whether the subscription is implicit.

17.	SMF to UE, via UPF: In case of PDU Type IPv6, the SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the UPF.

18.	If the PDU Session Establishment request is due to a handover between 3GPP access and non-3GPP access, i.e. the Request type is set to "Handover" the SMF executes the steps X-Y specified in clause 4.3.4 in order to release the user-plane over the source access (3GPP or non-3GPP access).

Editor's note:	The exact reference to steps of network initiated PDU session release procedure will be defined when the procedure is available.

19.	If the SMF identity is not included in step 4b by the UDM in the DNN subscription context, the SMF invokes the Nudm_UEContextManagement_Registration service operation including the SMF address, roaming mode of the PDU session (e.g. Home Routed, Local Breakout, non-roaming) and the DNN. The UDM stores the SMF identity, address, roaming mode of the PDU session (e.g. Home Routed, Local Breakout, non-roaming) and the associated DNN.

If during the procedure the PDU session establishment is not successful, the SMF informs the AMF. Once the AMF associates an SMF with a PDU session ID, the SMF is automatically subscribed to notifications of incoming N1 signalling associated with this PDU session ID. These notifications automatically provide any User Location information and Access Type that the AMF has received from the (R)AN in association with N1 signalling.

* * * * Start of 2nd Change * * * *

4.3.2.2.2	Home-routed Roaming

This procedure is used in case of home-routed roaming scenarios.

Figure 4.3.2.2.2-1: UE-requested PDU Session Establishment for home-routed roaming scenarios

1.	This step is the same as step 1 in clause 4.3.2.2.1.

2.	As in step 2 of clause 4.3.2.2.1 with the addition that the AMF also selects a SMF in HPLMN. The AMF stores an association of the PDU session ID and the SMF ID in VPLMN.

3.	As in step 3 of clause 4.3.2.2.1 with the addition that the AMF also provides the identity of the SMF in HPLMN it has selected in step 2 The AMF ID uniquely identifies the AMF serving the UE. The H-SMF is provided when the PDU Session is home-routed. The N1 SM information contains the PDU Session Establishment Request received from the UE.

Editor's note:	The parameter S-NSSAI may need to be updated to reflect agreements regarding slice selection parameters. This is FFS.

4.	The V-SMF selects a UPF in VPLMN as described in TS 23.501 [2], clause 6.3.3.

5.	The V-SMF initiates an N4 Session Establishment procedure with the selected V-UPF:

5a.	The V-SMF sends an N4 Session Establishment Request to the V-UPF. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to V-UPF in this step.

5b.	The V-UPF acknowledges by sending an N4 Session Establishment Response. If CN Tunnel Info is allocated by the V-UPF, the CN Tunnel Info is provided to V-SMF in this step.

6.	V-SMF to H-SMF: Create PDU Session Request (Subscriber Permanent Identity, DNN, S-NSSAI, PDU Session ID, V-SMF ID, V-CN-Tunnel-Info, PDU type, Protocol Configuration Options, User location information, SM PDU DN Request Container).

7-12.	These steps are the same as steps 4-9 in clause 4.3.2.2.1 with the following differences:

-	These steps are executed in Home PLMN;

-	The H-SMF stores an association of the PDU Session and V-SMF ID for this PDU Session for this UE.

13.	H-SMF to V-SMF: Create PDU Session Response (Authorized QoS Rule, SSC mode), H-CN Tunnel Info)

The H-CN Tunnel Info contains the tunnel information for up-link traffic towards H-UPF.

Multiple Authorized QoS Rules may be included in the Create PDU Session Response.

The H-SMF shall send to the V-SMF the indication, if the VPLMN is allowed to insert an UL CL or a Branching Point for a PDU session towards this DNN.

14-18.	These steps are the same as steps 10-14 in clause 4.3.2.2.1 with the following differences:

-	These steps are executed in Visited PLMN;

-	The V-SMF stores an association of the PDU Session and H-SMF ID for this PDU Session for this UE.

19a.	The V-SMF initiates an N4 Session Modification procedure with the V-UPF. The V-SMF provides Packet detection, enforcement and reporting rules to be installed on the V-UPF for this PDU Session, including AN Tunnel Info, H-CN Tunnel Info and V-CN Tunnel Info.

19b.	The V-UPF provides a N4 Session Establishment/Modification Response to the V-SMF.

After this step, the V-UPF delivers any down-link packets to the UE that may have been buffered for this PDU Session.

20.	This step is the same as step 18 in clause 4.3.2.2.1. with the following differences:

-	The SMF is a V-SMF

21.	H-SMF to UE, via H-UPF and V-UPF in VPLMN: In case of PDU Type IPv6, the H-SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the H-UPF and V-UPF.

22.	H-SMF may register the address to UDM as Step 18 of 4.3.2.2.1 The UDM stores the H-SMF identity, address, roaming mode of PDU session (e.g. Home Routed, Local Breakout, non-roaming) and the associated DNN.

NOTE:	The SMF in HPLMN may initiate step 21 already after step 13.

* * * * End of Changes * * * *