SA WG2 Meeting #122bis	S2-175795

21 - 25 Aug 2017, Sophia Antipolis, France	(revision of S2-17xxxx)

Source:	ETRI

Title:	TS 23.502: Updating a Network triggered Service Request procedure to re-activate PDU session

Document for:	Approval

Agenda Item:	6.5.10

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution proposes to update a Network triggered Service Request procedure to re-activate PDU session over 3GPP access when the UE is in CM-IDLE state over non-3GPP access in TS 23.502.

Introduction

It is proposed to update the section 4.2.3.4 Network triggered Service Request procedure to re-activate PDU session over 3GPP when the UE is in CM-IDLE state over untrusted non-3GPP access in TS 23.502 to be aligned with the selective activation of PDU session in TS 23.501.

Proposal

It is proposed to update the following texts into TS 23.502.

* * * Start of changes* * * *

4.2.3.4	Network triggered Service Request

Editor's note:	Procedure includes aspects required to support network slicing, and should include notification from UPF upon MT UP data arrival.

This procedure is used when the network needs to signal (e.g. N1 signalling to UE, Mobile-terminated SMS, User Plane resource establishment for PDU session(s) to deliver mobile terminating user data) with a UE. If the UE is in CM-IDLE state or CM-CONNECTED state in 3GPP access, the network initiates a network triggered Service Request procedure. If the UE is in CM-IDLE state, and asynchronous type communication is not activated, the network sends a Paging Request to (R)AN/UE. The Paging Request triggers the UE triggered Service Request procedure in the UE. If asynchronous type communication is activated, the network stores the received message and forward the message to the (R)AN and/or the UE (i.e. synchronizes the context with the (R)AN and/or the UE) when the UE enters CM-CONNECTED state.

If the UE is in CM-CONNECTED state in non-3GPP access, the network shall initiate a Network triggered Service Request procedure when the network needs to signal with a UE. If the UE is in CM-IDLE state in non-3GPP access and if the UE is simultaneously registered over 3GPP and non-3GPP accesses in a PLMN, the network shall initiate a Network triggered Service Request procedure over 3GPP access.

Editor's note:	The name of N11 message between AMF and SMF is FFS.

For this procedure, the impacted SMF and UPF are all under control of the PLMN serving the UE, e.g. in Home Routed roaming case the SMF and UPF in HPLMN are not involved.

Figure 4.2.3.4-1: Network Triggered Service Request

1.	When UPF receives downlink data of a PDU session and there is no (R)AN tunnel information stored in UPF for the PDU session, the UPF buffers the downlink data, unless the UPF has previously been notified by the SMF to not buffer the downlink data.

2a.	UPF to SMF: Data Notification (PDU session ID, Priority).

-	On arrival of the first downlink data packet, the UPF shall send Data Notification message to the SMF, if the SMF has not previously notified the UPF to not send the Data Notification to the SMF (in which case the next steps are skipped).

-	If the UPF receives additional downlink data packets for a QoS Flow in the same PDU Session with the same or a lower priority than used in any previous Data Notification for this PDU session, the UPF buffers these downlink data packets without sending a new Data Notification.

Editor's note:	If the UPF receives additional downlink data packets for a QoS Flow in the same PDU Session with a higher priority than used in any previous Data Notification for this PDU Session, the UPF shall send a Data Notification message to the SMF indicating the higher priority.

-	If the Paging Policy Differentiation feature (as specified in TS 23.501 [2] clause 5.4.3) is supported by the UPF and if it is activated by the SMF for this N4 session, the UPF shall also include the DSCP in TOS (IPv4) / TC (IPv6) value from the IP header of the downlink data packet.

-	If the SMF, while waiting for the User Plane to be established in UPF, receives N11 message notifying the new AMF serving the UE from the new AMF, the SMF re-sends the Data Notification message only to the new AMF.

2b.	SMF to UPF: Data Notification Ack.

Editor's note:	It is FFS whether this message is needed or not.

3a.	SMF to AMF: N11 message (SUPI, PDU session ID, N2 SM information (QoS profile, CN N3 Tunnel Info), Priority).

Unless the SMF was previously notified that the UE is unreachable, or the UE is reachable only for regulatory prioritized service and the PDU session is for regulatory prioritized service, upon reception of a Data Notification message, the SMF determines the AMF and sends an N11 message (SUPI, PDU session ID, N2 SM information (PDU Session ID, QoS profile, CN N3 Tunnel Info, S-NSSAI), Priority, Paging Policy Indication) to the AMF including the Priority and PDU Session ID received in the Data Notification message as part of step 2a.

If the SMF, while waiting for the User Plane Connection to be activated, receives any additional Data Notification message for the same PDU session but with higher priority than indicated in any previous Data Notification for this PDU session, the SMF sends a new N11 message indicating the higher priority and PDU Session ID to the AMF.

If the SMF, while waiting for the User Plane to be activated, receives an N11 message response from an AMF other than the one to which the SMF sent an N11 message, the SMF sends the N11 message to this AMF.

When supporting Paging Policy Differentiation, the SMF indicates in the N11 message the Paging Policy Indication related to the downlink data that triggered the Data Notification message, as described in TS 23.501 [2] clause 5.4.3.

NOTE 1:	AMF may receive request message(s) from other network functions which leads to signalling towards UE/RAN, e.g. network initiated deregistration, SMF initiated PDU session modification. If the UE is in CM-CONNECTED state and the AMF only delivers N1 message towards UE, the flow continues in step 7 below.

Editor's note:	Procedures for the extended buffering time are FFS.

Editor's note:	Procedures for the monitoring events for "Availability after DDN failure" are FFS.

Editor's note:	It is FFS whether N2 SM information in this message is optional and when it should be carried.

3b.	[conditional] The AMF responds to the SMF.

If the UE is in CM-IDLE state, and the AMF determines that the UE is not reachable for paging, the AMF shall either send an N11 message to the SMF, or other network functions from which AMF received the request message in step 3a, indicating the UE is not reachable, or the AMF performs asynchronous type communication and store the N11 message. If asynchronous type communication is invoked and the AMF stored an N11 message, the AMF initiates communication with the UE and (R)AN when the UE is reachable e.g. when the UE enters CM-CONNECTED state.

If the AMF has determined the UE is unreachable for the SMF (e.g., due to the UE in MICO mode or the UE is only registered over non-3GPP access and its state is CM-IDLE), then the AMF rejects the request from the SMF. The AMF may include in the reject message an indication that the SMF needs not send DL data notifications to the AMF, if the SMF has not subscribed to the event of the UE reachability. The AMF stores an indication that the SMF has been informed that the UE is unreachable.

If the UE is not in MICO mode and the AMF detects the UE is in Non-allowed area unless the request from the SMF is regulatory prioritized service, the AMF reject the request from the SMF and notify the SMF that the UE is reachable only for regulatory prioritized service. The AMF stores an indication that the SMF has been informed that the UE is reachable only for regulatory prioritized service.

Editor's note:	It is FFS whether the SMF should be able to include an asynchronous indication to notify AMF whether asynchronous type communication can be used for the N11 message. The relation between asynchronous type communication and the reachability procedures in clause 4.2.5 is FFS.

If the Registration procedure with AMF change is in progress when the old AMF receives a N11 message, the old AMF may reject N11 message with an indication that the N11 message has been temporarily rejected.

Upon reception of an N11 message Ack with an indication that its requested has been temporarily rejected, the SMF shall start a locally configured guard timer and wait for any N11 message to come from new AMF. Upon reception of an N11 message from an AMF, the SMF shall send an N11 message (with Data Notification) to the AMF from which it received the N11 message. Otherwise the SMF takes the step 3c at expiry of the guard timer. If the SMF decides the control plane buffering, the SMF shall indicate UPF to start forwarding the downlink data packets towards the SMF.

Editor's note:	It is FFS whether the above mechanism can apply to LADN case.

Editor's note:	It is FFS whether further signalling optimization is to be considered in case of SMFs associated with the AMF for a given UE.

3c.	[Conditional] SMF responds to the UPF

SMF may notify the UPF about the User Plane setup failure.

If the SMF receives an indication from the AMF that the UE is unreachable or reachable only for regulatory prioritized service, the SMF may, based on network policies, either:

-	indicate to the UPF to stop sending Data Notifications and/or to stop buffering DL data or apply extended buffering, or

-	refrains from sending further N11 messages for DL data to the AMF while the UE is unreachable.

Upon receiving the information that the N11 message requested from an SMF has been temporarily rejected, and receiving the Downlink Data Notification from UPF, the SMF may request the UPF to apply extended buffering.

4.	[Conditional] AMF sends Paging message to (R)AN node.

If the UE is in CM-CONNECTED state, the AMF performs step 8 to 16 in UE triggered Service Request procedure (see clause 4.2.3.3) to activate the User Plane Connection for this PDU session (i.e. establish the radio resources and N3 tunnel) without sending a Paging message to the (R)AN node and the UE. In step 8 of clause 4.2.3.3, the AMF does not send the Service Accept message to the UE. The rest of this procedure is omitted.

If the UE is in RM-REGISTERED state and CM-IDLE and reachable, the AMF sends a Paging message (NAS ID for paging, Registration Area list, Paging DRX length, Paging Priority indication,) to (R)AN node(s) belonging to the Registration Area(s) in which the UE is registered, see TS 38.331 [xx].

If the UE is in CM-IDLE in both non-3GPP and 3GPP accesses, the AMF may send a Paging message over 3GPP access to re-activate PDU session associated to the non-3GPP access. If the UE is in CM–IDLE in non-3GPP access and in CM-CONNECTED in 3GPP access, the AMF may send a Notification message over 3GPP access to re-activate PDU session associated to the non-3GPP access.

When supporting Paging Policy Differentiation, the AMF may include Paging Policy Indication in Paging Request message as described in TS 23.501 [2] clause 5.4.3

Paging strategies may be configured in the AMF for different combinations of DNN, Paging Policy Indication, PDU Session IDs from SMF when available (see clause 4.9) and other PDU Session context information identified by PDU Session ID received in N11 message.

Paging strategies may include:

-	paging retransmission scheme (e.g. how frequently the paging is repeated or with what time interval);

-	determining whether to send the Paging message to the (R)AN nodes during certain AMF high load conditions;

-	whether to apply sub-area based paging (e.g. first page in the last known cell-id or TA and retransmission in all registered TAs).

The AMF and the (R)AN may support further paging optimisations in order to reduce the signalling load and the network resources used to successfully page a UE by one or several of the following means:

-	by the AMF implementing specific paging strategies (e.g. the N2 Paging message is sent to the (R)AN nodes that served the UE last);

-	by the AMF considering Information On Recommended Cells And RAN nodes provided by the (R)AN at transition to CM-IDLE state. The AMF takes the (R)AN nodes related part of this information into account to determine the (R)AN nodes to be paged, and provides the information on recommended cells within the N2 Paging message to each of these (R)AN nodes;

-	by the (R)AN considering the Paging Attempt Count Information provided by the AMF at paging.

If the UE Radio Capability for Paging Information is available in the AMF, the AMF adds the UE Radio Capability for Paging Information in the N2 Paging message to the (R)AN nodes.

If the Information On Recommended Cells And (R)AN nodes For Paging is available in the AMF, the AMF shall take that information into account to determine the (R)AN nodes for paging and, when paging a (R)AN node, the AMF may transparently convey the information on recommended cells to the (R)AN node.

The AMF may include in the N2 Paging message(s) the paging attempt count information. The paging attempt count information shall be the same for all (R)AN nodes selected by the AMF for paging.

Editor's note:	Procedures for enhanced coverage are FFS

Editor's note:	Procedures and parameters for extended DRX are FFS.

Editor's note:	Procedures and parameters for CSG paging are FFS depending on whether CSG will be supported.

Editor's note:	The need to send Paging Cause to (R)AN and UE is FFS.

5.	[Conditional]The (R)AN node pages the UE.

If (R)AN nodes receive paging messages from the AMF, the UE is paged by the (R)AN node, see TS 38.331 [xx].

6.	[Conditional] AMF to SMF: N11 message ACK.

The AMF supervises the paging procedure with a timer. If the AMF receives no response from the UE to the Paging Request message, the AMF may apply further paging according to any applicable paging strategy described in step 3.

If the AMF receives no response from the UE, the AMF considers the UE as unreachable and the SM N2 message cannot be routed to the (R)AN, so the AMF shall return an "N11 message Reject" with an appropriate "failure cause", e.g. UE unreachability, to SMF or other network functions to indicate the failure of "message routing service", unless the AMF is aware of an ongoing MM procedure that prevents the UE from responding, i.e. the AMF receives an N14 Context Request message indicating that the UE performs Registration procedure with another AMF.

Editor's note:	Inter-system procedures e.g. EPC to 5G CN are FFS.

When an "N11 message Reject" is received, SMF informs the UPF.

Editor's note:	Procedure to pause of charging by SMF is FFS.

7.	When UE is in the CM-IDLE state, upon reception of paging request, the UE initiates the UE triggered Service Request procedure (clause 4.2.3.2). In step 4a of clause 4.2.3.2, the AMF sends N11 message to SMF(s) associated with the PDU session identified in MM NAS Service Request message if there is any, but not to SMF(s) from which it receives the N11 message in step 3a.

When UE is in the CM-IDLE state in both non-3GPP and 3GPP accesses, upon reception of paging request with the access associated to the PDU session, the UE initiates the UE triggered Service Request procedure (clause 4.2.3.2) to re-activate the PDU session over 3GPP access. When UE is in the CM–IDLE state in non-3GPP access and in the CM-CONNECTED state in 3GPP access, upon reception of notification message with the PDU session ID, the UE initiates the UE triggered Service Request procedure (clause 4.2.3.3) to indicate on whether the PDU session can be re-activated over 3GPP access.

Editor's note: 	The clause 4.2.3.2 and the clause 4.2.3.3 should be updated to re-activate PDU session over 3GPP access when the UE is in CM-IDE state over non-3GPP access.

8.	The UPF transmits the buffered downlink data toward UE via (R)AN node which performed the Service Request procedure.

Network sends downlink signalling if the procedure is triggered due to request from other network entities described in step 3a.

* * * End of Changes * * *