SA WG2 Meeting #122bis	S2-176001

August 21 – 25, 2017, Sophia Antipolis, France	(was S2-17----)

Source:	CATT

Title:	Update to UDM service

Document for:	Discussion

Agenda Item:	6.5.11

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution supplements Nudm_EventExposure service in NF Services provided by UDM.

- Discussion

In Table 7.2.5-1: NF Services provided by UDM of TS 23.501 V1.2.0 , there is no Nudm_EventExposure service.

- Proposal

It is proposed to agree on the proposed changes to TS 23.501.

*************** Start of change*********************

7.2.5	UDM Services

The following NF services are specified for UDM:

Table 7.2.5-1: NF Services provided by UDM

| Service Name | Description | Reference in TS 23.502 [3] | Example Consumer |
| --- | --- | --- | --- |
| Nudm_UE context management | 1. provide the NF consumer of the information related to UE's transaction information, e.g. UE's serving NF identity, UE status, etc. 2. allow the NF consumer to register, remove its information for the serving UE in the UDM | 5.2.3 | AMF, SMF, SMSF |
| Nudm_Subscriber data management | 1. Allow NF consumer to retrieve user subscription data when necessary 2. Provide updated user subscriber data to the subscribed NF consumer; | 5.2.3 | AMF, SMF |
| Nudm_Authentication | 1. Provide updated authentication related subscriber data to the subscribed NF consumer. | 5.2.3 | AUSF |
| Nudm_EventExposure | - The service enables the NF consumer to subscribe or get notified of | 4.2.5.2 | NEF, SMSF |

*************** End of change *********************