SA WG2 Meeting #122Bis	S2-176378

21 - 25 August 2017, Sophia Antipolis, France                     revision of 5508

Source:	China Mobile, Verizon

Title:	23.502 on NF service discovery procedures by NRF

Document for:	Discussion and Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: this contribution propose to move the NF/NF service discovery procedures from clause 5.2.6 to clause 4 and clarifies further on the procedures.

- Introduction

In SA2#121, the NF service discovery were approved in section 6.3.1 of TS23.501.

This proposal updates the system procedure and NRF services to support NF/NF service discovery. Some part of this proposal also motivated from S2-175060

- Proposal

* * * First Change * * *

All the text in the first change in this section is new but the changes are shown over original 5.2.6.1.2

4.x	Network Function Service Framework Procedure

4.x.1	Non-roaming scenario NF Service Discovery

Figure 4.x.1-1: NF/NF Service Discovery in the same PLMN

1.	The NF service consumer intends to discover services available in the network based on service name and target NF type. The NF service consumer invokes Nnrf_NFDiscovery_Request (Expected NF Service Name, NF Type of the expected NF instance, NF type of the NF consumer) from NRF in the same PLMN. The parameter may include optionally S-NSSAI, and other service related parameters.

NOTE:	Other service related parameters need depend on the NF type of the expected NF instance(s) and refer to the clause 6.2 "Network Function Selection" in TS 23.501 [2].

2.	The NRF authorizes the NF Service Discovery Request. Based on the profile of the expected NF/NF Service and the type of the NF service consumer, the NRF determines whether the NF service consumer is allowed to discover the expected NF instance(s) or NF service instance(s). If the expected NF instance(s) or NF service instance(s) are deployed in a certain network slice, NRF authorizes the discovery request according to the discovery configuration of the Network Slice, e.g. the expected NF instance(s) are only discoverable by the NF in the same network slice.

3.	If allowed, the NRF determines the discovered NF instance(s) or NF service instance(s) and provides the information of a set of discovered NF instance(s) or NF service instance(s) to the NF service consumer via NF Service Discovery Response message. The information includes: FQDN or IP address for each service end point.

NOTE: Service consumer need not be aware if information provided by NRF is based on NF instance(s) or NF service instance(s).

4.x.2	Roaming scenario NF Service Discovery Procedure

In case that the NF service consumer intends to discover the NF/NF service in home PLMN, the NRF in serving PLMN needs to requests "NF Discovery" service from NRF in the home PLMN. The procedure is depicted in the figure below:

Figure 4.x.2-1: NF/NF Service Discovery across PLMNs

1.	The NF service consumer invokes Nnrf_NFDiscovery_Request (Expected Service Name, NF type of the expected NF, the home PLMN ID, NF type of the NF service consumer). The request may also inclcude optionally S-NSSAI, and other service related parameters.

2.	The NRF in serving PLMN identifies NRF in home PLMN based on the home PLMN ID, and it requests "NF Discovery" service from NRF in home PLMN according the procedure in Figure 4.x.1-1 to get the expected NF instance (s) or NF service instance(s) deployed in the home PLMN. As the NRF in the serving PLMN triggers the "NF Discovery" on behalf of the NF service consumer, the NRF in the serving PLMN shall not replace the information of the service requester NF, i.e. NF consumer ID, in the Discovery Request message it sends to the NRF in home PLMN.

NOTE: Due to network topology hiding or network configuration, the home NRF may provide information of the IP address or the FQDN of a proxy function of the discovered NF/NF service to the serving NRF.

3.	The NRF in serving PLMN provides the information e.g. FQDN and IP address, of a set of the discovered NF or NF service instance(s) in NF Discovery Response message.

* * * Second Next Change * * *

5.2.6	NRF services

5.2.6.1	General

The following table shows the NRF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NRF

| Service Name | Service Operations | Known Consumer(s) | Operation Semantic |
| --- | --- | --- | --- |
| Nnrf_NFDiscovery | Request | AMF, SMF, PCF, NEF, NRF, NSSF | Request/ Response |

5.2.6.2	Nnrf_NFDiscovery service

5.2.6.2.1	General

Service description: This service enables one NF to discover a set of NF instances with specific NF service or a target NF type. The service also enables one NF service to discover a specific NF service. The service operations defined below allow the NF/NF services to communicate with NRF.

5.2.6.2.2	Nnrf_NFDiscovery service operation

Service operation name: Nnrf_NFDiscovery_Request

Description: provides the IP address or FQDN of the expected NF instance(s) or Service instance(s) to the NF service consumer.

Known NF Service Consumers: AMF, SMF, PCF, NEF, NRF in Serving PLMN, NSSF, UDM, AUSF

Inputs, Required: target NF Service Name, NF type of the target NF, NF type of the NF service consumer.

Inputs, Optional: S-NSSAI, target NF/NF service PLMN ID, Serving PLMN ID, the NF service consumer ID.

Outputs, Required: FQDN and/or IP address(es) for the target Service Name.

FQDN and IP addresses may belong to of a set of requested target NF instance(s), or NF service instance (s).

Outputs, Optional: none.

See clause 4.x.1/4.x.2 for details on the usage of this service operation.

* * * End of Changes * * *