SA WG2 Meeting #122bis	S2- 176487

21 - 25 August 2017, Sophia Antipolis, France	(revision of S2-175786)

Source:	LG Electronics

Title:	TS 23.501: Clarification on QoS parameters sent to the UE

Document for:	Approval

Agenda Item:	6.5.5 QoS concept and functionality

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution is proposed to clarify which QoS parameters are sent to the UE.

Discussion

According to TS 23.501 5.7.1, some QoS parameters are included in the QoS rule and sent to the UE as follows:

The UE performs the classification and marking of UL User plane traffic, i.e. the association of uplink traffic to QoS flows, based on QoS rules. These rules may be explicitly provided to the UE (at PDU Session establishment or QoS flow establishment), pre-configured in the UE or implicitly derived by UE by applying reflective QoS. A QoS rule contains a QoS rule identifier which is unique within the PDU session, the QFI of the associated QoS flow, one or more packet filters and a precedence value. Additionally, for an assigned QFI, the QoS rule contains the QoS parameters relevant to the UE. There can be more than one QoS rule associated with the same QoS Flow (i.e. with the same QFI).

However, it is not clear which QoS parameters are relevant to the UE.

In EPS, following parameters are sent to the UE.

-	QCI

-	Maximum bit rate for uplink / downlink

-	Guaranteed bit rate for uplink / downlink

-	APN-AMBR

It is proposed to send QoS parameters to the UE in 5GS as follows which is similar with EPS.

| QoS parameters | UE | AN | UPF |
| --- | --- | --- | --- |
| 5QI | O | O | O |
| ARP | X | O | O |
| GFBR for UL/DL | O | O | O |
| MFBR for UL/DL | O | O | O |
| Notification control | X | O | X |
| UE-AMBR | X | O | X |
| Session-AMBR | O | O | O |
| RQA | X | O | X |

Also wrong clause number which is referencing a removed clause is updated.

Proposal

* * * * Start of 1st Change * * * *

5.7.1	General Overview

The 5G QoS model supports a QoS flow based framework. The 5G QoS model supports both QoS flows that require guaranteed flow bit rate and QoS flows that do not require guaranteed flow bit rate. The 5G QoS model also supports reflective QoS (see clause 5.7.5).

The QoS flow is the finest granularity of QoS differentiation in the PDU session. A QoS Flow ID (QFI) is used to identify a QoS flow in the 5G system. User Plane traffic with the same QFI within a PDU session receives the same traffic forwarding treatment (e.g. scheduling, admission threshold). The QFI is carried in an encapsulation header on N3 (and N9) i.e. without any changes to the e2e packet header. It can be applied to PDUs with different types of payload, i.e. IP packets, unstructured PDUs and Ethernet frames. The QFI shall be unique within a PDU session.

NOTE 1:	Policing of User Plane traffic (e.g. MFBR enforcement) is not regarded as QoS differentiation and is done by UPFs on an SDF level granularity.

Within the 5GS, QoS flows are controlled by the SMF at PDU session establishment or at QoS flow establishment/modification.

When applicable, any QoS Flow is characterised by:

-	A QoS profile provided by the SMF to the AN via the AMF over the N2 reference point or preconfigured in the AN;

-	One or more network-provided QoS rule(s) provided by the SMF to the UE via the AMF over the N1 reference point and/or one or more UE derived QoS rule(s); and

-	SDF Classification and QoS related information (e.g. Session-AMBR) provided by the SMF to the UPF over the N4 reference point.

A QoS Flow may either be 'GBR' or 'Non-GBR' depending on its QoS profile. The QoS profile of a QoS flow contains QoS parameters of this flow as described below (details of QoS parameters are described in clause 5.7.2):

-	For each QoS flow, the QoS parameters shall include

-	A 5G QoS Identifier (5QI); and.

-	An Allocation and Retention Priority (ARP).

-	In case of a GBR QoS flow only, the QoS parameters shall also include:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL;

-	Maximum Flow Bit Rate (MFBR) - UL and DL; and

-	Notification control.

In case of Non-GBR QoS flow only, the QoS parameters may also include:-	Reflective QoS Attribute (RQA).

Three options to control QoS flows are supported:

1)	For non-GBR QoS flows only, and when standardized 5QIs or pre-configured 5QIs are used, the 5QI value can be used as the QFI of the QoS flow. The default ARP shall be pre-configured in the AN;

NOTE 2:	The above option is intended to be used for non-3GPP ANs (e.g. Fixed AN) scenarios when there is no need for any N1 signalling including PDU session establishment, nor any N2 signalling.

2)	For non-GBR QoS flows only, and when standardized 5QIs or pre-configured 5QIs are used, the 5QI value can be used as the QFI of the QoS flow. The default ARP is sent to RAN at PDU Session establishment and when NG-RAN is used every time the User Plane of the PDU session is activated; and

3)	For GBR and non-GBR QoS flows, an assigned QFI shall be used. The 5QI value may be a standardized, pre-configured or non-standardized.The QoS profile and the QFI of a QoS flow is provided to the (R)AN over N2 at PDU Session establishment or at QoS flow establishment/modification and when NG-RAN is used every time the User Plane of the PDU session is activated. .

NOTE 3:	The options 2 and 3 are intended to be used for 3GPP ANs.

NOTE 4:	Pre-configured 5QI values cannot be used when the UE is roaming. Whenever a 5QI value has not been pre-configured in the (R)AN and not signalled to the (R)AN, the QoS characteristics of the standardized 5QI with the lowest priority level applies.

The UE performs the classification and marking of UL User plane traffic, i.e. the association of uplink traffic to QoS flows, based on QoS rules. These rules may be explicitly provided to the UE (at PDU Session establishment or QoS flow establishment), pre-configured in the UE or implicitly derived by UE by applying reflective QoS. A QoS rule contains a QoS rule identifier which is unique within the PDU session, the QFI of the associated QoS flow, one or more packet filters and a precedence value. Additionally, for an assigned QFI, the QoS rule contains the QoS parameters relevant to the UE. There can be more than one QoS rule associated with the same QoS Flow (i.e. with the same QFI).

A default QoS rule is required for every PDU session. The default QoS rule is the only QoS rule of a PDU session that may contain no packet filter (in this case, the highest precedence value (i.e. lowest priority) has to be used). If the default QoS rule does not contain a packet filter, the default QoS rule defines the treatment of packets that do not match any other QoS rule in a PDU session.

Editor's note:	It is FFS whether there, in addition, is a need for pre-authorized QoS rules be provided to the UE.

The SMF performs the binding of SDFs to QoS flows based on the QoS and service requirements of the SDF (e.g. the received PCC rules). The SMF assigns the QFI for a new QoS flow and derives its QoS parameters from the information provided by the PCF. When applicable, the SMF provides the QFI together with the QoS profile to the (R)AN. The SMF provides the SDF template (i.e. the set of packet filters associated with the SDF received from the PCF) together with the SDF precedence, the QoS related information, and the corresponding packet marking information, i.e. the QFI, the DSCP value and optionally the Reflective QoS Indication to the UPF enabling classification, bandwidth enforcement and marking of User Plane traffic. When applicable, the SMF generates the QoS rule(s) for the PDU Session by allocating QoS rule identifiers, which are unique within the PDU session adding the QFI of the QoS flow, setting the packet filter(s) to the UL part of the SDF template and setting the QoS rule precedence to the SDF precedence. The QoS rules are then provided to the UE enabling classification and marking of UL User Plane traffic.

Editor's note:	Some applications, e.g. IMS, require also the DL part of the SDF template in the QoS rule. Whether the DL of the SDF template has to be sent for every QoS rule is FFS.

The principle for classification and marking of User Plane traffic and mapping of QoS Flows to AN resources is illustrated in Figure 5.7.1-1.

Figure 5.7.1-1: The principle for classification and User Plane marking for QoS Flows and mapping to AN Resources

In DL incoming data packets are classified based on SDF templates according to their SDF precedence (without initiating additional N4 signalling). The CN conveys the classification of the User Plane traffic belonging to a QoS flow through an N3 (and N9) User Plane marking using a QFI. The AN binds QoS flows to AN resources (i.e. Data Radio Bearers in case of in case of 3GPP RAN). There is no strict 1:1 relation between QoS flows and AN resources. It is up to the AN to establish the necessary AN resources to map the QoS flows to DRBsso that the UE receives the QFI (and reflective QoS (see clause 5.7.5) may be applied).

If no match is found and all QoS flows are related with one or more downlink packet filters, the UPF shall discard the downlink data packet.

In UL, the UE evaluates UL packets against the packet filters in the QoS rules based on the precedence value of QoS rules in increasing order until a matching QoS rule (i.e. whose packet filter matches the UL packet) is found. The UE uses the QFI in the corresponding matching QoS rule to bind the UL packet to a QoS Flow. The UE then binds QoS flows to AN resources.

If no match is found and the default QoS rule contains one or more uplink packet filters, the UE shall discard the uplink data packet.

The following characteristics apply for processing of Downlink traffic:

-	UPF maps User Plane traffic to QoS flows based on the SDF templates

-	UPF performs Session-AMBR enforcement and also performs PDU counting for support of charging.

-	UPF transmits the PDUs of the PDU session in a single tunnel between 5GC and (R)AN, the UPF includes the QFI in the encapsulation header. In addition, UPF may include an indication for reflective QoS activation in the encapsulation header.

-	UPF performs transport level packet marking in downlink, e.g. setting the DiffServ Code point in outer IP header. Transport level packet marking may be based on the 5QI and ARP of the associated QoS flow.

-	(R)AN maps PDUs from QoS flows to access-specific resources based on the QFIand the associated 5G QoS characteristics and parameters, also taking into account the N3 tunnel associated with the downlink packet.

NOTE 4:	Packet filters are not used for the mapping of QoS flows onto access-specific resources in (R)AN.

-	If reflective QoS applies, the UE creates a new derived QoS rule. The packet filter in the derived QoS rule is derived from the (i.e. the header of the) DL packet, and the QFI of the derived QoS rule is is set according to the QFI of the DL packet.

Following characteristics apply for processing of uplink traffic:

-	UE uses the stored QoS rules to determine mapping between UL User Plane traffic and QoS flows. UE marks the UL PDU with the QFI of the QoS rule containing the matching packet filter and transmits the UL PDUs using the corresponding access specific resource for the QoS flow based on the mapping provided by RAN.

-	(R)AN transmits the PDUs over N3 tunnel towards UPF. When passing an UL packet from (R)AN to CN, the (R)AN includes the QFI value, in the encapsulation header of the UL PDU, and selects the N3 tunnel.

-	(R)AN performs transport level packet marking in the uplink, transport level packet marking may be based on the 5QI and ARP of the associated QoS Flow.

-	UPF verifies whether QFIs in the UL PDUs are aligned with the QoS Rules provided to the UE or implicitly derived by the UE (e.g. in case of reflective QoS).

-	UPF performs Session-AMBR enforcement and counting of packets for charging.

For UL Classifier PDU sessions, UL and DL Session-AMBR shall be enforced in the UPF that supports the UL Classifier functionality. In addition, the DL Session-AMBR shall be enforced separately in every UPF that terminates the N6 interface (i.e. without requiring interaction between the UPFs) (see clause 5.6.4).

For multi-homed PDU sessions, UL and DL Session-AMBR shall be enforced in the UPF that supports the Branching Point functionality. In addition, the DL Session-AMBR shall be enforced separately in every UPF that terminates the N6 interface (i.e. without requiring interaction between the UPFs) (see clause 5.6.4).

NOTE 5:	The DL Session-AMBR is enforced in every UPF terminating the N6 interface to reduce unnecessary transport of traffic which may be discarded by the UPF performing the UL Classifier/Branching Point functionality due to the amount of the downlink traffic for the PDU session exceeding the DL Session-AMBR. Discarding DL packets in the UL Classifier/Branching Point could cause erroneous PDU counting for support of charging

The (R)AN shall enforce Max BitRate (UE-AMBR) limit in UL and DL per UE for non-GBR QoS flows. The UE shall perform UL rate limitation on PDU Session basis for non-GBR traffic using Session-AMBR, if the UE receives a session-AMBR.

Rate limit enforcement per PDU session applies for flows that do not require guaranteed flow bit rate. MBR per SDF is mandatory for GBR QoS flows but optional for non-GBR QoS flows. The MBR is enforced in the UPF.

The QoS control for Unstructured PDUs is performed at the PDU session level. When a PDU session is set up for transferring unstructured PDUs, SMF provides the QFI which will be applied to any packet of the PDU session to the UPF and UE.

Editor's note:	Whether and how the QoS flow level QoS control is supported for unstructured PDUs is FFS.

5.7.2	5G QoS Parameters

A 5QI is a scalar that is used as a reference to 5G QoS characteristics defined in clause 5.7.4, i.e. access node-specific parameters that control QoS forwarding treatment for the QoS flow (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

The 5QI in the standardized value range have one-to-one mapping to a standardized combination of 5G QoS characteristics as specified in Table 5.7.4-1.

For non-standardized combinations of 5G QoS characteristics, a 5QI value from the non-standardized value range together with the 5G QoS characteristics are signalled over N2, N11 and N7 at the time of PDU session or QoS flow establishment and in addition, over N2 every time the User Plane of the PDU session is activated.

NOTE 1:	On N3, each PDU (i.e. in the tunnel used for the PDU session) is associated with one 5QI via the QFI carried in the encapsulation header.

The QoS parameter ARP contains information about the priority level, the pre-emption capability and the pre-emption vulnerability. The priority level defines the relative importance of a resource request. This allows deciding whether a new QoS flow may be accepted or needs to be rejected in case of resource limitations (typically used for admission control of GBR traffic). It may also be used to decide which existing QoS flow to pre-empt during resource limitations.

The range of the ARP priority level is 1 to 15 with 1 as the highest level of priority. The pre-emption capability information defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level. The pre-emption vulnerability information defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level. The pre-emption capability and the pre-emption vulnerability shall be either set to 'yes' or 'no'.

The ARP priority levels 1-8 should only be assigned to resources for services that are authorized to receive prioritized treatment within an operator domain (i.e. that are authorized by the serving network). The ARP priority levels 9-15 may be assigned to resources that are authorized by the home network and thus applicable when a UE is roaming.

NOTE 2:	This ensures that future releases may use ARP priority level 1-8 to indicate e.g. emergency and other priority services within an operator domain in a backward compatible manner. This does not prevent the use of ARP priority level 1-8 in roaming situation in case appropriate roaming agreements exist that ensure a compatible use of these priority levels.

The Reflective QoS Attribute (RQA) is an optional parameter that, in the case of NG-RAN, may be signalled to RAN via N2 when Reflective QoS control is used, as described in clause 5.7.5.4 The RQA indicates that certain traffic on this QoS flow may be subject to Reflective QoS.

In addition, the QoS flow may be associated with the parameter:

-	Notification control.

The Notification control may be provided for GBR QoS flows. The Notification control indicates whether notification should be made by the RAN if the QoS targets cannot be fulfilled for a QoS flow during the lifetime of the QoS flow. If it is set and QoS targets cannot be fulfilled, RAN sends a notification towards SMF.

Editor's note:	It is FFS whether the Notification control is needed for non GBR QoS flows.

For GBR QoS flows, the 5G QoS profile additionally include the following QoS parameters:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL;

-	Maximum Flow Bit Rate (MFBR) -- UL and DL.

The GFBR denotes the bit rate that may be expected to be provided by a GBR QoS flow. The MFBR limits the bit rate that may be expected to be provided by a GBR QoS flow (e.g. excess traffic may get discarded by a rate shaping function).

GFBR and MFBR are signalled on N2 and N11 for each of the GBR QoS Flows for setting up the 5G QoS profile.

The MBR (and GBR, if applicable) per SDF, based on the information received from PCF, is signalled on N7 and N4.

Each PDU Session of a UE is associated with the following aggregate rate limit QoS parameter:

-	per Session Aggregate Maximum Bit Rate (Session-AMBR).

The subscribed Session-AMBR is a subscription parameter which is retrieved by the SMF from UDM. SMF may use the subscribed Session-AMBR or modify it based on local policy or use the authorized Session-AMBR received from PCF to get the Session-AMBR , which is signalled to the appropriate UPF entity/ies (and to the (R)AN to enable the calculation of the UE-AMBR). The Session-AMBR limits the aggregate bit rate that can be expected to be provided across all Non-GBR QoS flows for a specific PDU session.

Each UE is associated with the following aggregate rate limit QoS parameter:

-	per UE Aggregate Maximum Bit Rate (UE-AMBR).

The UE-AMBR limits the aggregate bit rate that can be expected to be provided across all Non-GBR QoS flows of a UE. Each (R)AN shall set its UE-AMBR to the sum of the Session-AMBR of all PDU Sessions with active user plane to this (R)AN up to the value of the subscribed UE-AMBR. The subscribed UE-AMBR is a subscription parameter which is retrieved from UDM and provided to the (R)AN by the AMF.

For each PDU Session, SMF retrieves the default 5QI/ARP from UDM. The default 5QI/ARP is used by SMF to set QoS parameters for the QoS flow corresponding to the default QoS rule. The SMF may change the default 5QI/ARP based on local configuration or interaction with the PCF (to retrieve the authorized default 5QI/ARP which overrides the default 5QI/ARP).

The SMF sends some of QoS parameters to the UE. During the PDU session establishment procedure, the SMF provides Session-AMBR. Other QoS parameters required by the UE (e.g. 5QI, RQA, GFBR and MFBR) is provided to the UE when relevant QoS flow is established or modified.

NOTE 3:	In case of non-standardized 5QI, the SMF also provides related QoS characteristics defined by the operator.

* * * * End of Changes * * * *