SA WG2 Meeting #S2-122	S2-174117

26 - 30 June 2017, San Jose Del Cabo, Mexico

3GPP TSG RAN WG2 Meeting #98			 R2-1706143

# Hangzhou, China, 15th-19th May 2017

Title:	Reply LS to SA2 on QoS Parameters

Response to:	R2-1702492 (S2-171618)

Release:	Rel-15

Work Item:	NR_newRAT

Source:	RAN2

To:	SA2

Cc:	RAN3

Contact Person:

#### Name:	Henrik Enbuske

E-mail Address:	henrik dot enbuske at ericsson dot com

1. Overall Description:

RAN2 thanks SA2 for the LS on 5GS QoS framework and parameters. RAN2 discussed the need for additional or clarified QoS parameters from a radio interface point of view as a result of the SA2 LS and would like to provide the following feedback.

From RAN2 perspective the existing QoS parameters, and in particular the concept of QCI/5QI to abstract QoS requirements between CN and RAN should be maintained in NR/NGC.

Regarding the related information per QoS flow in the SA2 LS, RAN2 would like to recommend SA2 to clarify the use and corresponding actions from CN/RAN related to the notification control to CN/RAN and their influence to the RAN design if the QoS targets cannot be fulfilled in RAN. For example, if the gNB is expected to release a corresponding bearer/QoS flow for which a notification control to the CN is sent, and for which a response or action have not been received from the CN. RAN2 also recommends SA2 to consider clarifying how to interpret the Packet Delay Budget and Packet Error Loss Rate requirement of a certain QoS flow to fulfil reliability requirements, and specifically for delay critical services like URLLC in cases when packets are delayed more than the PDB.

RAN2 also discussed company proposals on additional QoS parameters and characteristics that may be relevant to RAN and as a result RAN2 would like SA2 to consider specifying a characteristics parameter defining an Averaging window.  The Averaging window should be defined as the data rate offered to each QoS flow over a moving averaging window. The RAN may use this filtered value of the data rate to determine whether it fulfils the GFBR.

2. Actions:

To SA2

ACTION: 	RAN2 kindly asks SA2 to take information above into account and provide feedback if any.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting NR#2		27 - 29 June 2017,			Qingdao, China

TSG RAN2 Meeting #99		21 - 25 August 2017,		Berlin, Germany