3GPP TSG-SA2 Meeting #122	S2-174175

# San Jose Del Cabo, Mexico, 26th Jun 2017 - 30th Jun 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.682 | CR | 0306 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of SCEF description (clause 4.4.8) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NAPS |  |  |  |  |  | Date: |  |  | 2017-06-25 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The desciption of SCEF in clause 4.4.8 focuses on the SCEF frameworks functions and omits the actual services and capabilities exposed by the SCEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds a list of services and capabilities exposed by the SCEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## Start of changes

### 4.4.8	Service Capability Exposure Function

The Service Capability Exposure Function (SCEF) provides a means to securely expose the services and capabilities provided by 3GPP network interfaces. The SCEF provides a means for the discovery of the exposed services and capabilities. The SCEF provides access to network capabilities through homogenous network application programming interfaces (e.g. Network APIs) defined over T8 interface. The SCEF abstracts the services from the underlying 3GPP network interfaces and protocols.

Individual instances of SCEF may vary depending on what service capabilities are exposed and what API features are supported.

The SCEF is always within the trust domain. An application can belong to the trust domain or may lie outside the trust domain.

The exposure framework functionality of the SCEF may include the following:

-	Authentication and Authorization:

-	Identification of the API consumer,

-	Profile management,

-	ACL (access control list) management.

NOTE 1:	The details of security aspects of T8 interface are outside the scope of this specification.

-	Ability for the external entities to discover the exposed service capabilities

-	Policy enforcement:

-	Infrastructural Policy: policies to protect platforms and network. An example of which maybe ensuring that a service node such as SMS-SC is not overloaded.

-	Business Policy: policies related to the specific functionalities exposed. An example may be number portability, service routing, subscriber consent etc.

-	Application Layer Policy: policies that are primarily focused on message payload or throughput provided by an application. An example may be throttling.

-	Assurance:

-	Integration with O&M systems,

-	Assurance process related to usage of APIs.

-	Accounting for inter operator settlements.

NOTE 2:	The details of accounting aspects of T8 interface are outside the scope of this specification.

-	Access: issues related to external interconnection and point of contact

-	Abstraction: hides the underlying 3GPP network interfaces and protocols to allow full network integration. The following functions are among those that may be supported:

-	Underlying protocol connectivity, routing and traffic control,

-	Mapping specific APIs onto appropriate network interfaces,

-	Protocol translation.

NOTE 3:	Abstraction is applied only in cases where required functionality is not natively provided by 3GPP network

The services and capabilities exposed by the SCEF include:

-	Group Message Delivery (see clause 4.5.5),

-	Monitoring events (see clause 4.5.6),

-	High latency communication (see clause 4.5.7),

-	Informing about potential network issues (see clause 4.5.8),

-	Resource management of background data transfer (see clause 4.5.9),

-	E-UTRAN network resource optimizations based on communication patterns provided to the MME (see clause 4.5.10),

-	Support of setting up an AS session with required QoS (see clause 4.5.11),

-	Change the chargeable party at session set-up or during the session (see clause 4.5.12),

-	Non-IP Data Delivery (see clause 4.5.14),

-	Packet Flow Description management (see clause 4.5.15),

-	Enhanced Coverage restriction control (see clause 4.5.17),

-	Network Parameter Configuration (see clause 4.5.20),

-	Accessing MTC-IWF Functionality (Device Triggering and  MSISDN-less MO-SMS) via T8 (see clause 5.17),

The SCEF shall protect the other PLMN entities (e.g. HSS, MME) from requests exceeding the permission arranged in the SLA with the third-party service provider.

When needed, the SCEF supports mapping between information exchanged with SCS/AS (e.g. geographical identifiers) and information exchanged with internal PLMN functions (e.g. cell-Id / ENB-Id / TAI / MBMS SAI , etc.). This mapping is assumed to be provided by the SCEF based on local configuration data.

## End of changes