SA WG2 Meeting #122	S2-174431

26 - 30 June 2017, San Jose del Cabo, Mexico	(revision of S2-17xxxx)

Source:	LG Electronics

Title:	TS 23.502: Clarification on Service Request procedures

Document for:	Discussion / Approval

Agenda Item:	6.5.7

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposed to clarify and clean-up the service request procedures in the current specification.

1.	Discussion

In the current Service Request procedures, some clarifications and clean-up is required.

1) Use of Service Accept as a notification of service request success

5G system introduces to use Service Accept message for notifying the UE the successful completion of the Service Request procedure. However, it is not clear in which condition this message is used.

In the older systems, various ways have been used for this purpose in the service request procedure. The condition can be divided into two major categories: CM state and service type. In Idle mode, UE can send service request message for either signalling, data, or paging response while UE send service request message for user data in connected mode. In each condition, the network indicates the result of service request message in various way. If is not successful, the network sends the Service Reject message with an appropriate cause value. Brief summary of notifying successful completion of service request message is shown below:

|  | UMTS (23.060) | EPS (23.401) |
| --- | --- | --- |
| Idle, Signalling | Receiving RRC SMC command | N/A For CPSR, Service Accept |
| Idle, Data | Receiving RRC SMC command | DRB setup (lower layer indication) |
| Idle, Paging response | Receiving RRC SMC command | DRB setup (lower layer indication) |
| Connected, Data | Receiving NAS Service Accept | N/A |

Even though Service Accept is not a new idea, the use of it has been limited to specific condition. In the most case, the UE can realize the successful completion of the service request procedure as per some indication from the lower layer (e.g. RRC), and additional notification in upper layer is redundant and not necessary.

Another role of Service Accept is context synchronization. In UMTS, since the activation of PDP context is done selectively, the UE and the network may need to synchronize their context status during service request procedure. For this purpose, Service Accept message may be required independent to the success of SR procedure.

Still, even though the Service Accept is transferred to the UE, AS layer may lead to failure case (e.g. not enough resource for radio bearer). This means that the reception of the Service Accept does not guarantee the successful completion of radio bearer setup. So the following NOTEs are added:

NOTE:	The reception of the Service Accept message does not imply the successful activation of the User Plane radio resources.

Whether the other methods for notifying the UE the success of the SR procedure, e.g. lower layer indication for RRC SMC or DRB setup, is FFS, and this aspect shall be studied and analysed in stage 3, especially in CT1 and RAN2. So we added the following Editor’s note:

Editor's note:	How the UE is notified for the successful completion of the service request procedure is FFS and will be further studied in stage 3.

2) Clarification of network initiated case

In case of Network triggered service request procedure in CM-CONNECTED state, the AMF does not need to send Service Accept message since the UE has never sent the NAS Service Request message.

3) minor clarifications

In 4.2.3.2,

“For any Service Request, the AMF may respond with a Service Response message to synchronize PDU session status between UE and network. The AMF may also respond with Service Reject message to UE, if the Service Request cannot be accepted by network.”

There’s no “Service Response” message defined, but there are two possible NAS response to the Service Request message, which are Service Accept and Service Reject. So the sentence shall be generalized as “AMF may respond with a response message to …”

In 4.2.3.3, step 1,

The UE sends NAS message Service Request towards the AMF encapsulated in an RRC message to the RAN. The MM NAS Service Request message shall be encrypted and integrity protected. The PDU session ID(s) in NAS message Service Request message indicates the PDU session that the UE selects to activate.

According to S3-171491, agreed in SA3#87 meeting, SA3 agreed on the following:

Confidentiality protection of the RRC-signalling, and NAS-signalling is optional to use. Confidentiality protection should be used whenever regulations permit. (…) Integrity protection of the RRC-signalling, and NAS-signalling is mandatory to use, except in the following cases:

So ciphering of NAS signalling is optional to use, even though it is highly recommended, while the integrity protection is mandatory to use. This is the same requirements for the EPC. So the texts above shall be modified.

In the introduction of clause 4.2.3.2,

Editor's note:	The state names (e.g. CM-IDLE, CM-CONNECTED) of the UE need to be aligned when decided

Since the state name of the connection management is already agreed as CM-DILE and CM-CONNECTED, so this EN is not needed.

4) Editorial Fixes

In this contribution, several editorial errors has been fixed, including:

- Wrong formatted Editor’s notes

- Reference for other specifications

- Reference to the other procedures

2.	Proposal

According to the discussion above, we would like to propose to add the following texts in TS 23.502

* * * * Start of 1st Change * * * *

4.2.3	Service Request procedures

4.2.3.1	General

4.2.3.2	UE triggered Service Request in CM-IDLE state

Editor's note:	Procedure includes aspects required to support network slicing.

The Service Request procedure is used by a 5G UE in CM-IDLE state to request the establishment of a secure connection to an AMF. The UE in CM-IDLE state initiates the Service Request procedure in order to send uplink signalling messages, user data, or response to a network paging request. After receiving the Service Request message, the AMF may perform authentication, and the AMF shall perform the security procedure. After the establishment of a secure signalling connection to an AMF, the UE or network may send signalling messages, e.g. PDU session establishment from UE to the network, or the SMF, via the AMF, may start the User Plane resource establishment for the PDU sessions requested by network and/or indicated in the Service Request message.

For any Service Request, the AMF may respond with a response message to synchronize PDU session status between UE and network. The AMF may also respond with Service Reject message to UE, if the Service Request cannot be accepted by network.

For Service Request due to user data, network may take further actions if User Plane resource establishment is not successful.

NOTE 1:	The procedure in this clause 4.2.3.2 is not applicable for an access network (once the UE is registered in the network) in which the UE is always considered as in CM-CONNECTED state and in which the User Plane resource is always considered established for an active PDU session.

Editor's note:	The names of N2 message and N11 message are FFS.

Figure 4.2.3.2-1: UE triggered Service Request procedure

1.	UE to (R)AN: MM NAS Service Request (PDU session ID(s), security parameters, PDU session status).

The UE sends NAS message Service Request towards the AMF encapsulated in an RRC message to the RAN. The RRC message(s) that can be used to carry the 5G Temporary ID and this NAS message are described in RAN specifications.

If the Service Request is triggered for user data, the UE includes the PDU session ID(s) in NAS Service Request message to indicate the PDU session(s) that the UE needs to use. If the Service Request is triggered for signalling only, the UE doesn't include any PDU session ID. When this procedure is triggered for paging response, if the UE needs to use some PDU session(s), the UE includes the PDU session ID(s) in MM NAS Service Request message to indicate the PDU session(s) that the UE needs to use. Otherwise the UE will not include any PDU session ID.

The PDU session status indicates the PDU sessions available in the UE.

Editor's note:	It is FFS whether there is need to indicate the user data, signalling or paging response.

2.	(R)AN to AMF: N2 Message(MM NAS Service Request, 5G Temporary ID, Location information, RAT type, RRC establishment cause). Details of this step are described in RAN specifications. If the AMF can't handle the Service Request it will reject it.

5G Temporary ID is obtained in RRC procedure. RAN selects the AMF according to Temporary ID. The Location Information and RAT type relates to the cell in which the UE is camping.

Based on the PDU session status, the AMF may initiate PDU session release procedure if the PDU session is not available in the UE.

Editor's note:	The name for the global cell-id e.g. whether ECGI is used or some other term is FFS and will need to be determined also in collaboration with RAN WG.

Editor's note:	Procedures for the monitoring events for "Availability after DDN failure" are FFS.

Editor's note:	Procedures and parameters for CSG paging are FFS depending on whether CSG will be supported.

3.	If the Service Request was not sent integrity protected or integrity protection is indicated as failed, the AMF shall initiate NAS authentication/security procedure as defined in clause 4.6.

If the UE triggered the Service Request to establish a signalling connection only, after the security exchange the UE and the network can send signalling and steps 4 to 8 and 11 to 17 are skipped.

4.	[Conditional] AMF to SMF: N11 Message (PDU session ID(s), Cause(s), UE location information).

The N11 message is sent in one or more of the following scenarios:

-	If the MM NAS Service Request message includes PDU session ID(s), or this procedure is triggered by SMF but PDU session IDs from UE correlates to other SMFs than the one triggering the procedure, the AMF sends N11 message to SMF(s) associated with the PDU session ID(s) with Cause set to indicate "establishment of user plane resources" for a PDU session.

-	If the UE was in MICO mode and the AMF had notified an SMF of the UE being unreachable and that SMF needs not send DL data notifications to the AMF, the AMF informs the SMF that the UE is reachable.

NOTE 2: the AMF will also notify any other NF that subscribed to UE reachability that the UE is reachable.

5.	Based on the new location info, the SMF checks the UPF Selection Criteria according to clause 6.3.3 of TS 23.501 [2], and determines to perform one of the following:

-	continue using the current UPF(s);

-	selects a new intermediate UPF, if the UE has moved out of the service area of UPF that is connecting to RAN, while maintaining the UPF(s) acting as PDU Session Anchor; or

-	trigger re-establishment of the PDU Session to perform relocation of the UPF acting as PDU Session anchor.

Editor's note:	The above text needs to be confirmed and is a result from implementing clashing P-CRs.

6a.	[Conditional] SMF to new UPF: N4 Session Establishment Request

If the SMF selects a new UPF to act as intermediate UPF for the PDU session, an N4 Session Establishment Request message is sent to the new UPF, providing Packet detection, enforcement and reporting rules to be installed on the intermediate UPF. The PDU session anchor info for this PDU Session is also provided to the intermediate UPF.

6b.	new UPF (intermediate) to SMF: N4 Session Establishment Response

The new intermediate UPF sends an N4 Session Establishment Response message to the SMF. In case the UPF allocates CN tunnel info, it provides CN DL tunnel info and UL tunnel info (i.e. CN N3 tunnel info) to the SMF. The SMF starts a timer, to be used in step 17a to release the resource in old intermediate UPF if there is one.

7a.	[Conditional] SMF to UPF (PSA): N4 Session Modification Request

The SMF sends N4 Session Modification Request message to PDU session anchor UPF, UPF (PSA), providing the DL tunnel information from new intermediate UPF.

7b.	UPF (PSA) to SMF: N4 Session Modification Response

The UPF (PSA) sends N4 Session Modification Response message to SMF.

8.	[Conditional] SMF to AMF: N11 Message (N1 SM information (PDU session ID, PDU Session re-establishment indication), N2 SM information (PDU Session ID, QoS profile, CN N3 Tunnel Info, S-NSSAI)) to the AMF.

Upon reception of the N11 Message in step 4 with cause including "establishment of user plane resources", the SMF determines whether UPF reallocation is performed, based on the UE location information, UPF service area and operator policies:

-	For PDU session that SMF determines to be served by the current PDU Session Anchor UPF in step 5, the SMF generates only N2 SM information and sends N11 Message to the AMF to establish the User Plane(s). The N2 SM information contains information that the AMF shall provide to the RAN.

-	For PDU sessions that SMF determines that UPF reallocation for PDU Session Anchor UPF is needed in step 5, SMF may send N11 Message containing only N1 SM information to UE via AMF. The N1 SM information includes the corresponding PDU session ID and PDU Session re-establishment indication, which is the same as step 2 described in clause 4.3.5.1.1.

Upon reception of the N11 Message in step 4 with an indication that the UE is reachable, if the SMF has pending DL data the SMF sends N11 Message to the AMF to establish the User Plane(s) for the PDU sessions, otherwise the SMF resumes sending DL data notifications to the AMF in case of DL data.

Editor's note:	The above text needs to be confirmed and is a result from implementing clashing P-CRs.

9.	AMF to (R)AN: N2 Request (N2 SM information received from SMF, security context, AMF Signalling Connection ID, Handover Restriction List, MM NAS Service Accept).

RAN stores the Security Context, AMF Signalling Connection Id, QoS Information for the QoS Flows of the PDU Sessions that are activated and N3 Tunnel IDs in the UE RAN context. The step is described in detail in RAN specifications. Handover Restriction List is described in TS 23.501 [2] clause 5.3.4.1 "Mobility Restrictions".

MM NAS Service Accept includes PDU session status in AMF. If N1 SM information is received from step 8, the Service Accept message also includes N1 SM information.

AMF shall include at least one N2 SM information from SMF if the procedure is triggered for PDU session User Plane setup. AMF may send additional N2 SM information from SMFs in separate N2 message(s) (e.g. N2 tunnel setup request), if there is any. Alternatively, if multiple SMFs are involved, the AMF may send one N2 Request message to (R)AN after all the N11 messages from SMFs are received. In such case, the N2 Request message includes the N2 SM information received in each of the N11 messages and information to enable AMF to associate responses to relevant SMF.

10.	(R)AN to UE: The RAN performs RRC Connection Reconfiguration with the UE depending on the QoS Information for all the QoS Flows of the PDU Sessions activated and Data Radio Bearers. The User Plane security is established at this step, which is described in detail in RAN specifications.

The RAN forwards the MM NAS Service Accept to the UE. The UE locally deletes context of PDU sessions that are not available in 5G CN.

If the N1 SM information is present in the Service Accept and indicates that the any PDU session needs to be re-established, the UE initiates PDU session re-establishment after Service Request procedure is completed. That is, for SSC mode 2, step 3 and step 4 is defined in clause 4.3.5.1.1 is performed. For SSC mode 3, step 3 and 4 defined in clause 4.3.5.2 is performed.

NOTE 2:	The reception of the Service Accept message does not imply the successful activation of the User Plane radio resources.

Editor's note:	How the UE is notified for the successful completion of the service request procedure is FFS and will be further studied in stage 3.

11.	After the User Plane radio resources are setup, the uplink data from the UE can now be forwarded to RAN. The 5G RAN sends the uplink data to the UPF address and Tunnel ID provided in the step 4.

12.	[Conditional] (R)AN to AMF: N2 Request Ack (N2 SM information (RAN Tunnel info, List of accepted QoS Flows for the PDU Sessions activated, List of rejected QoS Flows for the PDU Sessions activated). This step is described in detail in RAN specifications.

The message may include N2 SM information(s), e.g. RAN tunnel information. RAN may respond N2 SM information with separate N2 message (e.g. N2 tunnel setup response) if AMF sends separate N2 message in step 5.

If multiple N2 SM information are included in the N2 Request message in step 5, the N2 Request Ack includes multiple N2 SM information and information to enable the AMF to associate the responses to relevant SMF.

Editor's note:	Further detailing of the messages in N2 is FFS and is to be coordinated with RAN3.

13.	[Conditional] AMF to SMF: N11 Message (N2 SM information (RAN Tunnel info), RAT Type) per accepted PDU Session to the SMF.

If the AMF received N2 SM information (one or multiple) in step 8, then the AMF shall forward the N2 SM information to the relevant SMF. If the UE Time Zone has changed compared to the last reported UE Time Zone then the AMF shall include the UE Time Zone IE in this message.

Editor's note:	It is FFS how to handle the non-accepted QoS Flows.

Editor's note:	Procedure to pause charging by SMF is FFS.

14.	[Optional] SMF to PCF: If dynamic PCC is deployed, SMF may initiate IP-CAN Session Modification and provides new location information to the PCF.

See clause x.y.z for details.

15a.	[Conditional] SMF to new intermediate UPF: N4 Session Modification Request (RAN tunnel info).

If a User Plane is to be setup or modified the SMF initiates a N4 Session Modification procedure and provides RAN Tunnel Info. See clause x.y.z for more details.

15b.	[Conditional] UPF to SMF: N4 Session Update Response.

See clause x.y.z for more details.

16.	[Conditional] SMF to AMF: N11 Message ACK.

Editor's note:	It is FFS whether this message is needed.

17a.	[Conditional] SMF to old intermediate UPF: N4 Session Release Request

If there is an old intermediate UPF, the SMF initiates resource release, after timer in step 6b expires, by sending an N4 Session Release Request (Release Cause) to old intermediate UPF.

17b.	Old intermediate UPF to SMF: N4 Session Release Response

The old UPF acknowledges with an N4 Session Release Response message to confirm the release of resources.

4.2.3.3	UE triggered Service Request in CM-CONNECTED state

Editor's note:	Procedure includes aspects required to support network slicing.

The Service Request procedure is used by a 5G UE in CM-CONNECTED to request establish User Plane resources for the PDU sessions.

Editor's note:	It is FFS how to handle if the not all the PDU session requested by the UE can be accepted by network.

The network may take further actions if User Plane resource establishment is not successful.

NOTE 1:	The procedure in this clause 4.2.3.3 is not applicable for an access network (once the UE is registered in the network) in which the UE is always considered as in CM-CONNECTED state and in which the User Plane resource is always considered established for an active PDU session.

Editor's note:	The names of N2 message and N11 message are FFS.

Figure 4.2.3.3-1: UE triggered Service Request procedure in CM-CONNECTED state

1.	UE to (R)AN: MM NAS Service Request (PDU session ID(s)).

The UE sends NAS message Service Request towards the AMF encapsulated in an RRC message to the RAN. The MM NAS Service Request message shall be integrity protected. The PDU session ID(s) in NAS message Service Request message indicates the PDU session that the UE selects to activate.

2.	(R)AN to AMF: N2 Message(MM NAS Service Request). Details of this step are described in RAN specifications.

The (R)AN forwards the MM NAS Service Request message to the AMF based on the existing N2 connection. If the Service Request cannot be handled by the AMF, the AMF reject it with an appropriate cause.

3.	[Conditional] AMF to SMF: N11 Message (PDU session ID(s)).

The AMF sends N11 message to SMF(s) associated with the PDU session ID(s).

4.	Based on the new location info, the SMF checks the UPF Selection Criteria according to clause 6.3.3 of TS 23.501 [2]. If UE has moved out of the service area of UPF that connecting to RAN, SMF selects a new intermediate UPF.

5a.	[Conditional] SMF to new intermediate UPF: N4 Session Establishment Request

If the SMF selects a new intermediate UPF for the PDU session, an N4 Session Establishment Request message is sent to the new intermediate UPF, providing Packet detection, enforcement and reporting rules to be installed on the T-UPF. The PDU session anchor info for this PDU Session is also provided to the T-UPF.

5b.	new UPF to SMF: N4 Session Establishment Response

The new UPF sends an N4 Session Establishment Response message to the SMF. If the UPF allocates CN tunnel info, the UPF provides CN DL tunnel info and UL tunnel info (i.e. CN N3 tunnel info) to the SMF. The SMF starts a timer, to be used in step 17a to release the resource in old UPF if there is one.

6a.	[Conditional] SMF to UPF (PSA): N4 Session Modification Request

The SMF sends N4 Session Modification Request message to PDU session anchor UPF, UPF (PSA), providing the DL tunnel information for the new intermediate UPF.

6b.	UPF (PSA) to SMF: N4 Session Modification Response

The UPF (PSA) sends N4 Session Modification Response message to SMF.

7.	[Conditional]SMF to AMF: N11 Message (N2 SM information (PDU Session ID, QoS profile, CN N3 Tunnel Info, S-NSSAI)) to the AMF.

The SMF generates N2 SM information and sends N11 Message to the AMF to establish the User Plane(s) for the PDU sessions. The N2 SM information contains information that the AMF shall provide to the RAN.

8.	[Conditional]AMF to (R)AN: N2 Request (N2 SM information (QoS profile, CN N3 Tunnel Info) received from SMF, MM NAS Service Accept).

If there are multiple PDU Sessions that involves multiple SMFs, AMF does not need wait for responses from all SMFs in step 3. In case of the Network initiated Service Request procedure, the AMF does not send MM NAS Service Accept message to the UE.

9.	(R)AN to UE: The RAN performs RRC Connection Reconfiguration with the UE depending on the QoS Information for all the QoS Flows of the PDU Sessions and Data Radio Bearers activated.

The RAN forwards the MM NAS Service Accept to the UE.

NOTE 2:	The reception of the Service Accept message does not imply the successful activation of the User Plane radio resources.

Editor's note:	How the UE is notified for the successful completion of the service request procedure is FFS and will be further studied in stage 3.

10.	After the User Plane radio resources for the selected PDU session are setup, the uplink data from the UE can now be forwarded to RAN. The 5G RAN sends the uplink data to the UPF address and Tunnel ID provided in the step 8.

11.	[Conditional] (R)AN to AMF: N2 Request Ack (N2 SM information (RAN Tunnel info, List of accepted QoS Flows for the PDU Sessions activated, List of rejected QoS Flows for the PDU Sessions activated). This step is described in detail in RAN specifications.

The message may include N2 SM information(s), e.g. RAN tunnel information. RAN may respond N2 SM information with separate N2 message (e.g. N2 tunnel setup response).

Editor's note:	Further detailing of the messages in N2 is FFS and is to be coordinated with RAN3.

12.	[Conditional] AMF to SMF: N11 Message (N2 SM information (RAN Tunnel info, List of accepted QoS Flows, List of rejected QoS Flows)) per accepted PDU Session to the SMF.

Editor's note:	It is FFS whether the non accepted QoS Flows exists and how to handle the non-accepted QoS Flows.

Editor's note:	Procedure to pause charging by SMF is FFS.

12.	[Optional] SMF to PCF: If dynamic PCC is deployed, SMF may initiate IP-CAN Session Modification and provides new location information to the PCF.

See clause x.y.z for details.

14a.	[Conditional] SMF to UPF: N4 Session Update Request (RAN tunnel info and List of accepted QoS Flows).

If a User Plane is to be setup or modified the SMF initiates a N4 Session Modification procedure and provides RAN Tunnel Info. See clause 4.3.3.2 for more details.

14b.	[Conditional] UPF to SMF: N4 Session Update Response.

See clause x.y.z for more details.

15.	[Conditional] SMF to AMF: N11 Message ACK.

Editor's note:	It is FFS whether this message is needed.

16a.	[Conditional] SMF to old UPF (intermediate): N4 Session Release Request

If there is an old UPF (intermediate), the SMF initiates resource release, after timer in step 6b expires, by sending an N4 Session Release Request (Release Cause) to old UPF.

16b.	Old UPF (intermediate) to SMF: N4 Session Release Response

The old UPF (intermediate) acknowledges with an N4 Session Release Response message to confirm the release of resources.

4.2.3.4	Network triggered Service Request

Editor's note:	Procedure includes aspects required to support network slicing, and should include notification from UPF upon MT UP data arrival.

This procedure is used when the network needs to signal (e.g. N1 signalling to UE, Mobile-terminated SMS, User Plane resource establishment for PDU session(s) to deliver mobile terminating user data) with a UE. If the UE is in CM-IDLE state or CM-CONNECTED state, the network initiates a network triggered Service Request procedure. If the UE is in CM-IDLE state, and asynchronous type communication is not activated, the network sends a Paging Request to (R)AN/UE. The Paging Request triggers the Service Request procedure in the UE. If asynchronous type communication is activated, the network stores the received message and forward the message to the (R)AN and/or the UE (i.e. synchronizes the context with the (R)AN and/or the UE) when the UE enters CM-CONNECTED state.

Editor's note:	The name of N11 message between AMF and SMF is FFS.

Figure 4.2.3.4-1: Network Triggered Service Request

1.	When UPF receives downlink data of a PDU session and there is no (R)AN tunnel information stored in UPF for the PDU session, the UPF buffers the downlink data, unless the UPF has previously been notified by the SMF to not buffer the downlink data.

2a.	UPF to SMF: Data Notification (PDU session ID, Priority).

-	On arrival of the first downlink data packet, the UPF shall send Data Notification message to the SMF, if the SMF has not previously notified the UPF to not send the Data Notification to the SMF (in which case the next steps are skipped).

-	If the UPF receives additional downlink data packets for a QoS Flow in the same PDU Session with the same or a lower priority than used in any previous Data Notification for this PDU session, the UPF buffers these downlink data packets without sending a new Data Notification.

Editor's note:	If the UPF receives additional downlink data packets for a QoS Flow in the same PDU Session with a higher priority than used in any previous Data Notification for this PDU Session, the UPF shall send a Data Notification message to the SMF indicating the higher priority.

-	If the Paging Policy Differentiation feature (as specified in TS 23.501 [2] clause 5.4.3) is supported by the UPF and if it is activated by the SMF for this N4 session, the UPF shall also include the DSCP in TOS (IPv4) / TC (IPv6) value from the IP header of the downlink data packet.

-	If the SMF, while waiting for the User Plane to be established in UPF, receives N11 message notifying the new AMF serving the UE from the new AMF, the SMF re-sends the Data Notification message only to the new AMF.

2b.	SMF to UPF: Data Notification Ack.

Editor's note:	It is FFS whether this message is needed or not.

3a.	SMF to AMF: N11 message (SUPI, PDU session ID, N2 SM information (QoS profile, CN N3 Tunnel Info), Priority).

Unless the SMF was previously notified that the UE is unreachable, upon reception of a Data Notification message, the SMF determines the AMF and sends an N11 message (SUPI, PDU session ID, N2 SM information (PDU Session ID, QoS profile, CN N3 Tunnel Info, S-NSSAI), Priority, Paging Policy Indication) to the AMF including the Priority and PDU Session ID received in the Data Notification message as part of step 2a.

If the SMF, while waiting for the User Plane Connection to be activated, receives any additional Data Notification message for the same PDU session but with higher priority than indicated in any previous Data Notification for this PDU session, the SMF sends a new N11 message indicating the higher priority and PDU Session ID to the AMF.

If the SMF, while waiting for the User Plane to be activated, receives an N11 message response from an AMF other than the one to which the SMF sent an N11 message, the SMF sends the N11 message to this AMF.

When supporting Paging Policy Differentiation, the SMF indicates in the N11 message the Paging Policy Indication related to the downlink data that triggered the Data Notification message, as described in TS 23.501 [2] clause 5.4.3.

NOTE 1:	AMF may receive request message(s) from other network functions which leads to signalling towards UE/(R)AN, e.g. network initiated deregistration, SMF initiated PDU session modification. If the UE is in CM-CONNECTED state and the AMF only delivers N1 message towards UE, the flow continues in step 7 below.

Editor's note:	Procedures for the extended buffering time are FFS.

Editor's note:	Procedures for the monitoring events for "Availability after DDN failure" are FFS.

Editor's note:	It is FFS whether this message carry QoS profile and N3 tunnel information of the PDU session.

3b.	[conditional] The AMF responds to the SMF.

If the UE is in CM-IDLE state, and the AMF determines that the UE is not reachable for paging, the AMF shall either send an N11 message to the SMF, or other network functions from which AMF received the request message in step 3a, indicating the UE is not reachable, or the AMF performs asynchronous type communication and store the N11 message. If asynchronous type communication is invoked and the AMF stored an N11 message, the AMF initiates communication with the UE and (R)AN when the UE is reachable e.g. when the UE enters CM-CONNECTED state.

If the UE is in MICO mode, the AMF rejects the request from the SMF and may notify the SMF that the UE is unreachable (in case the SMF had not subscribed to the UE reachability) with an indication that the SMF needs not send DL data notifications to the AMF. The AMF stores an indication that the SMF has been informed that the UE is unreachable.

Editor's note:	It is FFS whether the SMF should be able to include an asynchronous indication to notify AMF whether asynchronous type communication can be used for the N11 message. The relation between asynchronous type communication and the reachability procedures in clause 4.2.5 is FFS.

If the Registration procedure with AMF change is in progress when the old AMF receives a N11 message, the old AMF may reject N11 message with an indication that the N11 message has been temporarily rejected.

Editor's note:	It is FFS whether further signalling optimization is to be considered in case of SMFs associated with the AMF for a given UE.

3c.	[Conditional] SMF responds to the UPF

SMF may notify the UPF about the User Plane setup failure.

If the SMF receives an indication from the AMF that the UE is unreachable the SMF may, based on network policies, either:

-	indicate to the UPF to stop sending Data Notifications and/or to stop buffering DL data or apply extended buffering, or

-	refrains from sending further N11 messages for DL data to the AMF while the UE is in MICO mode.

Upon receiving the information that the N11 message requested from an SMF has been temporarily rejected, and receiving the Downlink Data Notification from UPF, the SMF may request the UPF to apply extended buffering.

4.	[Conditional] AMF sends Paging message to (R)AN node.

If the UE is in CM-CONNECTED state, the AMF performs step 8 to 16 in UE triggered Service Request procedure (see clause 4.2.3.3) to activate the User Plane Connection for this PDU session (i.e. establish the radio resources and N3 tunnel) without sending a Paging message to the (R)AN node and the UE. In step 8 of clause 4.2.3.3, the AMF does not send the Service Accept message to the UE. The rest of this procedure is omitted.

If the UE is in RM-REGISTERED state and CM-IDLE and reachable, the AMF sends a Paging message (NAS ID for paging, Registration Area list, Paging DRX length, Paging Priority indication) to (R)AN node(s) belonging to the Registration Area(s) in which the UE is registered, see TS 38.331 [xx].

When supporting Paging Policy Differentiation, the AMF may include Paging Policy Indication in Paging Request message as described in TS 23.501 [2] clause 5.4.3.

Paging strategies may be configured in the AMF for different combinations of DNN, Paging Policy Indication, PDU Session IDs from SMF when available (see clause 4.9) and other PDU Session context information identified by PDU Session ID received in N11 message.

Paging strategies may include:

-	paging retransmission scheme (e.g. how frequently the paging is repeated or with what time interval);

-	determining whether to send the Paging message to the (R)AN nodes during certain AMF high load conditions;

-	whether to apply sub-area based paging (e.g. first page in the last known cell-id or TA and retransmission in all registered TAs).

The AMF and the (R)AN may support further paging optimisations in order to reduce the signalling load and the network resources used to successfully page a UE by one or several of the following means:

-	by the AMF implementing specific paging strategies (e.g. the N2 Paging message is sent to the (R)AN nodes that served the UE last);

-	by the AMF considering Information On Recommended Cells And RAN nodes provided by the (R)AN at transition to CM-IDLE state. The AMF takes the (R)AN nodes related part of this information into account to determine the (R)AN nodes to be paged, and provides the information on recommended cells within the N2 Paging message to each of these (R)AN nodes;

-	by the (R)AN considering the Paging Attempt Count Information provided by the AMF at paging.

If the UE Radio Capability for Paging Information is available in the AMF, the AMF adds the UE Radio Capability for Paging Information in the N2 Paging message to the (R)AN nodes.

If the Information On Recommended Cells And (R)AN nodes For Paging is available in the AMF, the AMF shall take that information into account to determine the (R)AN nodes for paging and, when paging a (R)AN node, the AMF may transparently convey the information on recommended cells to the (R)AN node.

The AMF may include in the N2 Paging message(s) the paging attempt count information. The paging attempt count information shall be the same for all (R)AN nodes selected by the AMF for paging.

Editor's note:	Procedures for enhanced coverage are FFS

Editor's note:	Procedures and parameters for extended DRX are FFS.

Editor's note:	Procedures and parameters for CSG paging are FFS depending on whether CSG will be supported.

Editor's note:	The need to send Paging Cause to (R)AN and UE is FFS.

5.	[Conditional]The (R)AN node pages the UE.

If (R)AN nodes receive paging messages from the AMF, the UE is paged by the (R)AN node, see TS 38.331 [xx].

6.	[Conditional] AMF to SMF: N11 message ACK.

The AMF supervises the paging procedure with a timer. If the AMF receives no response from the UE to the Paging Request message, the AMF may apply further paging according to any applicable paging strategy described in step 3.

If the AMF receives no response from the UE, the AMF considers the UE as unreachable and the SM N2 message cannot be routed to the (R)AN, so the AMF shall return an "N11 message Reject" with an appropriate "failure cause", e.g. UE unreachability, to SMF or other network functions to indicate the failure of "message routing service", unless the AMF is aware of an ongoing MM procedure that prevents the UE from responding, i.e. the AMF receives an N14 Context Request message indicating that the UE performs Registration procedure with another AMF.

Editor's note:	Inter-system procedures e.g. EPC to 5G CN are FFS.

When an "N11 message Reject" is received, SMF informs the UPF.

Editor's note:	Procedure to pause of charging by SMF is FFS.

7.	When UE is in the CM-IDLE state, upon reception of paging request, the UE initiates the UE triggered Service Request procedure (clause 4.2.3.2). In step 4 of clause 4.2.3.2, the AMF sends N11 message to SMF(s) associated with the PDU session identified by PDU session ID(s) in MM NAS Service Request message if there is any, but not to SMF(s) from which it receives the N11 message in step 3a.

8.	The UPF transmits the buffered downlink data toward UE via (R)AN node which performed the Service Request procedure.

Network sends downlink signalling if the procedure is triggered due to request from other network entities described in step 3a.

* * * * End of Changes * * * *