SA WG2 Meeting #122	S2-174500

26 - 30 June 2017, Los Cabos, Mexico	(revision of S2-zzz)

Source:	CATT

Title:	Key Issue: RAT selection for a V2X application

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	FS_eV2XARC / Rel-15

Abstract of the contribution: Introduce Key Issue based requirement [R.5.1-015] in TS22.186 section which came from TS22.886 section 5.15 “Use case on multi-RAT”.

1. Discussion

The following requirements came from the use case named “Use case on multi-RAT”, which applies multi-RAT UE supporting V2X application.

---- Paste from 22.186 ----

[R.5.1-015] The 3GPP system shall be able to support the operators to select which 3GPP RAT to use for a V2X application.

Editor’s note:	The meaning of V2X application needs further clarification

---- Paste from 22.186 ----

Regarding multi RAT enabled UE supporting V2X application, selecting the suitable RAT for each application is fundamental to meet specific V2X requirement, e.g. for delay stringent application, the LTE with shorten TTI should be selected rather than normal LTE.

In ordering to meet this requirements, at least following aspects should be studied (please see actual changes). Given that normally RAT selection is normally a system level issue, RAN working group should be involved during the study of this key issue when necessary.

2. Proposal

It is proposed to include the following Key Issue into TR 23.786.

* * * * First Change * * *.*

5.X	Key Issue #X: RAT selection for a V2X application

5.X.1	General description

The V2X UE may support multiple radio access technologies (RATs), including the LTE and the 5G NR. For such UE supporting V2X application, the most suitable RAT(s) for each application should be selected based on varied criterion. For example, for the V2X application requires low latency, RAT which met required latency should be select.

Editor’s note:	The meaning of V2X application needs further clarification

To support that enable the operators to select which 3GPP RAT to use for a V2X application, the following aspects need to be studied at least:

-	What are the parameters should be considered as input to RAT selection for each V2X application? E.g. QoS parameters, current radio coverage/signals, operator policy, user preference, and etc

-	When and how the RAT selection is performed? Just select the RAT before sending/receiving the V2X message, or by static configuration which RAT each V2X application should UE in UE supporting V2X application?

-	How can Core Network (possible together with RAN) help the UE supporting the V2X application to do the RAT selection? What is the overall of such procedure ? Which core network entity e.g. PCF, are involved in the overall procedure, and how?

When studying the above aspects, the following needs to be considered:

-	When the UE supporting V2X application is non-roaming and when the UE is roaming;

Editor’s note:	The RAN WG should be involved during the study of this key issue when necessary.

* * * * End of Changes * * *.*