SA WG2 Meeting #122	S2-174541

26 - 30 June 2017, San Jose Del Cabo, Mexico	(revision of S2-17xxxx)

Source:	China Mobile, Vodafone, CATR, ZTE, Deutsche Telekom, Verizon, AT&T

Title:	Introduce Mobile Termination service to AMF

Document for:	Discussion and Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution proposes to update AMF services through introducing Mobile Termination service in 23.502.

- Introduction

In TS 23.502 section 5.2.2, AMF has two separate services Namf_Communication and Namf_EventExposure service. In view of AMF functionalities listed in TS 23.501 Section 6.2.1, AMF provides many types of functionalities. Among them some of functionality, e.g., Reachability management is crucial to enable UE reachable through paging.

Why Mobile Termination (MT) services require a separate services:

- The Paging consumes quite a lot network resources in current network, say more than 30% or even much higher (beyond 50%) if TA is not well configured in 4G.

- On the other hand, only part of UE requires paging service. For example, MICO UE does not require paging and so is for fixed-wireless UE.

Why MT services can be designed as a separate services:

Mobile Termination action is related with other AMF service on the UE context. Namely, AMF paging need to have knowledge of UE location (Registration area), UE capability (MICO or not), UE mobility pattern (Restriction area). While, paging is only for context READ and any modification of UE context could be done through other AMF services, using service internal interaction which is not standardized in this release.

Additionally, having a separate service will help the scalability requirements that may be needed while facilitating the interactions with non-3GPP accesses.

- Conclusion and Proposal

It is proposed to update 23.502 cause 5 the AMF service as below:

* * * First Change * * *

5.2.2	AMF Services

5.2.2.1	General

The following table illustrates the AMF Services.

Table 5.2.2.1-1: List of AMF Services

| Service Name | Service Operations | Known Consumer(s) | Operation Semantic | Reference |
| --- | --- | --- | --- | --- |
| Namf_Communication | UEContextTransfer | Peer AMF | Request/ Response | Step 4 and 5 of clause 4.2.2.2.2. |
|  | UERegistrationCompleteNotify | Peer AMF | Subscribe / Notify | Step 10 of clause 4.2.2.2.2. |
|  | N1MessageNotify | SMF, SMSF, PCF, NEF | Subscribe / Notify | Step 3 clause 4.3.2.2.1. |
|  | N1MessageSubscribe | SMF, SMSF, PCF, NEF | Subscribe / Notify | Step 5a of clause 4.13.3.6. |
|  | N1MessageUnSubscribe | SMF, SMSF, PCF, NEF | Subscribe / Notify | Step 7 of clause 4.13.3.3. Step 6c of clause 4.13.3.6. |
|  | N1MessageTransfer | SMF, SMSF, PCF, NEF | Request/ Response | Step 10 of clause 4.3.2.2.1. Step 14 of lause 4.3.2.2.2. Step 5a of clause 4.13.3.6. |
|  | N2MessageTrigger | SMF | Request/ Response | Step 4b of clause 4.2.3.2. |
|  | N2InfoSubscribe | SMF | Subscribe / Notify | Step 10 of clause 4.3.2.2.1. Step 14 of clause 4.3.2.2.2. |
|  | N2InfoUnSubscribe | SMF | Subscribe / Notify |  |
|  | N2InfoNotify | SMF | Subscribe / Notify | Step 14 of clause 4.3.2.2.1. Step 18 of clause 4.3.2.2.2. Step 9 of clause 4.2.3.2. |
| Namf_EventExposure | Subscribe | NEF, SMF, PCF, UDM | Subscribe / Notify | Step 10 of clause 4.3.2.2.1. Step 14 of clause 4.3.2.2.2. |
|  | UnSubscribe | NEF, SMF, PCF, UDM | Subscribe / Notify |  |
|  | Notify | NEF, SMF, PCF, UDM | Subscribe / Notify | Step 4a of clause 4.2.3.2. Step 3a of clause 4.2.3.3. |
| Namf_MobileTermination | EnableUEReachability | NEF, SMF, PCF, UDM | Request/ Response | Step 3a, 3b of clause 4.2.3.4. |

Editor's note:	Whether additional NF services can be identified or be decomposed from existing NF services are FFS.

* * * Second Change * * *

5.2.2.x	Namf_MobileTermination service

5.2.2.x.1	General

Service description: It provides a NF the service that make sure UE is reachable to send MT signaling or data. The following are the key functionalities of this NF service

-	paging UE if UE is in IDLE state and respond other NF when receive service request.

-  response to the requester NF if UE is in CONNECTED state.

5.2.2.x.2	Namf_MobileTermination_ EnableReachability service operation

Service operation name: Namf_MobileTerminatio_ EnableReachability.

Description: The consumer NF uses this service operation to make UE reachable which may lead to page UE and make UE enter connected state if required.

Known NF Consumers: NEF, SMF, PCF, UDM

Inputs, Required: NF ID, UE ID.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

See step 3 of clause 4.2.3.4 for details on the usage of this service operation, and 4.2.2.3.3 for UDM initiated de-registration.

The AMF accepts the request and respond the consumer NF if UE is in CONNECTED state. If the UE is in IDLE state, the AMF will page UE and respond to the consumer NF when receive service request.

*************** end of change *********************