SA WG2 Meeting #122	S2-174571

26 - 30 June 2017, Cabo, Mexico	(revision of S2-17xxxx)

Source:	NTT DOCOMO

Title:	No-Nx Interworking Procedures

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Captures the No-Nx interworking procedures proposed in S2-174569 in 23.502.

# 1. Introduction

Network side

The following changes to capture no-Nx interworking are captured (source S2-174569):

1. Such networks also provide an indication that interworking without Nx interface is supported to UEs during initial Registration in 5GC.

2. When UE performs initial Registration in 5GC, the AMF, based on UE's indication of support of EPC NAS in UE network capability IE, does not include "initial attach" indicator to the HSS+UDM. This results in HSS not cancelling the registration of MME, if any.

3. When PDU session are created in 5GC, the PGW-C+SMF stores its information along with DDN in the HSS.

NOTE: This is already supported in 5GC (Step 19 of PDU Session establishment)

4. The HSS provides the information about dynamically allocated PGW-C+SMF and APN/DNN information to the target CN network during location update procedure.

NOTE: No specification changes are needed for this, as this is standard HSS+UDM behavior.

To provide IP address preservation to UEs operating in single-registration mode when the UE moves from EPC to 5GC, the network supports 2, 3, 5 and the following below:

5.	When UE performs mobility Registration in 5GC and provides a 5G-GUTI mapped 4G-GUTI, the AMF reject the procedure with a "Handover PDU Session Setup Support" indication to the UE.

UE side

- 	SR UE will perform mobility Registration providing 5G-GUTI mapped from 4G-GUTI, and the network with no-Nx mobility support will reject this. Captured the part of SR UE performing mobility Registration.

-	DR UE does an initial Registration which is already covered. DR UE acts as if it is never registered in 3GPP access. Nothing needs to be updated to capture this.

# 2.	Updates to 23.502

### **************  START CHANGES **********************

##### 4.2.2.2.2	General Registration

Figure 4.2.2.2.2-1: Registration procedure

1.	UE to (R)AN: AN message (AN parameters, Registration Request (Registration type, SUPI or 5G-GUTI, Security parameters, NSSAI, UE 5GCN Capability, PDU session status, PDU session(s) to be re-activated and MICO mode preference)).

Editor's note:	The required AN parameters is FFS.

In case of 5G-RAN, the AN parameters include e.g. SUPI or the 5G-GUTI, the Selected Network and NSSAI.

The Registration type indicates if the UE wants to perform an "initial registration" (i.e. the UE is in RM-DEREGISTERED state), a "mobility registration update " (i.e. the UE is in registered state and initiates a Registration procedure due to mobility) or a "periodic registration update" (i.e. the UE is in registered state and initiates a Registration procedure due to the periodic update timer expiry, see clause 4.2.2.2.1). The UE performing an initial registration (i.e., the UE is in RM-DEREGISTERED state) to a PLMN for which the UE does not already have a 5G-GUTI, the UE shall include its SUPI in the registration attempt. In other cases, the 5G-GUTI is included and it indicates the last serving AMF. If the UE is already registered via a non-3GPP access in a PLMN different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the 3GPP access, the UE shall not provide over the 3GPP access the 5G-GUTI allocated by the AMF during the registration procedure over the non-3GPP access. Also, if the UE is already registered via a 3GPP access in a PLMN (i.e. the registered PLMN), different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the non-3GPP access, the UE shall not provide over the non-3GPP access the 5G-GUTI allocated by the AMF during the registration procedure over the 3GPP access.

If the UE is already registered in EPC and operates in single-registration mode (see TS 23.501[x]), the UE shall set the Registration type to “mobility registration update” and the UE shall provide a UE Temporary ID mapped from the GUTI (see clause 5.17.2.2: 5G-GUTI mapped from 4G-GUTI). The UE may also provide an additional native 5G GUTI, if one is available at the UE.

The Security parameters are used for Authentication and integrity protection. NSSAI indicates the Network Slice Selection Assistance Information (as defined in clause 5.15 of TS 23.501 [2]. The PDU session status indicates the previously established PDU sessions in the UE. The "PDU session(s) to be re-activated" is included to indicate the PDU session(s) that the UE intends to activate.

Editor's note:	The content of the Security parameters is FFS.

Editor's note:	Changes required to support "stickiness" is FFS.

Editor's note:	It is FFS if HO attach indication is to be added.

Editor's note:	It is FFS how UE Radio capabilities are provided.

2.	If a SUPI is included or the 5G-GUTI does not indicate a valid AMF the (R)AN, based on (R)AT and NSSAI, if available, selects an AMF.

The (R)AN selects an AMF as described in TS 23.501 [2], clause X.

If the (R)AN cannot select an appropriate AMF, it forwards the Registration Request to an AMF which has been configured, in (R)AN, to perform AMF selection.

3.	(R)AN to new AMF: N2 message (N2 parameters, Registration Request (Registration type, Subscriber Permanent Identifier or 5G-GUTI, Security parameters, NSSAI and MICO mode preference)).

When 5G-RAN is used, the N2 parameters include the Location Information, Cell Identity and the RAT type related to the cell in which the UE is camping.

If the Registration type indicated by the UE is periodic registration update, then steps 4 to 17 may be omitted.

4.	[Conditional] new AMF to old AMF: Information Request (complete Registration Request).

If the UE's 5G-GUTI was included in the Registration Request and the serving AMF has changed since last registration, the new AMF may send Information Request to old AMF including the complete Registration Request IE to request the UE's SUPI and MM Context.

If the Registration type is "mobility registration update" and the UE provided a 5G-GUTI mapped from 4G-GUTI, and the AMF is configured to support EPC interworking without Nx procedure, the AMF rejects the Registration Request with "Handover PDU Session Setup Support" indicator to the UE. The UE subsequently performs Registration with type "initial registration" and moves PDU session to 5GC using the "Existing PDU Session Indication" (see clause 5.3.2.2.1 and 5.3.2.2.2).

5.	[Conditional] old AMF to new AMF: Information Response (SUPI, MM Context, SMF information).

Old AMF responds with Information Response to new AMF including the UE's SUPI and MM Context.

If old AMF holds information about active PDU Sessions, the old AMF includes SMF information including SMF identities and PDU session identities.

6.	[Conditional] new AMF to UE: Identity Request ().

If the SUPI is not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE.

7.	[Conditional] UE to new AMF: Identity Response ().

The UE responds with an Identity Response message including the SUPI.

8.	The AMF may decide to invoke an AUSF. In that case, the AMF, shall be based on SUPI, select an AUSF as described in TS 23.501 [2], clause X.

9a.	The AUSF shall initiate authentication of the UE.

The authentication is performed as described in clause X.

Editor's note:	It is FFS how UDM is selected in this step.

Editor's note:	It is FFS whether the AUSF or the AMF initiates the authentication.

If network slicing is used, the AMF decides if the Registration Request needs to be rerouted as described in clause 4.2.2.2.3, where the initial AMF refers to the AMF.

9b	The AMF shall initiate NAS security functions.

The NAS security is performed as described in clause X.

10.	[Conditional] new AMF to old AMF: Information Acknowledged ().

If the AMF has changed the new AMF acknowledge the transfer of UE MM context.

If the authentication/security procedure fails, then the Registration shall be rejected, and the new AMF sends a reject indication to the old AMF. The old AMF continues as if the Information Request was never received.

11.	[Conditional] new AMF to UE: Identity Request ().

If the PEI was not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE to retrieve the PEI.

12.	Optionally the new AMF initiates ME identity check.

The PEI check is performed as described in clause 4.7.

13.	If step 14 is to be performed, the new AMF, based on the SUPI, selects a UDM.

The AMF selects an UDM as described in TS 23.501 [2], clause X.

14.	If the AMF has changed since the last registration, or if there is no valid subscription context for the UE in the AMF, or if the UE provides a SUPI which doesn't refer to a valid context in the AMF, or if the UE registers to the same AMF it has already registered to a non-3GPP access (i.e. the UE is registered over a non-3GPP access and initiates this registration procedure to add a 3GPP access), the new AMF initiates the Update Location procedure. The new AMF provides the access type it serves for the UE to the UDM and the access type is set to "3GPP access". The UDM stores the associated access type together with the serving AMF. This will cause the UDM to initiate a Cancel Location to the old AMF corresponding to 3GPP access, if one exists. The old AMF removes the MM context and notifies all possibly associated SMF(s), and the new AMF creates an MM context for the UE after getting the AMF related subscription data from the UDM.

The Update Location procedure is performed as described in clause X.

If the Registration type is "initial Registration" and the AMF determined that the old node is an MME from UE's 5G-GUTI and the AMF is configured to support EPC interworking without Nx procedure, the AMF does not include the "initial Registration" indication to the UDM.

Editor's note:	Whether interactions with UDM and PCF includes mobility restriction related information is FFS.

Editor's note:	The PEI is to be provided to the UDM in the Update Location procedure.

Editor's note:	It is FFS how to update the figure with the "UDM initiates Cancel Location to old AMF".

15.	Conditionally the new AMF, based on the SUPI, selects a PCF.

The AMF selects a PCF as described in TS 23.501 [2], clause X.

Editor's note:	It is FFS whether the Permanent User ID is used for PCF selection.

Editor's note:	The conditions when the AMF selects a PCF is FFS.

16.	[Optional] new AMF to PCF: UE Context Establishment Request (SUPI).

If the AMF has not yet obtained Access and Mobility policy for the UE or if the Access and Mobility policy in the AMF are no longer valid, the AMF requests the PCF to apply operator policies for the UE.

17.	PCF to new AMF: UE Context Establishment Acknowledged (Access and Mobility policy data).

The PCF acknowledges the UE Context Establishment Request message and provides the Access and Mobility policy data for the UE to the AMF.

18.	[Conditional] new AMF to SMF: N11 Request ().

The N11 Request is sent in one or both of the following scenarios:

-	If the AMF is changed, the new AMF notifies each SMF of the new AMF serving the UE. The AMF verifies PDU session status from the UE with the available SMF information. In case the AMF has changed, it is assumed that the old AMF provides the available SMF information. The new AMF requests the SMF to release any network resources related to PDU sessions that the UE indicated as not established.

-	If the UE was in MICO mode and the AMF had notified an SMF of the UE being unreachable and that the SMF needs not to send DL data notifications to the AMF, the AMF informs the SMF that the UE is reachable.

NOTE:	the AMF will also notify any other NF that subscribed to UE reachability that the UE is reachable.

Editor's note:	It is FFS whether AMF perform the PDU session status logic or each SMF does it.

19.	SMF to new AMF: N11 Response ().

The SMF may decide to trigger e.g. UPF relocation as described in clause 4.3.5.

If the Registration type indicated by the UE is periodic registration update, then steps 20 and 21 may be omitted.

Editor's note:	It is FFS how SMFs subscribes to AMF services.

Editor's note:	SMF relocation in relation to a Registration procedure is FFS.

20.	[Conditional] old AMF to PCF: UE Context Termination Request ().

If the old AMF previously requested UE context to be established in the PCF, the old AMF terminates the UE context in the PCF.

21.	PCF to old AMF: UE Context Termination Acknowledged ().

22.	New AMF to UE: Registration Accept (5G-GUTI, Registration area. Mobility restrictions, PDU session status, NSSAI, Periodic registration update timer, LADN Information and accepted MICO mode, No-Nx EPC Interworking Indicator).

The AMF sends a Registration Accept message to the UE indicating that the registration has been accepted. 5G-GUTI is included if the AMF allocates a new 5G-GUTI. Mobility restrictions is included in case mobility restrictions applies for the UE. The AMF indicates the established PDU sessions to the UE in the PDU session status. The UE removes locally any internal resources related to PDU sessions that are not marked as established in the received PDU session status and for which the UE has requested PDU session establishment and not received SMF response yet. If the PDU session status information was in the Registration Request, the AMF shall indicate the PDU session status to the UE. The NSSAI includes the Allowed S-NSSAIs. If the UE subscription data includes subscribed LADN identification information, the AMF shall include in the Registration Accept message the LADN Information for LADNs, defined in TS 23.501 [2] clause 5.6.5, that are available within the Registration area determined by the AMF for the UE. If the UE included MICO mode in the request, then AMF responds whether MICO mode should be used.

If the "PDU session(s) to be re-activated" is included in the Registration Request, the AMF sends an N11 Message to SMF(s) associated with the PDU Session ID(s) to establish the User Plane(s) for the PDU session(s) in conjunction with the Registration Accept message. The steps 4b to 12 specified in clause 4.2.3.2 are executed to complete the User Plane(s) setup without sending MM NAS Service Accept from the AMF to (R)AN described in step 5 of clause 4.2.3.2.

If the Registration type is "initial Registration" and the AMF determined that the old node is an MME from UE's 5G-GUTI and the AMF is configured to support EPC interworking without Nx procedure, the AMF includes a "No-Nx EPC Interworking" indicator to the UE.

Editor's note:	It is FFS whether NSSAI is provided to RAN by AMF.

Editor's note:	It is FFS whether LADN subscription information (i.e. information which indicates whether the UE subscribes the LADN service) is included in UE subscription data or is managed by policy information by PCF.

23.	[Conditional] UE to new AMF: Registration Complete ().

The UE sends a Registration Complete message to the AMF to acknowledge if a new 5G-GUTI was assigned.

When the "PDU session(s) to be re-activated" is not included in the Registration Request, the AMF releases the signalling connection with UE, according to clause 4.2.6.

Editor's note:	Details of N2 interaction is FFS.

### **************  NEXT CHANGES    **********************

##### 4.3.2.2.1	Non-roaming and Roaming with Local Breakout

This clause specifies PDU Session establishment in the non-roaming and roaming with local breakout cases. The procedure is used to establish a new PDU session as well as to hand over an existing PDU Session between 3GPP access and non-3GPP access.

In case of roaming, the AMF determines if a PDU session is to be established in LBO or Home Routing. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

The procedure assumes that the UE has already registered on the AMF thus the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI, DNN, PDU Session ID, Request type, N1 SM information).

In order to establish a new PDU session, the UE generates a new PDU Session ID.

The UE initiates the UE Requested PDU Session establishment procedure by the transmission of a NAS message containing a PDU Session Establishment Request within the N1 SM information. The PDU Session Establishment Request may include a PDU Type, SSC mode, Protocol Configuration Options.

The Request Type indicates "Initial request" if the PDU Session Establishment is a request to establish a new PDU Session and indicates "Existing PDU Session" if the request refers to an existing PDU Session between 3GPP access and non-3GPP access or to an existing PDN Connection in EPC.

The NAS message sent by the UE is encapsulated by the AN in a N2 message towards the AMF that should include User location information and Access Technology Type Information.

The N1 SM information may contain SM PDU DN Request Container containing information for the PDU session authorization by the external DN.

The AMF receives from the AN the NAS SM message (built in step 1) together with User Location Information (e.g. Cell Id in case of the RAN).

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on that Request Type indicates "initial request" and that the PDU Session ID that is not used for any existing PDU Session(s) of the UE. If the NAS message does not contain an S-NSSAI, the AMF may determine a default S-NSSAI for the requested PDU session according to the UE subscription. The AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2. The AMF stores an association of the PDU session ID and the SMF ID.

The case where the Request Type indicates "Existing PDU Session", and either the AMF does not recognize the PDU Session ID or the subscription context from UDM does not contain an SMF ID corresponding to the DNN constitutes an error case.

If the Request Type indicates "Existing PDU Session", the AMF selects the SMF provided by the HSS+UDM in the subscription information corresponding to the DNN provided by the UE.

Editor’s Note: It is FFS if the network may decide not to allow PDU session continuity based on operator policy eg. for specific DNNs.

3.	From AMF to SMF: SM Request (Subscriber Permanent ID, DNN, Handover Indication, S-NSSAI, PDU Session ID, AMF ID, N1 SM information (PDU Session ID, PDU Session Establishment Request), User location information, Access Technology Type, PEI).

The AMF ID uniquely identifies the AMF serving the UE. The N1 SM information contains the PDU Session Establishment Request received from the UE. The PEI (Permanent Equipment Identifier) is defined in 23.501 [2] clause 5.9.3.

If the Request Type indicates "Existing PDU Session", Handover Indication is included.

4a.	SMF to UDM: Subscription Data Request (Subscriber Permanent ID, DNN).

If the Request Type in step in step 3 indicates "Existing PDU Session" the SMF determines that the request is due to handover between 3GPP access and non-3GPP access. The SMF identifies the existing PDU Session based on the PDU Session ID.

If the SMF has not yet retrieved the SM-related subscription data for the UE related with the DNN, the SMF requests this subscription data.

4b.	UDM to SMF: Subscription Data Response.

Subscription data includes the authorized PDU type(s), authorized SSC mode(s), Default QoS profile.

Editor's note:	Further detailing of SM-related subscription data is FFS.

The SMF checks whether the UE request is compliant with the user subscription and with local policies. If that is not the case the SMF rejects the UE request via NAS SM signalling (including a relevant SM rejection cause) relayed by the AMF, the SMF indicates to the AMF that the PDU session ID is to be considered as released and the rest of the procedure is skipped.

5.	SMF to DN via UPF

If the SMF needs to authorize/authenticate the establishment of the PDU session as described in clause  5.6.6 of TS 23.501 [2], the SMF selects an UPF as described in TS 23.501 [2] clause 6.3.3 and triggers the PDU session establishment authentication/authorization as described in clause 4.3.2.3.

If the PDU session establishment authentication/authorization fails, the SMF terminates the PDU session establishment procedure and indicates a rejection to the UE.

6a.	If dynamic PCC is deployed, the SMF performs PCF selection.

6b.	The SMF may initiate PDU-CAN Session Establishment towards the PCF to get the default PCC Rules for the PDU Session. If the Request Type in step 3 indicates "Existing PDU Session", the PCF initiates a PDU-CAN Session Modification instead.

Editor's note:	The purpose of step 5 is to receive PCC rules before selecting UPF. If PCC rules are not needed as input for UPF selection, step 5 can be skipped.

7.	If the Request Type in step 3 indicates "Initial request", the SMF selects an SSC mode for the PDU Session. If step 5 is not performed, the SMF also selects an UPF as described in TS 23.501 [2] clause 6.3.3. In case of PDU Type IPv4 or IPv6, the SMF allocates an IP address/prefix for the PDU Session as described in TS 23.501 [2] clause 5.8.1. For Unstructured PDU Type the SMF may allocate an IPv6 prefix for the PDU session and N6 point-to-point tunnelling (based on UDP/IPv6) as described in TS 23.501 [2] clause 5.6.9.4.

If the Request Type is "Existing PDU Session", the SMF maintains the same IP address/prefix that has already been allocated to the UE in the source network.

8.	If dynamic PCC is deployed and the PDU-CAN Session Establishment has not yet been done, the SMF initiate PDU-CAN Session Establishment towards the PCF to get the default PCC Rules for the PDU Session. Otherwise, if Request Type is "initial request" and dynamic PCC is deployed and PDU Type is IPv4 or IPv6, SMF initiates PDU-CAN Session Modification and provides the allocated UE IP address/prefix to the PCF.

9.	If Request Type indicates "initial request" and step 5 was not performed, the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF:

9a.	The SMF sends an N4 Session Establishment/Modification Request to the UPF and provides Packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to UPF in this step.

9b.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If CN Tunnel Info is allocated by the UPF, the CN Tunnel Info is provided to SMF in this step.

If the Request Type indicates "Existing PDU Session", and the SMF creates CN tunnel information, then this step is skipped. Otherwise, this step is performed to obtain the CN tunnel information from the UPF using the N4 Session Modification Procedure.

10.	SMF to AMF: SM Response (Cause, N2 SM information (PDU Session ID, QoS Profile(s), CN Tunnel Info), N1 SM information (PDU Session Establishment Accept (Authorized QoS Rule, SSC mode, S-NSSAI, allocated IPv4 address))).

Editor's note:	It is FFS how the AMF and SMF associate the N11 transactions with a corresponding UE context.

The N2 SM information carries information that the AMF shall forward to the (R)AN which includes:

The CN Tunnel Info corresponds to the Core Network address of the N3 tunnel corresponding to the PDU session.

The QoS Profile provides the (R)AN with the mapping between QoS parameters and QoS Flow Identifiers. Multiple QoS profiles can be provided to the (R)AN. This is further described in TS 23.501 [2] clause 5.7.

The PDU Session ID may be used by AN signalling with the UE to indicate to the UE the association between AN resources and a PDU session for the UE.

The N1 SM information contains the PDU Session Establishment Accept that the AMF shall provide to the UE.

Multiple Authorized QoS Rules may be included in the PDU Session Establishment Accept within the N1 SM information and in the N2 SM information.

The SM Response further contains PDU Session ID and information allowing the AMF to know which the target UE as well to determine which access towards the UE to use.

NOTE:	The access information is to deal with the case where a UE is simultaneously connected over 3GPP and Non 3GPP access.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to determine which access towards the UE to use.

11.	AMF to (R)AN: N2 PDU Session Request (N2 SM information, NAS message (PDU Session ID, PDU Session Establishment Accept)).

The AMF sends the NAS message containing PDU Session ID and PDU Session Establishment Accept targeted to the UE and the N2 SM information received from the SMF within the N2 PDU Session Request to the (R)AN.

12.	(R)AN to UE: The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE establishing the necessary RAN resources related to the Authorized QoS Rules for the PDU Session request received in step 10.

(R)AN also allocates (R)AN N3 tunnel information for the PDU Session.

(R)AN forwards the NAS message (PDU Session ID, N1 SM information (PDU Session Establishment Accept)) provided in step 10 to the UE. (R)AN shall only provide the NAS message to the UE if the necessary RAN resources are established and the allocation of (R)AN tunnel information are successful.

13.	(R)AN to AMF: N2 PDU Session Response (PDU Session ID, Cause, N2 SM information (PDU Session ID, (R)AN Tunnel Info, List of accepted/rejected QoS profile(s))).

The (R)AN Tunnel Info corresponds to the Access Network address of the N3 tunnel corresponding to the PDU session.

14.	AMF to SMF: SM Request (N2 SM information, Handover Indication).

The AMF forwards the N2 SM information received from (R)AN to the SMF.

Editor's note:	It is FFS if steps are needed to indicate from the UE to the core network that the UE has successfully established the PDU Session, or whether it is sufficient with a successful establishment in (R)AN indicated in step 12. For example, it is FFS if the UE shall send a NAS PDU Session Establishment Complete message to indicate that the UE has successfully established the PDU Session.

If Request Type indicates "Existing PDU Session", the Handover Indication is also included

15a.	If the N4 session for this PDU Session was not established already, the SMF initiates an N4 Session Establishment procedure with the UPF. Otherwise, the SMF initiates an N4 Session Modification procedure with the UPF. The SMF provides AN Tunnel Info and CN Tunnel Info. The CN Tunnel Info only needs to be provided if the SMF selected CN Tunnel Info in step 8.

NOTE:	If the PDU Session Establishment Request was due to mobility between 3GPP and non-3GPP access, the downlink data path is switched towards the target access in this step.

15b.	The UPF provides a N4 Session Establishment/Modification Response to the SMF.

16.	SMF to AMF: SM response (Cause).

After this step, the AMF forwards relevant events to the SMF, e.g. at handover where the (R)AN Tunnel Info changes or the AMF is relocated.

Editor's note:	It is FFS if the SMF has to explicitly subscribe to these events or whether the subscription is implicit.

17.	SMF to UE, via UPF: In case of PDU Type IPv6, the SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the UPF.

18.	If the PDU Session Establishment request is due to a handover between 3GPP access and non-3GPP access, i.e. the Request type is set to "existing PDU Session" the SMF executes the steps X-Y specified in clause 4.3.4 in order to release the user-plane over the source access (3GPP or non-3GPP access).

Editor's note:	The exact reference to steps of network initiated PDU session release procedure will be defined when the procedure is available.

If the PDU Session Establishment request is due to a handover from EPC, the SMF+PGW-C shall initiate the PDN GW Initiated PDN Disconnection procedure in EPC as defined in clause 5.10.3 of TS 23.401[x]

19.	If the SMF identity is not included in step 4b by the UDM in the DNN subscription context, the SMF invokes the "UDM_Register UE serving NF" service (described in 5.2.3.1) including the SMF address and the DNN. The UDM stores the SMF identity, address and the associated DNN.

If during the procedure the PDU session establishment is not successful, the SMF informs the AMF. Once the AMF associates an SMF with a PDU session ID, the SMF is automatically subscribed to notifications of incoming N1 signalling associated with this PDU session ID. These notifications automatically provide any User Location information and Access Type that the AMF has received from the (R)AN in association with N1 signalling.

### **************  END CHANGES    **********************