SA WG2 Meeting #121	S2-174679

26 - 30 June 2017, San Jose Del Cabo

Source:	China Telecom

Title:	23.501 NSSF Services

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution specifies the services of NSSF to 23.501.

- Discussion

During the SA2#121, there were discussions on two architecture approaches to support the Network Slice Instance Selection,i.e. NSSF vs. NRF+. Measuring the two options, additionally there is a statement in the editor's note in clause 6.2.6 of 23.501 that whether NRF is used only for NF instance discovery or both NF discovery and selection is FFS, we would like to offer a specified description on independent NS instance selection for the discussion.

2	Proposal

It is proposed to agree on the proposed changes to TS 23.501.

*************** Start of changes *********************

7.2.10	NSSF Services

The following NF services are specified for NSSF:

# Table 7.2.9-1: NF Services provided by NSSF

| Service Name | Description | Reference in TS 23.502 [3] | Example Consumer | Mechanism |
| --- | --- | --- | --- | --- |
| Nnssf_NSSelection | Provides the selected NS and optional AMF information to the Requester. | 5.2.12 | AMF/V-NSSF | Request-Response |

*************** End of changes *********************