SA WG2 Meeting #122	S2-174682

26 - 30 June 2017, San Jose Del Cabo

Source:	China Telecom

Title:	Update to 23.502 NRF Services

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution updates the description of NRF services in TS 23.502.

- Introduction

According to NRF service in TS 23.502 clause 5.2.7, NRF service is reorganised into the following table.

There is also a discussion on NF service discovery. As specified in TS 23.502, clause 5.1.1, NF service discovery is enabled via the NF discovery. For enabling NF service discovery, the service related parameters should be optionally into the input information. Besides, for NF service discovery, it is FFS that whether the identifier of target service is included in the discovery response, or the IP address or FQDN of the NF instance(s) able to provide the expected NF Service is sufficient.

2	Proposal

It is proposed to agree on the proposed changes to TS 23.502.

*************** Start of changes *********************

5.2.7	NRF Services

5.2.7.1	General

The following table illustrates the NRF Services.

# Table 5.2.7.1-1: NF Services provided by NRF

| NF service | Description | NF service operations | Known Consumer (s) | Mechanisms | Reference |
| --- | --- | --- | --- | --- | --- |
| Nnrf_NF Discovery | provides the IP address or FQDN of the expected NF instance(s) to the consumer NF | NF Discovery | AMF, SMF, PCF, NEF, NRF in another PLMN | Request-Response | 4.2.2.2 (step 8,13,15) 4.2.2.2.3 (step 3,4) 4.3.2.2.1 (step 6,7) 4.3.6.2 (step 3) 4.13.3.1 (step 2) |

Editor's note: Whether NRF is used only for NF instance discovery or both NF discovery and selection is FFS.

Editor’s note: NF and NF services may in principle be discovered independently, however in this release of the specification, the independent discovery of a service within a particular System Procedure will be evaluated on a case by case basis.

5.2.7.2	Nnrf_NF Discovery Service

5.2.7.2.1	General

# Table 5.2.7.2.1-1: Nnrf_NF Discovery service operations

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| NF Discovery | NRF receives the NF Discovery Request from the NF consumer intend to discover the expected NF instance(s), authorizes the NF Discovery Request and determines whether the NF consumer is allowed to discover the expected NF instance(s). If allowed, the NRF determines the discovered NF instance(s) and provides the information of a set of discovered NF instance(s) to the NF consumer via NF Discovery Response message. | Inputs, Required: NF type of the target NF, NF type of the NF consumer. Inputs, Optional: slice information of the target NF, PLMN ID of the PLMN target NF belongs to, service related parameters | the IP address or FQDN of the expected NF instance(s) |

Service operation supplementary description:

When the NF consumer intends to discover the NF in another PLMN, the NRF in serving PLMN needs to request "NF Discovery" service from NRF in the remote PLMN. The NRF in serving PLMN identifies NRF in remote PLMN based on the Remote PLMN ID, and it requests "NF Discovery" service from NRF in remote PLMN to get the expected NF instance (s) deployed in the remote PLMN. As the NRF in the serving PLMN triggers the "NF Discovery" on behalf of the NF consumer, the NRF in the serving PLMN shall not replace the information of the service requester NF, i.e. NF consumer ID, in the Discovery Request message it sends to the NRF in remote PLMN.

5.2.7.2.2	Nnrf_NF Discovery service operations

Service operation name: Nnrf_NF Discovery

Description: provides the IP address or FQDN of the expected NF instance(s) to the consumer NF.

Known NF Consumers: AMF, SMF, PCF, NEF, NRF in another PLMN

Concurrent use: none

Pre-requisite conditions: NF consumer is triggered to communicate with a target NF instance

Post conditions: NRF stores the NF consumer information, e.g. consumer NF ID.

Inputs, Required: NF type of the target NF, NF type of the NF consumer.

Inputs, Optional: slice information of the target NF, PLMN ID of the PLMN target NF belongs to, service related parameters

NOTE:	Other service related parameters need depend on the NF type of the expected NF instance(s) and refer to the clause 6.2 "Network Function Selection" in TS 23.501 [2].

Editor's note:	what parameter is used as slice information is FFS

Editor’s note: NF and NF services may in principle be discovered independently, however in this release of the specification, the independent discovery of a service within a particular System Procedure will be evaluated on a case by case basis.

Outputs, Required: FQDN and IP address of a set of requested target NF

Outputs, Optional: none

Editor's note: for NF service discovery, it is FFS that whether the identifier of target service is included in the discovery response, or the IP address or FQDN of the NF instance(s) able to provide the expected NF Service is sufficient.

See clause 4.2.2.2 (step 8, 13, 15) and 4.2.2.2.3 (step 3, 4) for the detail usage of this service operation for AMF in the registration procedures. In 4.2.2.2 (step 8, 13, 15), the initial AMF discovers and selects AUSF, UDM and PCF. In 4.2.2.2.3 (step 3, 4), the initial AMF discovers and selects the target AMF in the registration procedure with AMF relocation.

See clause 4.3.2.2.1 (step 6, 7) for the detail usage of this service operation for SMF in the session management procedures. In 4.3.2.2.1 (step 6), SMF discovers and selects PCF in the PDU Session establishment. In 4.3.2.2.1 (step 7), SMF discovers and selects UPF in PDU Session establishment.

See clause 4.3.6.2 (step 3) for the detail usage of this service operation for PCF discovery and selection in Processing AF requests to influence traffic routeing. In 4.3.6.2 (step 3), the selection of the PCF(s) that will receive the AF request is done by the NEF when a NEF is involved or by the AF itself otherwise.

See clause 4.13.3.1 (step 2) for the detail usage of this service operation for AMF in the SMS over NAS procedures. In 4.13.3.1 (step 2), AMF discovers and selects an SMSF.

*************** End of changes *********************