SA WG2 Meeting #122	S2-174874

26 - 30 June 2017   San Jose Del Cabo, Mexico

Source:	Huawei

Title:	TS23.502: Clarifications on NWDA Service Based Interface

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This document proposes alignment between TS23.501 and TS23.502 on the definition of the reference point N23 as service-oriented interface exhibited by NWDA.

Discussion

TS 23.502 v0.4.0 section sec 5.2.11 lists and describes the services provided by NWDA. In that section, N23 is described as a service-oriented interface exhibited by NWDA. However, TS23.501 v0.5.0 only defines N23 as a reference point between NWDA and PCF.

In addition, TS 23.502 v0.4.0 section sec 5.2.11 defines the possibility of PCF to retrieve data analytics from NWDA via the N23_Request/Response services, hence not necessarily subscribing to NWDA events.

Editorial clarifications for TS23.502 v0.4.0 are proposed.

Proposal

The following (tracked) changes to TS 23.502 v0.4.0 are proposed.

***** START of 1st CHANGE *****

5.2.11	NWDA Services

Editor's note:`How NWDA services work while roaming is FFS.

5.2.11.1	General

The following table illustrates the NWDA Services.

Table 5.2.11.1-1: NF Services provided by NWDA

| Service Name | Service Operations | Known Consumer(s) | Operation Semantic | Reference |
| --- | --- | --- | --- | --- |
| Nnwda_Events_Subscription | Subscribe | PCF | Subscribe / Notify | FFS |
|  | Unsubscribe | PCF | Subscribe / Notify | FFS |
|  | Notify | PCF | Subscribe / Notify | FFS |
| Nnwda_Analytics_Info | Request | PCF | Request/ Response | FFS |
|  | Response | PCF | Request/ Response | FFS |

5.2.11.2	Nnwda_Events_Subscription Service

5.2.11.2.1	General

Service Description: this service enables the consumer to subscribe/unsubscribe for NWDA slice congestion events notification. Periodic notification and notification upon threshold exceeded can be subscribed.

5.2.11.2.2	Nnwda_Events_Subscription_Subscribe service operation

Service operation name: Nnwda_Events_Subscription_Subscribe

Description: Subscribe  to NWDA event with specific parameters.

Known NF Consumers: PCF

Inputs Required: Slice information . Includes parameters listed in Table 5.x.x-1 for the specific NWDA subscribed service being requested.

Editor's note:	The Slice Information rather than the ID regarding to the SCI generation should be provided. The detailed information is FFS.

Inputs, Optional: Periodicity, SCI Threshold value.

Outputs Required: Confirmation of the Subscription.

Outputs, Optional: none.

5.2.11.2.3	Nnwda_Events_Subscription_Unsubscribe service operation

Service operation name: Nnwda_Events_Subscription_Unsubscribe

Description: unsubscribe to NWDA event with specific parameters.

Known NF Consumers: PCF

Inputs, Required: Slice information, Includes parameters listed in Table 5.x.x-1 for the specific NWDA unsubscribed service being requested.

Editor's note:	The Slice Information rather than the ID regarding to the SCI generation should be provided. The detailed information is FFS.

Inputs, Optional: Periodicity, SCI Threshold value.

Outputs, Required: Confirmation of the unsubscription.

Outputs, Optional: none.

5.2.11.2.4	Nnwda_ Events_Subscription_Notify service operation

Service operation name: Nnwda_Events_Subscription_Notify

Description: NWDA notifies the consumer instance of the event that has subscribed to the specific NWDA service. Depending upon type of subscription this notification is either on a periodic basis or triggered whenever a threshold is crossed.

Known NF Consumers: PCF

Inputs Required: Earlier asynchronous subscription to the event.

Inputs, Optional: FFS.

Outputs, Required: Message Notify with parameter(s) to the consumer  instance that had subscribed to the event.

Outputs, Optional: FFS.

5.2.11.3	Nnwda_Analytics_Info service

5.2.11.3.1	General

Service description: this service enables the consumer to requests and get from NWDA  operator specific analytics. These represent operator specific analytics that have a meaning only in its network.

Editor's note:	Format and details of Analytic are FFS.

5.2.11.3.2	Nnwda_Analytics_Info_Request service operation

Service operation name: Nnwda_Analytics_Info_Request

Description: the consumer requests NWDA operator specific analytics.

Known NF Consumers: PCF.

Inputs Required: Analytic ID identifying the requested Operator specific Analytic. The request  includes parameters listed in Table 5.x.x-2 for the specific NWDA service being requested.

Inputs, Optional: FFS.

Outputs, Required: None.

Outputs, Optional: None.

5.2.11.3.3	Nnwda_Analytics_Info_Response service operation

Service operation name: Nnwda_Analytics_Info_Response

Description: the NWDA returns to the consumer the requested operator specific analytics.

Known NF Consumers: PCF

Inputs, Required: None.

Inputs, Optional: None.

Outputs, Required: Analytics with parameters indicated in Table 5.x.x-3.

Outputs, Optional: None.

***** END of 1st CHANGES *****