SA WG2 Meeting #122	S2-175020

26 - 30 June 2017, San Jose del Cabo, Mexico	(revision of S2-174429)

Source:	LG Electronics

Title:	TS 23.501: Proposal for Control plane protocol stack: for NAS

Document for:	Discussion / Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes a protocol stack model for 5GS control plane, especially between the UE and the 5GC.

1.	Discussion

This contribution proposes a protocol stack model for 5GS control plane, especially between the UE and the 5GC. There were some related contributions on this issue, and this contributions tries to make a consolidated general solution proposal, based on the agreements in other WGs, e.g. RAN3, CT1.

Detailed discussion on the open issue is depicted in the discussion paper in S2-174428.

Some notable suggestions in this contribution are as follow:

- Structure of the clause

- As suggested in S2-172649, the subclauses under the clause 8.2 Control plane protocol stack are structured as “AN-5GC stacks”, “UE-5GC stacks” and “for N3GPP access”.

- Also for each subclause, general descriptions are introduces as suggested in S2-172649

- UE-5GC protocol stack (NAS / N1)

- As suggested in S2-172649, protocol stack between the UE and 5GC (AMF and SMF) is similar.

- Changed the name of the NAS protocol by removing “N1” since N1 is the name of the interface, and it is not necessarily included in the name of the protocol.

- For the name of the protocol, suggested to use “NAS-MM” to be aligned with CT WG1, who are using “5GMM” functionality.

- Proposal 1: There is a single NAS protocol layer between the UE and the AMF. This NAS protocol is responsible for 5GMM functionalities, including transport of non-MM messages between UE and AMF. This layer can be referred as “NAS-MM”

- ENs are added regarding the name of the NAS protocol.

** Update from S2-174429

- Remove the change for N2 related stacks.

- Update the general description accommodating Qualcomm’s proposal merged.

- Add a high-level figure for NAS transport of non-MM message.

- Add a reference to 8.2.4 for N3GPP access stack.

- Add an EN regarding whether other NAS protocol for NFs other than SMF is needed.

2.	Proposal

According to the discussion above, we would like to propose to add the following texts in TS 23.501

* * * * Start of 1st Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 4282: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[xa]	IETF RFC 4960: "Stream Control Transmission Protocol".

[xb]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[xc]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[xd]	3GPP TS 38.413: " NG Radio Access Network (NG-RAN); NG Application Protocol".

* * * * Start of 2nd Change * * * *

8.2	Control plane Protocol stacks

8.2.1	Control Plane between the AN and the 5G Core: N2

8.2.1.1	General

Editor's note:	N2 maps to NG-C as defined by RAN groups in TS 38.413 [xx].

Following procedures are defined over N2:

-	Procedures related with N2 Interface Management and that are not related to an individual UE, such as for Configuration or Reset of the N2 interface. These procedures are intended to be applicable to any access but may correspond to messages that carry some information only on some access (such as information on the default Paging DRX used only for 3GPP access).

-	Procedures related with an individual UE:

-	Procedures related with NAS Transport. These procedures are intended to be applicable to any access but may correspond to messages that for UL NAS transport carry some access dependent information such as User Location Information ( e.g. Cell-Id over 3GPP access or other kind of User Location Information  for Untrusted Non 3GPP access).

-	Procedures related with UE context management. These procedures are intended to be applicable to any access. The corresponding messages may carry:

-	some information only on some access (such as Handover Restriction List used only for 3GPP access).

-	some information (related e.g. with N3 addressing and with QoS requirements) that is to be transparently forwarded by AMF between the (R)AN and the SMF.

.-	Procedures related with resources for PDU sessions. These procedures are intended to be applicable to any access. They may correspond to messages that carry information (related e.g. with N3 addressing and with QoS requirements) that is to be transparently forwarded by AMF between the (R)AN and the SMF.

-	Procedures related with Hand-Over management. These procedures are intended for 3GPP access only.

The Control Plane interface between the AN and the 5G Core supports:

-	The connection of multiple different kinds of AN (e.g. 3GPP RAN, N3IWF for Un-trusted access to 5GC) to the 5CG via an unique Control Plane protocol: A single N2 AP protocol is used for both the 3GPP access and non-3GPP access.

-	The decoupling between AMF and other functions such as SMF that may need to control the services supported by AN(s) (e.g. control of the UP resources in the AN for a PDU session).

-	For this purpose, N2-AP may support a subset of messages and Information that the AMF is just responsible to relay between N2 and the SMF. This subset of N2-AP is called N2-AP-SM for the SMF.

8.2.1.2	Control Plane Interface between the AN and the 5G Core

Editor's note:	FFS.

8.2.2		Control Plane protocol stacks between the UE and the 5GC

8.2.2.1	General

The NAS protocol on N1 comprises a NAS-MM and a NAS-SM components.

There are multiple cases of protocols between the UE and a core network function (excluding the AMF) that need to be transported over N1. Such cases include:

- Session Management Signalling

- SMS

Editor’s note: It is FFS if other protocols (e.g. LCS) are defined in this Release.

The NAS-MM supports generic capabilities:

-	NAS procedures that terminate at the AMF. This includes:

-	Handles Registration Management and Connection Management state machines and procedures, including NAS transport;

-	Provide a secure NAS connection (integrity protection, ciphering) between the UE and the AMF, including for the transport of payload;

-	Provide access control if it applies;

-	NAS transport of different types of payload or messages that do not terminate at the AMF, e.g. NAS-SM, SMS, between the UE and the AMF. This includes:

-	Information about the Payload type;

-	Additional Information for forwarding purposes

-	The Payload (e.g. the SM message in case of SM signalling);

Editor’s note: It is up to CT1 to decide whether the same NAS messages are used for transporting SM signalling, SMS, other services, or different NAS messages are needed.

-	There is a Single NAS protocol that applies on both 3GPP and non-3GPP access.

Security of the NAS messages is provided based on the security context established between the UE and the AMF.

Figure 8.2.2.1-1 depicts NAS transport of SM signalling and SMS as example.

Figure 8.2.2.1-1 Example of NAS transport for SM, SMS and other services

8.2.2.2	UE - AMF

Legend:

-	NAS-MM: The NAS protocol for MM functionality supports registration management functionality, connection management functionality and user plane connection activation and deactivation. It is also responsible of ciphering and integrity protection of NAS signalling. 5G NAS protocol is defined in TS 24.xxx [xx]

-	AN Protocol layer: This set of protocols/layers depends on the AN. In case of 3GPP RAN, the radio protocol between the UE and the AN node (eNodeB or gNodeB) is specified in TS 36.300 [xb] and TS 38.300 [xc]. In case of non-3GPP access, see clause 8.2.4.

Figure 8.2.2.2-1: Control Plane between the UE and the AMF

8.2.2.3	UE – SMF

The NAS-SM supports the handling of Session Management between the UE and the SMF.

The SM signalling message is handled, i.e. created and processed, in the SM layer of UE and the SMF. The content of the SM signalling message is not interpreted by the AMF.

The NAS-MM layer handles the SM signalling is as follows:

- For transmission of SM signalling:

-	The NAS-MM layer creates a NAS-MM message, including security header, indicating NAS transport of SM signalling, additional information for the receiving NAS-MM to derive how and where to forward the SM signalling message.

- For reception of SM signalling:

-	The receiving NAS-MM processes the NAS-MM part of the message, i.e. performs integrity check, interprets the additional information to derive how and where to derive the SM signalling message.

The SM message part shall include the PDU session ID.

Legend:

-	NAS-SM: The NAS protocol for SM functionality supports user plane PDU session establishment, modification and release. It is transferred via the AMF, and transparent to the AMF. 5G NAS protocol is defined in TS 24.xxx [xx]

Figure 8.2.2.3-1: Control Plane protocol stack between the UE and the SMF

Editor's note:	Whether the end-to-end NAS protocol between the UE and the NF other than AMF and SMF needs to be defined is FFS and is up to CT WG1.

8.2.3		Control Plane protocol stacks between the functions in 5GC

Editor's note:	How to design the control plane protocol stacks between 5GC functions which is defined as a service-based interface is FFS.

8.2.4	Control Plane for untrusted non 3GPP Access

Legend:

-	N2 Application Protocol (N2-AP): Application Layer Protocol between the N3IWF and the AMF.

-	N12 Application Protocol (N12-AP): Application Layer Protocol between the AMF and the AUSF.

-	The N3IWF creates a NAS Attach Request message on behalf of the UE and send this message over N2 to AMF

Figure 8.2.4-1: Control Plane for initial part of attach procedure via N3IWF

Editor's note:	The need for NAS layer between N3IWF and AMF is FFS.

Legend:

-	N2 Application Protocol (N2-AP): Application Layer Protocol between the N3IWF and the AMF.

-	IPsec transport mode and GRE is used to encapsulate NAS payload between UE and N3IWF.

Figure 8.2.4-2: Control Plane for NAS when CP IPsec SA is established

Editor's note:	It is FFS whether GRE encapsulation of NAS is needed or not.

Legend:

-	N2 Application Protocol (N2-AP): Application Layer Protocol between the N3IWF and the AMF.

-	IKEv2 is used to establish the Child SA between UE and N3IWF for each PDU session.

Figure 8.2.4-3: Control Plane for user plane establishment via N3IWF

Editor's note:	The name of N2-AP and the protocol stack for N2 should be confirmed with RAN WG3.

Editor's note:	The name of N12-AP and the protocol stack for N12 is FFS.

* * * * End of Changes * * * *