SA WG2 Meeting #121	S2-175249

26-30 June 2017, San Jose Del Cabo, Mexico	(was S2-17xxxx)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, Samsung, AT&T, SK Telecom, KT

Title:	TS 23.501: AMF Selection

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes description for AMF selection

# 1	Introduction

Terminology updates (Temporary ID) and clarification.

# 2	Proposal

It is proposed to modify TS 23.501 as follows…

* * * * Start Change * * *.*

## 6.3	Network function discovery and selection

### 6.3.5	AMF selection

The AMF selection functionality is applicable to both 3GPP access and non-3GPP access. The AMF selection functionality can be supported by the 5G-AN (e.g. RAN, N3IWF) and is used to select an AMF for a given UE. An AMF supports the AMF selection functionality to select an AMF for relocation or because the initially selected AMF was not an appropriate AMF to serve the UE (e.g. due to change of allowed NSSAI based on subscription).

Editor’s note: whether there are other NFs that will support AMF selection function is FFS.

Editor’s note: terminology alignment with GUTI definition needs to be done.

5G AN selects an AMF Set and an AMF from the AMF Set under the following circumstances:

- When the UE provides no 5G-S-TMSI nor the GUAMI to the 5G-AN.

- When the UE provides 5G-S-TMSI or GUAMI but the routing information (i.e. AMF identified based on AMF Set ID, AMF pointer) present in the 5G-S-TMSI or GUAMI is not sufficient and/or not usable (e.g. UE provides 5G-S-TMSI with an AMF region ID from a different region).

The AMF selection functionality in the 5G-AN may consider the following factors for selecting the AMF Set:

-	Requested NSSAI.

-	Local operator policies.

Editor’s Note: whether the N3IWF also considers N3IWF support for specific slices is FFS.

It considers the following factors for selecting an AMF from AMF Set:

-	Availability of candidate AMFs.

-	Load balancing across candidate AMFs (e.g. considering weight factors of candidate AMFs in the AMF Set).

When 5G-S-TMSI or GUAMI is provided by the UE to the 5G-AN contains an AMF Set ID that is usable and the AMF identified by AMF pointer that is not usable (e.g. AN detects that the AMF has failed) or the corresponding AMF indicates it is unavailable (e.g. out of operation), then the 5G-AN uses the AMF Set ID for AMF selection considering the factors above for selecting an AMF from the AMF Set.

Editor’s note: 	AMF selection will be revisited due to the solution principles for AMF planned maintenance, AMF change, network slicing etc is FFS.

The AMF selection functionality in the AMF shall utilize the NRF to discover the AMF instance(s) unless AMF information is available by other means, e.g. locally configured on AMF. The NRF provides the IP address or the FQDN of AMF instance(s) to the AMF. In addition to the factors listed above, AMF selection function within the AMF considers the following factor for selecting any AMF from the AMF Set:

-	Subscription information from UDM (e.g. allowed NSSAI).

* * * * End Change * * *.*