SA WG2 Meeting #123	S2-177066

October 23 – 27, 2017, Ljubljana, Slovenia	(was S2-17xxxx)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, Verizon

Title:	TS 23.502: OI#13 – UDM Services for subscriber data access.

Document for:	Approval

Agenda Item:	6.5.11 / 6.5.1

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes updates to UDM Services

# 1	Rationale and Proposal

It is possible for UDM to leverage UDR as a backend thus UDM can act as an FE to access UDR. Current UDM FE services assume that the subscription data retrieved by the NFs are structured i.e. even if UDM retrieves unstructured data from UDR, it returns structured data towards the requester e.g. AMF, SMF, SMSF. We propose that this should be made generic to also allow the possibility for UDM FE to return unstructured subscription data as a repository data to selective requester NFs e.g. PCF. This should be possible in case of some NFs such as PCF.

Proposal 1: Introduce the means for NF consumer (PCF) to retrieve repository data via UDM.

It seems the terms “Update subscribe request”, “Update unsubscribe request” are confusing as it gives the impression that the NF consumer can use this operation to update UE’s subscription in the UDM.

Proposal 2: Modify the name of the service operations to simply “subscribe”, “unsubscribe” request.

There are some NF consumers that will also need the ability to actually update i.e. modify and add information to UE’s subscription data in the UDM.

Proposal 3: Add a service operation for NF consumer to be able to update, provide new information to UE’s subscription data in the UDM.

During SA2#122E, known NF consumer was updated to Example NF consumer in alignment with the term used in the NF services table in TS 23.501. Thus, align the descriptions with that.

Proposal 4: Change known NF consumer -> Example NF consumer.

# 2	Proposed modification

It is proposed to modify TS 23.502 as follows:

* * * * Start Change * * *.*

### 5.2.3	UDM Services

#### 5.2.3.1	General

The following table illustrates the UDM Services.

Table 5.2.3-1: NF Services provided by UDM

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Subscriber Data Management | Get | Request/Response | AMF, SMF, SMSF, PCF |
|  | Subscribe Request | Subscribe/Notify | AMF, SMF, SMSF, PCF |
|  | Unsubscribe | Subscribe/Notify | AMF, SMF, SMSF, PCF |
|  | Notification | Subscribe/Notify | AMF, SMF, SMSF, PCF |
|  | Update | Request/Response | NEF, PCF |
| UE context | Registration | Request/Response | AMF, SMF, SMSF |
| Management | RemoveNotification | Subscribe/Notify | AMF, SMF, SMSF |
|  | Deregistration | Request/Response | AMF, SMF, SMSF |
|  | Get | Request/Response | NEF |
|  | Update | Request/Response | AMF |
| UE Authentication | Request | Request/Response | AUSF |
| EventExposure | Subscribe | Subscribe/Notify | NEF |
|  | Unsubscribe |  | NEF |
|  | Notify |  | NEF |

#### 5.2.3.2	Nudm_UEContextManagement service

Editor's note:	Whether NF type can be derived from other information or NF type is needed explicitly is FFS.

##### 5.2.3.2.1	Nudm_UEContextManagement_Registration

Service operation name: Nudm_UEContextManagement_Registration

Description: Register UE's serving NF on the UDM. This operation implies the following:

-	The authorization to provide subscription data for the corresponding NF consumer and the UE context (identified by input parameters). If this is successful the consumer is set as a serving NF for the corresponding UE context.

-	When the consumer is AMF, it is implicitly subscribed to be notified when it is deregistered in UDM. This notification is done by means of Nudm_UEContextManagement_DeregistrationNotification operation.

Example Consumers: AMF, SMF, SMSF

Inputs, Required: NF ID, SUPI, NF type, Access type (if NF type is AMF), DNN (if NF type is SMF).

Inputs, Optional:

Editor's note:	Whether this operation may convey an indication for subscription data retrieval is FFS. This depends on whether UE Context Management and Subscriber Data Management services are defined as a single or separated services, and whether Registration and data Get are considered two independent operations, what is as well FFS.

Outputs, Required: Result indication.

Outputs, Optional: None.

See step 14 of clause 4.2.2.2.2 for an example usage of this service operation.

##### 5.2.3.2.2	Nudm_UEContextManagement_DeregistrationNotification

Service operation name: Nudm_UEContextManagement_DeregistrationNotification

Description: UDM notifies the NF consumer which has previously registered (using Nudm_UEContextManagement_Registration operation) the NF ID has been deregistered in the UDM. As a result, the consumer is no longer registered in UDM as a serving NF for that UE.

NOTE:	This notification corresponds to an implicit or explicit subscription.

Example Consumers: AMF, SMF, SMSF

Inputs, Required: SUPI, serving NF removal reason.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

See step 14c of sub-clause 4.2.2.2.2 for an example usage of this service operation. The serving NF removal reason tells the reason for sending the remove notification to the consumer NF. The reason can be one of the following:

-	UE Initial Registration.

-	UE Registration area change.

-	Subscription Withdrawn.

Editor's note:	Additional reasons applicable for remove notification towards SMF, SMSF are FFS.

##### 5.2.3.2.3	Nudm_UEContextManagement_Deregistration

Service operation name: Nudm_UEContextManagement_Deregistration

Description: The NF consumer requests the UDM to delete the information related to the NF in the UE context. This implies that subscriptions to notification are as well removed.

Example Consumers: AMF, SMF, SMSF

Inputs, Required: SUPI, NF type, Access type

Inputs, Optional: None

Outputs, Required: Result Indication.

Outputs, Optional: None

##### 5.2.3.2.4	Nudm_UE ContextManagement_Get

Service operation name: Nudm_UEContextManagement_Get

Description: The  NF consumer request the UDM to get the  NF ID of the NF serving of the UE.

Example Consumers: NEF.

Inputs, Required: UE ID, NF Type.

Inputs, Optional: None.

Outputs, Required: SUPI, NF ID of the NF corresponding to the NF type requested by NF consumer.

Outputs, Optional: None.

##### 5.2.3.2.5	Nudm_UEContextManagement_Update

Service operation name: Nudm_UEContextManagement_Update

Description: Consumer updates some UE related information (e.g. UE capabilities).

Example Consumers: AMF

Inputs, Required: NF ID, SUPI, NF type, UE context information

Inputs, Optional: None

Outputs, Required: Result Indication.

Outputs, Optional: None

#### 5.2.3.3	Nudm_SubscriberDataManagement Service

Editor's note:	Whether NF type can be derived from other information or NF type is needed explicitly is FFS.

##### 5.2.3.3.1	Nudm_SubscriberDataManagement_Get

Service Operation name: Nudm_SubscriberDataManagement_Get

Description: Consumer NF gets the subscriber data from UDM. The UDM checks the UE ID and NF type to retrieve corresponding subscriber data in the UDR and provides the data to the NF consumer. In case of NF consumer is SMF, the subscriber data may contain e.g., PDU type(s), authorized SSC mode(s), default 5QI/ARP

Example Consumers: SMF, SMSF, AMF, PCF.

Inputs, Required: SUPI, NF ID, Requested subscription data type (e.g. mobility subscription, session management subscription, unstructured repository data).

Inputs, Optional: DNN if the NF type is SMF.

Outputs, Required: The consumer NF gets the requested subscription data. This may either be structured or unstructured repository data depending on the requested data type.

Outputs, Optional: None.

The UDM shall provide the requested subscription data only if the consumer NF is authorized to get the specific subscription data requested.

##### 5.2.3.3.2	Nudm_SubscriberDataManagement_Notification

Service operation name: Nudm_SubscriberDataManagement_Notification

Description: The UDM notifies NF consumer of the updates of UE's Subscriber Data.

NOTE:	This notification corresponds to an implicit or explicit subscription.

Example Consumers: AMF, SMF, SMSF, PCF.

Inputs, Required: SUPI, Updated subscription Data (e.g. mobility subscription, session management subscription, repository data).

Inputs, Optional: None.

Outputs, Required: None.

The UDM invokes this service operation under the following cases:

-	When the subscriber data is updated at the UDM, the updated subscription information is notified to the serving NF that has subscribed for the specific subscription data type to be notified.

-	When the serving NF for the UE has included the "subscription data retrieval indication" along with the type of subscription data to be retrieved, the complete subscription data information of the requested type is notified to the serving NF.

In case updates subscription information is related to session management the subscriber data may contain e.g., PDU type(s), authorized SSC mode(s), default 5QI/ARP.

Outputs, Optional: None.

##### 5.2.3.3.3	Nudm_SubscriberDataManagement_Subscribe

Service operation name: Nudm_SubscriberDataManagement_Subscribe

Description: The NF consumer subscribes for updates to UE's Subscriber Data.

Example Consumers: AMF, SMF, SMSF, PCF.

Inputs, Required: SUPI, Updated subscription Data (e.g. mobility subscription, session management subscription, repository data).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.3.3.4	Nudm_SubscriberDataManagement_Unsubscribe

Service operation name: Nudm_SubscriberDataManagement_Unsubscribe

Description: The NF consumer unsubscribes from updates to UE's Subscriber Data.

Example Consumers: AMF, SMF, SMSF, PCF.

Inputs, Required: SUPI, Updated subscription Data (e.g. mobility subscription, session management subscription, repository data).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.3.3.4	Nudm_SubscriberDataManagement_Update

Service operation name: Nudm_SubscriberDataManagement_Update

Description: The NF consumer requests updates to UE’s subscription data in the UDM (e.g. update communication pattern based on application SP input) for identified UE(s).

Example Consumers: NEF, PCF.

Inputs, Required: PUI, External Group Identity or SUPI, Updated subscription Data (e.g. communication pattern parameters as in table 5.4.x-1, repository data).

Inputs, Optional: None.

Outputs, Required: Result indication

Outputs, Optional: None.

#### 5.2.3.4	Nudm_UEAuthentication Service

Editor's note:	Whether NF type can be derived from other information or NF type is needed explicitly is FFS.

##### 5.2.3.4.1	Nudm_UEAuthentication_Get

Service operation name: Nudm_UEAuthentication_Get

Description: Requester NF gets the authentication data from UDM. For AKA based authentication, this operation can be also used to recover from synchronization failure situations.

Known NF Consumers: AUSF

Inputs, Required: SUPI.

Inputs, Optional: Synchronization Failure indication and related information (i.e. RAND/AUTS).

Outputs, Required: Retrieve Authentication data for a certain UE

Outputs, Optional: None

#### 5.2.3.5	Nudm_EventExposure service

##### 5.2.3.5.1	Service description

See clause 5.4.2.

##### 5.2.3.5.2	Service operations

###### 5.2.3.5.2.1	Nudm_EventExposure_Subscribe

Service operation name: Nudm_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the subscription is already defined in UDM, then the subscription is updated.

NF Consumers: NEF.

Inputs (required): SUPI, Event Id(s).

Inputs (optional): Reporting options (e.g. Maximum Number of Reports or Monitoring Duration, for Monitoring Events).

Outputs (required): Operation execution result indication.

Outputs (optional): First corresponding event report is included, if corresponding information is available at reception of the subscription request of the event (e.g. One Time reporting for Monitoring Events).

###### 5.2.3.5.2.2	Nudm_EventExposure_Unsubscribe

Service operation name: Nudm_EventExposure_Unsubscribe

Description: the consumer deletes the subscription of an event if already defined in UDM.

NF Consumers: NEF.

Inputs (required): SUPI, Event Id(s).

Outputs (required): Operation execution result indication.

###### 5.2.3.5.2.3	Nudm_EventExposure_Notify

Service operation name: Nudm_EventExposure_Notify

Description: UDM reports the event to the consumer that has previously subscribed.

NF Consumers: NEF.

Inputs (required): Monitoring Indication.

Inputs (optional): None.

Outputs (required): None.