SA WG2 Meeting #123	S2-177341

October 23 – 27, 2017, Ljubljana, Slovenia

Source:	OPPO

Title:	Add NAS Signalling Connection Release Cause due to RAN Paging Failure

Document for:	Approval

Agenda Item:	6.5.7.5

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution involves adding NAS signalling connection release cause due to RAN paging failure.

- Discussion

In the last SA2 meeting, it has been agreed that when RAN paging fails, RAN may initiate the NAS signalling connection release procedure as descripted in [1].

“If the RAN paging procedure, as defined in TS 38.300 [27], is not successful in establishing contact with the UE the procedure shall be handled by the network as follows:

-	If NG-RAN has a pending NAS PDU for transmission, the RAN node shall initiate the NAS signaling connection release procedure (see TS 23.502 [3]) to move the UE CM state in the AMF to CM-IDLE state and indicate to the AMF the NAS non-delivery.

-	If NG RAN has only pending user plane data for transmission, the NG-RAN node may keep the N2 connection active or initiate the NAS signaling connection release procedure (see TS 23.502 [3]) based on local configuration in NG-RAN.”

In this contribution, the corresponding change is added in TS 23.501 specification.

2	Proposal

It is proposed to agree on the proposed changes to TS 23.502.

*************** Start of changes *********************

### 4.2.6	NAS signalling connection release

This procedure is used to release the logical NG-AP signalling connection and the associated N3 User Plane connections, and (R)AN RRC signalling and resources.

When the NG-AP signalling connection is lost due to (R)AN or AMF failure, the NAS signalling connection release is performed locally by the AMF or the (R)AN as described in the procedure flow below without using or relying on any of the signalling shown between (R)AN and AMF. The NAS signalling connection release causes all UP connections of the UE to be deactivated.

The initiation of NAS signalling connection release may be due to:

-	(R)AN-initiated with cause e.g. O&M Intervention, Unspecified Failure, (R)AN (e.g. Radio) Link Failure, User Inactivity, Release due to UE generated signalling connection release, mobility restriction, RAN paging failure in CM-CONNECTED with Inactive state etc.; or

-	AMF-initiated with cause e.g. Unspecified Failure, etc.

Both (R)AN-initiated and AMF-initiated NAS signalling connection release procedures are shown in Figure 4.2.6-1.

For this procedure, the impacted SMF and UPF are all under control of the PLMN serving the UE, e.g. in Home Routed roaming case the SMF and UPF in HPLMN are not involved.

Figure 4.2.6-1: NAS signalling connection release procedure

1.	If there is some confirmed (R)AN conditions (e.g. Radio Link Failure) or for other (R)AN internal reason, the (R)AN may decide to initiate the UE context release in the (R)AN. In this case, the (R)AN sends an N2 UE Context Release Request (Cause, List of PDU Session ID(s) with active N3 user plane) message to the AMF. Cause indicates the reason for the release (e.g. AN Link Failure, O&M intervention, unspecified failure, RAN paging failure in CM-CONNECTED with Inactive state etc.). The List of PDU Session ID(s) indicates the PDU Sessions served by (R)AN of the UE. This step is described in TS 38.413 [10], clause 8.3.2 "UE Context Release Request (gNB initiated)".

2.	AMF to (R)AN: If the AMF receives the N2 UE Context Release Request message or due to an internal AMF event, the AMF sends an N2 UE Context Release Command (Cause) to the (R)AN. The Cause indicates either the Cause from (R)AN in step 1 or the Cause due to an AMF internal event. In case the (R)AN is a 3GPP RAN this step is described in detail in TS 38.413 [10], clause 8.3.3 "UE Context Release (AMF initiated)". In case the (R)AN is an N3IWF this step is described in clause 4.12.

3.	[Conditional] If the (R)AN connection (e.g. RRC connection or NWu connection) with the UE is not already released (step 1), the (R)AN requests the UE to release the (R)AN connection. Upon receiving (R)AN connection release confirmation from the UE, the (R)AN deletes the UE's context.

4.	The (R)AN confirms the N2 Release by returning an N2 UE Context Release Complete (List of PDU Session ID(s) with active N3 user plane) message to the AMF. The List of PDU Session ID(s) indicates the PDU Sessions served by (R)AN of the UE. The N2 signalling connection between the AMF and the (R)AN for that UE is released. The (R)AN provides the list of recommended cells / TAs / NG-RAN node identifiers for paging to the AMF.

This step shall be performed promptly after step 2, i.e. it shall not be delayed, for example, in situations where the UE does not acknowledge the RRC Connection Release.

5.	[Conditional] AMF to SMF: For each of the PDU Sessions in the N2 UE Context Release Complete, the AMF invokes Nsmf_PDUSession_UpdateSMContext(PDU Session ID, PDU Session Deactivation, Cause). The Cause in step 5 is the same Cause in step 2. If List of PDU Session ID(s) with active N3 user plane is included in step 1b, the step 5 to 7 are performed before step 2.

For each of the PDU Sessions whose UP connection have been deactivated because the UE is outside of the availability of the LADN, the AMF reports the UE location information to the SMFs that have subscribed for the UE location reporting.

6a	[Conditional] SMF to UPF: N4 Session Modification Request (AN tunnel info to be removed, Buffering on/off).

The SMF initiates an N4 Session Modification procedure indicating the need to remove AN Tunnel Info.  Buffering on/off indicates whether the UPF shall buffer incoming DL PDU or not. For the PDU Sessions corresponding to the LADN, the SMF may enable the Downlink Data Notification in case that previously the SMF notified the UPF to discard downlink data for the PDU Sessions and/or to not provide further Data Notification message.

See clause 4.4 for more details.

The treatment of GBR QoS flows follow TS 23.501 [2], clause 5.7 "QoS model".

6b.	[Conditional] UPF to SMF: N4 Session Modification Response acknowledging the SMF request.

See clause 4.4 for more details.

7. [Conditional] SMF to AMF: Nsmf_PDUSession_UpdateSMContext Ack for step 5. AMF considers the N2 and N3 as released. When the AMF receives all acknowledgments from the SMF(s), the AMF enters CM-IDLE state.

*************** End of changes *********************