SA WG2 Meeting #124	S2-178320

November 27 – December 1, 2017, Reno, USA

Source:	Cisco, Charter Communications, Comcast, CableLabs, NTT DOCOMO, Nokia, Nokia Shanghai Bell

Title:	23.502: 5G to 4G mobility for SR UE without N26 interface

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This pCR simplifies the interworking for SR mode UEs w/o N26 interface (see incoming LS from CT1 (S2-178239)

# 1	Discussion

In their LS CT1 has asked why two different options exist for SR UE when it moves from 5GC to EPC when it has already received indication that "dual registration mode" is supported, i.e. the network supports no-N26 procedures, i.e. the network does not forward UE context using the N26 interface.

Having the single option of UE performing handover attach enables IP address preservation also for the UE. Also, UE performing TAU in such situations will lead to failure of the TAU and UE subsequently performing Attach. Since the UE in 5GS has already been provided indication that N256 is not supported, the UE should just perform handover attach. One procedure, also simpler for specifications.

**** FIRST CHANGE ****

### 4.11.2	Interworking procedures without N26 interface

#### 4.11.2.1	General

#### 4.11.2.2	5GS to EPS Mobility

The following procedure is used by UEs in single-registration or dual registration mode on mobility from EPS to 5GS.

Figure 4.11.2.2-1. Mobility procedure from 5GS to EPS without N26 interface

NOTE 1:	The network has indicated the "Dual Registration support" to the UE during the initial 5GC registration, which indicates the network does not support interworking procedures with N26 interface.

0.	UE is registered in 5GS.

2.	Step 1 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401 [13] with the following modification:

The UE indicates that it is moving from 5GC, and provides either 4G-GUTI mapped from 5G-GUTI (for single-registration mode), or native 4G-GUTI (dual-registration mode), or if the UE sent a TAU in Step 2 and it was rejected because the MME could not derive the UE identity, the UE provides IMSI.

If the UE wants to transfer a PDU Session to EPC as part of the Attach procedure, it includes a PDN CONNECTIVITY Request message in the Attach Request and provides a Request type "Handover", DNN/APN of the PDU Session (TS 23.401 [13], clause 5.3.2.1).

3.	Step 2 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401 [13].

4.	Steps 4-7 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401 [13], with the following modifications:

If the UE provided a 4G-GUTI mapped from 5G-GUTI and the MME is configured to support 5GS-EPS interworking without N26 procedure, the MME does not perform Step 3, Identity request to old MME/SGSN/AMF in clause 5.3.2.1 in TS 23.401 [13].

NOTE 2:	As the 4G-GUTI mapped from 5G-GUTI is unknown identity to the MME, the MME sends an Identity Request to the UE to request the IMSI. The UE responds with Identity Response (IMSI).

5.	Step 8 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401 [13], with the following modifications:

If the MME determined that the old node is an AMF based on the indication from the UE and the MME is configured to support 5GS-EPS interworking without N26 procedure, the MME does not include the "initial attach" indication to the HSS+UDM. The HSS+UDM does not send cancel location to the old AMF.

NOTE 3:	If the UE does not maintain registration in 5GC, upon reachability time-out, the AMF can implicitly detach the UE and release the possible remaining PDU Sessions in 5GC.

6.	Step 11 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401 [13], with the following modifications:

The subscription profile the MME receives from HSS+UDM includes the DNN/APN and PGW-C+SMF ID for each PDU Session established in 5GC.

7.	Steps 12-24 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401 [13], with the following modifications:

The MME determines the PGW-C+SMF address for the Create Session Request based on the APN received from the UE and the subscription profile received from the HSS+UDM in Step 9.

Editor's note: How to correlate the sessions between source and target systems when the UE has more than one session for the APN/DNN is FFS.

As a result of the procedure the PGW-U+UPF starts routing DL data packets to the Serving GW for the default and any dedicated EPS bearers established for this PDN connection.

8.	Step 25 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401 [13].

9.	Step 26 as in clause 5.3.2.1 (E-UTRAN Initial Attach) in TS 23.401 [13].

10.	If the UE has remaining PDU Sessions in 5GS which it wants to transfer to EPS and maintain the same IP address/prefix, the UE performs the UE requested PDN Connectivity Procedure as specified in TS 23.401 [13] clause 5.10.2 and sets the Request Type to "handover" in Step 1 of the procedure. UE provides an APN corresponding to the PDU Session it wants to transfer to EPS.

UEs in single-registration mode performs this step for each PDU Session immediately after completing the E-UTRAN Initial Attach procedure. UEs in dual-registration mode may perform this step any time after the completing of E-UTRAN Initial Attach procedure. Also, UEs in dual-registration mode may perform this step only for a subset of PDU Sessions.

The MME determines the PGW-C+SMF address for the Create Session Request based on the APN received from the UE and the subscription profile received from the HSS+UDM in Step 9 or when the HSS+UDM notifies the MME for the new PGW-C+SMF ID in the updated subscription profile.

As a result of the procedure the PGW-U+UPF starts routing DL data packets to the Serving GW for the default and any dedicated EPS bearers established for this PDN connection.

**** END OF CHANGES ****