SA WG2 Meeting #124	S2-178855

November 27 – December 1, 2017, Reno, NV, USA	(was S2-17xxxx)

Source:	ITRI

Title:	Update to 23.501: Removal of FFS in Section 6.3.5 AMF selection related to N3IWF supporting network slicing

Document for:	Approval

Agenda Item:	6.5.10

Work Item / Release:	5GS_Ph1/ Rel15

Abstract of the contribution: This contribution proposes to remove Editor’s note in Section 6.3.5 in TS 23.501.

1	Discussion

This contribution aims to remove the following Editor’s note in Section 6.3.5 of TS 23.501:

- Whether the N3IWF also considers N3IWF support for specific slices is FFS

In this section, it describes the AMF selection functionality in 5G-AN including 3GPP and non 3GPP AN may consider Requested NSSAI and Local operator policies for supporting the AMF selection of network slicing. In 3GPP, NG-RAN select the AMF according to the Requested NSSAI and shall route the NAS signaling between this UE and the selected AMF. In addition, if NG-RAN is unable to select an AMF based on the Requested NSSAI, it routes the NAS signaling to an AMF from a set of default AMFs. N3IWF can be applied the same AMF selection in which N3IWF can select AMF according to UE provided Requested NSSAI or according to local operator policies. Therefore, this Editor’s note can be deleted without adding text.

2	Proposal

It is proposed to adopt the following content in TS 23.501.

* * * * (First Change) * * * *

6.3.5	AMF selection

The AMF selection functionality is applicable to both 3GPP access and non-3GPP access. The AMF selection functionality can be supported by the 5G-AN (e.g. RAN, N3IWF) and is used to select an AMF for a given UE. An AMF supports the AMF selection functionality to select an AMF for relocation or because the initially selected AMF was not an appropriate AMF to serve the UE (e.g. due to change of Allowed NSSAI). Other CP NF(s), e.g. SMF, supports the AMF selection functionality to select an AMF from the AMF set when the original AMF serving a UE is unavailable.

5G-AN selects an AMF Set and an AMF from the AMF Set under the following circumstances:

1)	When the UE provides no 5G-S-TMSI nor the GUAMI to the 5G-AN.

2)	When the UE provides 5G-S-TMSI or GUAMI but the routing information (i.e. AMF identified based on AMF Set ID, AMF pointer) present in the 5G-S-TMSI or GUAMI is not sufficient and/or not usable (e.g. UE provides GUAMI with an AMF region ID from a different region).

3)	AMF has instructed AN that the AMF (identified by GUAMI(s)) is unavailable and no target AMF is identified and/or AN has detected that the AMF has failed.

Other CP NFs selects an AMF from the AMF Set under the following circumstances:

4)	When the AMF has instructed AN that a certain AMF identified by GUAMI(s) is unavailable and the CP NF was not notified of target AMF, and/or CP NF has detected that the AMF has failed.

The AMF selection functionality in the 5G-AN may consider the following factors for selecting the AMF Set:

-	Requested NSSAI.

-	Local operator policies.

Editor's note:	whether the N3IWF also considers N3IWF support for specific slices is FFS.

AMF selection functionality in the 5G-AN or CP NFs considers the following factors for selecting an AMF from AMF Set:

-	Availability of candidate AMFs.

-	Load balancing across candidate AMFs (e.g. considering weight factors of candidate AMFs in the AMF Set).

When 5G-S-TMSI or GUAMI is provided by the UE to the 5G-AN contains an AMF Set ID that is usable, and the AMF identified by AMF pointer that is not usable (e.g. AN detects that the AMF has failed) or the corresponding AMF indicates it is unavailable (e.g. out of operation) then the 5G-AN uses the AMF Set ID for selecting another AMF from the AMF set considering the factors above.

The AMF selection functionality in the AMF or other CP NFs shall utilize the NRF to discover the AMF instance(s) unless AMF information is available by other means, e.g. locally configured on AMF or other CP NFs. The NRF provides the IP address or the FQDN of AMF instance(s) to the AMF or other CP NFs. In the context of Network Slicing, the AMF selection is described in clause 5.15.5.2.1.

-	AMF selection functionality in AMF or other CP NFs use GUAMI to discover the AMF instance, the NRF provides the IP address, or the FQDN of an AMF instance, or a list of candidate AMF instances in the same AMF Set together with additional information (e.g. priority) if the indicated GUAMI cannot be found;

-	AMF selection functionality in AMF use AMF Set ID to discover the AMF instance(s), the NRF provides a list of AMF instances in the same AMF Set together with additional information (e.g. priority).

* * * * End of First Change * * * *