SA WG2 Meeting #124	S2-179181

November 27 – December 1, 2017, Reno, NV, USA	(was S2-178909)

Source:	Rogers Communications Canada, Ericsson, Nokia, T-Mobile USA?, Qualcomm?, Motorola, Broadcom?, …

Title:	23.501: Combined N3IWF-ePDG Selection

Document for:	Approval

Agenda Item:	6.5.10

Work Item / Release:	5GS_Ph1/ Rel15

Abstract of the contribution: This contribution defines a combined procedure for non-3GPP access selection with flexibility to address different deployment requirements.

# 1	Discussion

Operators may have various strategies for the 4G to 5G transition for non-3GPP access. The deployments where both ePDG and N3IWF supported are feasible. The proposed procedure provides flexibility needed for selecting a most appropriate access both in HPLMN and in roaming.

# 2	Proposal

The following changes are proposed for TS 23.501.

* * * * Start of First Change * * * *

### 6.3.6	N3IWF selection

#### 6.3.6.1	General

The N3IWF selection procedure is based on the ePDG selection mechanism specified in TS 23.402 [43], clause 4.5.4, enhanced to accommodate the N3IWF and to support selection in PLMNs where both non-3GPP accesses (i.e., ePDG and N3IWF) are available.

For this purpose, the following modifications are introduced:

- The N3IWF FQDN formats, similar to the corresponding ePDG FQDN formats defined in TS 23.003[19]:

  - N3IWF Operator Identifier based FQDN; and

  - N3IWF Tracking Area Identity based FQDN.

Editor's note: The reference to TS 23.003 might be revised if the definitions related to N3IWF are included in a different TS.

- The Non-3GPP Access (N3A) selection information, based on the ePDG selection information defined in TS 23.402[43], clause 4.5.4 extended to include the N3IWF parameters similar to the respective ePDG parameters, as well as two additional parameters per entry in the selection information list:

  - “Preference” parameter defining operator’s preference for a specific non-3GPP access when both accesses are supported; and

  - “Alignment” parameter defining whether the attachment over 3GPP access has to be taken into account for the selection.

- The N3IWF Identifier, similar in purpose and format to the ePDG Identifier defined in TS 23.402[43], clause 4.5.4.

The “Preference” and “Alignment” parameters described in point 2) above are also configured separately for HPLMN outside of the N3A selection information for use when the selection is based on the ePDG/N3IWF Identifiers, according to TS 23.402[43], clause 4.5.4.

#### 6.3.6.2	Combined N3IWF/ePDG Selection Procedure

The present procedure follows the logic specified in the ePDG selection procedure in TS 23.402, clause 4.5.4, to identify a candidate PLMN for non-3GPP access selection.

For identified candidate PLMN the selection performs as follows:

- The UE shall determine if the non-3GPP access node selection is for a “voice service” or for a “data service”. The means of that determination is implementation-specific (e.g. by the characteristics of DNN/APN associated with the requested service or by using the “voice centric” vs “data centric” designation, etc.)

- For the “voice service”, the UE shall choose a non-3GPP access node type (i.e., ePDG or N3IWF) taking into account the “Alignment” and “Preference” parameters specified in clause 6.3.6.1, as follows:

If the “Alignment” parameter is configured and applicable (i.e. the UE is attached to either one of 5GC and EPC, but not both) then

- the UE shall choose the access node type based on the core network type the UE is attached to over the 3GPP access (i.e., the UE attached to 5GC chooses N3IWF, and the UE attached to EPC chooses ePDG);

- Otherwise, the UE shall choose the access node type based on the “Preference” parameter, for that PLMN.

The resulting non-3GPP access node type choice is used for the selection.

If the selection fails, including the case when during the registration over non-3GPP access the UE receives indication that the IMS Voice over non-3GPP access is not supported (see clause 5.16.3.2a), the UE attempts selecting the other non-3GPP access node type defined for that PLMN, if any. If that selection fails or not possible, the UE proceeds to the next PLMN, if any, according to the procedure specified in TS 23.402[43], clause 4.5.4.

- For the “data service”, the UE shall perform the selection by giving preference to the N3IWF and ignoring the “Preference” and “Alignment” parameters settings. If N3IWF selection fails or is not possible, the UE should attempt selecting an ePDG in the same PLMN, if configured. If ePDG selection fails or is not possible the UE proceeds to the next PLMN, if any, according to the procedure specified in TS 23.402[43], clause 4.5.4.

NOTE:	A UE performing both selections for a “voice service” and for a “data service” could get simultaneously attached to non-3GPP access nodes of both type (i.e., to a N3IWF and an ePDG) in the same PLMN or in different PLMNs.

* * * * End of First Change * * * *

* * * Start of Second Change * * *

#### 5.16.3.2	IMS voice over PS Session Supported Indication over 3GPP access

The serving PLMN AMF shall send an indication toward the UE during the Registration procedure over 3GPP access to indicate if an IMS voice over PS session is supported or not supported over 3GPP access. A UE with "IMS voice over PS" voice capability over 3GPP access should take this indication into account when performing voice domain selection, as described in clause 5.16.3.5.

The serving PLMN AMF may only indicate IMS voice over PS session supported over 3GPP access in one of the following cases:

-	If the network is able to provide a successful IMS voice over PS session over NR connected to 5GC with a 5G QoS Flow that supports voice as specified in clause 5.7.

-	If the network is not able to provide a successful IMS voice over PS session over NR connected to 5GC, but is able for one of the following:

-	If E-UTRA connected to 5GC supports voice, and the NG-RAN is able to trigger handover to E-UTRA connected to 5GC at QoS Flow establishment for voice; or

-	If the UE supports HO to EPS, the EPS supports voice, and the NG-RAN is able to trigger handover to EPS at QoS Flow establishment for voice.

The serving PLMN provides this indication based e.g. on local policy, HPLMN and how extended NG-RAN coverage is. The AMF in serving PLMN shall indicate that IMS voice over PS is not supported if serving PLMN does not have an IMS roaming agreement with HPLMN. This indication is per Registration Area.

Editor's note: If interactions between the AMF and RAN to determine functionality equivalent to Voice Support Match Indicator of EPC are needed, they are FFS.

#### 5.16.3.2a	IMS voice over PS Session Supported Indication over non-3GPP access

The serving PLMN AMF shall send an indication toward the UE during the Registration procedure over non-3GPP access to indicate whether an IMS voice over PS session is supported or not supported via non-3GPP access. A UE with "IMS voice over PS" voice capability over non-3GPP access should take this indication into account when performing the selection between N3IWF and ePDG described in clause 6.3.6.

The serving PLMN AMF may only indicate IMS voice over PS session supported over non-3GPP access if the network is able to provide a successful IMS voice over PS session over N3IWF connected to 5GC with a 5G QoS Flow that supports voice as specified in clause 5.7.

* * * End of Second Change * * *