# SA WG2 Meeting #S2-125	S2-180018

# 22 -26 January, 2018, Gothenburg, Sweden	(revision of S2-178933)

SA WG2 Meeting #S2-124	S2-178933

27 November - 01 December, 2017, Reno, Nevada, USA

# Liaison Statement

| Liaison Statement Title: | LS Reply to 3GPP SA2 on Status Icon related to 5G |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| 5GI#12 | 08/11/2017 | Conference Call |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| 5GI#10 Doc 003 | 2017-08-28 | Dongwook Kim (GSMA) |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| Future Networks Programme |  | GSMALiaisons@gsma.com |
| Security Classification | Non-Confidential |  |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | GSMA Future Networks Programme, 5G Introduction |
| External Recipients: | To: 3GPP SA2, 3GPP RAN2 CC: 3GPP SA, 3GPP SA1, 3GPP RAN, 3GPP CT1 , 3GPP CT |

# Introduction

GSMA Future Networks Programme thanks 3GPP SA2 for the inquiry on requirements and granularity of Service indicators and/or RAT indicators related to 5G (S2-175303). GSMA Future Networks Programme is conducting a project aimed to analyse possible paths for operators in introducing and migrating from 3GPP 4G network to 5G network from technical, economic and strategic perspective. The inquiry is timely and appropriate for the discussion within 5G Introduction project. The operator members of the 5G Introduction project have discussed the request and the results are provided below. Operators agree that it would be beneficial to have consistency across device OEMs of the RAT indicated to the user.

# Operator Requirement

Survey and discussion among operator members of the 5G Introduction project indicates that four configurations should be supported for deciding when to present a 5G status icon.  The configurations are presented from the most restrictive to the most relaxed, their adoption is decided according to operator and regional regulatory requirement.

In the configurations, the UE is allowed to use NR (based on information sent from the HSS or locally generated by the VPLMN policy in the MME).

Considerations of core network also indicates that 5G status icon should be displayed in two cases. First, when UE is camping on SA E-UTRA cell connected to 5GC. Second, when UE is camping on NG-RAN cell connected to 5GC.

NOTE: The considerations of core network are in accordance with the definition of NG-RAN (SA NR connected to 5GC; SA E-UTRA connected to 5GC; NR is the master with E-UTRA extensions and E-UTRA is the master with NR extensions) in 3GPP TS 23.501 V1.4.0.

The configurations consist of the following states and are as described in Table 1:

EPS NR NSA (EN-DC) capable UE attached to EPC and currently in IDLE state under or in RRC_connected state connected to E-UTRAN cell not supporting LTE-NR dual connectivity

EPS NR NSA (EN-DC) capable UE attached to EPC and currently in IDLE state under or in RRC_Connected state connected to AND active on LTE for uplink and downlink on only E-UTRAN cell supporting LTE-NR dual connectivity and has not detected NR coverage (i.e. UE is not under NR coverage and/or not configured to make NR measurements)

EPS NR NSA (EN-DC) capable UE attached to EPC and currently in RRC_Connected state connected to E-UTRAN cell (supporting dual connectivity) and active on LTE for uplink and downlink only and has detected NR coverage (i.e. UE is under NR coverage and has been configured to make NR measurements)

EPS NR NSA (EN-DC) capable UE attached to EPC and currently in IDLE state under E-UTRAN cell supporting LTE-NR dual connectivity and has detected NR coverage (i.e. UE is under NR coverage and has been configured to make NR measurements)

EPS NR NSA (EN-DC) capable UE attached to EPC and currently in RRC_Connected state connected to E-UTRAN cell (supporting dual connectivity) and active on LTE and NR for uplink and/or downlink

5GS capable UE attached to 5GC and currently in IDLE state under or in RRC_Connected state connected to NG-RAN (eLTE (option 5 or 7) or NR (option 2 or 4) cell)

| State | Config. A | Config. B | Config. C | Config. D |
| --- | --- | --- | --- | --- |
| 1 (IDLE under or Connected to LTE cell not supporting NSA) | 4G | 4G | 4G | 4G |
| 2 (IDLE under or Connected to LTE cell supporting NSA and no detection of NR coverage) | 4G | 4G | 4G | 5G |
| 3 (Connected to LTE only under LTE cell supporting NSA and detection of NR coverage) | 4G | 4G | 5G | 5G |
| 4 (IDLE under LTE cell supporting NSA and detection of NR coverage) | 4G | 5G | 5G | 5G |
| 5 (Connected to LTE + NR under LTE cell supporting NSA) | 5G | 5G | 5G | 5G |
| 6 (IDLE under or connected to NG-RAN while attached to 5GC) | 5G | 5G | 5G | 5G |

. UE state/indicator configuration matrix

As there is no consensus on a single preferred configuration, it is desirable to make the display of 5G status icon in the UE configurable such that the display of 5G status icon can be made depending on operator preference.

# Actions

ACTION to SA2 and RAN2:

3GPP SA2 and RAN2 are asked to:

Take into consideration the above requirement regarding service indicators in the User Interface.

Verify that the UE has all the information needed to determine which state it is in and the associated complexity and impact for such determination (e.g battery consumption, signalling and so on)

# Next meetings

5GI_MNO #07 	20 December 2017

5GI_MNO #08 	January 2017