3GPP TSG-SA2 Meeting #125 	S2-180705

# Gothenburg, Sweden, 21-26 January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on Network Slicing aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Telecom Italia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-01-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Various changes are included to clarify and correct a number of subclauses of the clause 5.15 on Network slicing |  |  |  |  |  |  |  |  |
| Summary of change: |  | A Note intended to say no coexistence rules are supported in rel-15 is made clear. It is clarified the mapping of Allowed NSSAI to configrured NSSAI of HPLMN is provided in roaming case. It is clarified the configured NSSAI for a PLMN need not be configured only by HPLMN The clause on Registration to a set of network slices is fixed. Some editorials also in all affected clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | wrong implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.1; 5.15.4.1, 5.15.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 5.15	Network slicing

### 5.15.1	General

A Network Slice is defined within a PLMN and shall include:

-	the Core Network Control Plane and User Plane Network Functions, as described in clause 4.2,

and, in the serving PLMN, at least one of the following:

-	the NG Radio Access Network described in 3GPP TS 38.300 [27],

-	the N3IWF functions to the non-3GPP Access Network described in clause 4.2.7.2.

Network slicing support for roaming is described in clause 5.15.6.

Network slices may differ for supported features and network functions optimisations, in which case such Network Slices may have e.g. different S-NSSAIs with different Slice/Service Types (see sub-clause 5.15.2.1). The operator can deploy multiple Network Slice instances delivering exactly the same features but for different groups of UEs, e.g. as they deliver a different committed service and/or because they are dedicated to a customer, in which case such Network Slices may have e.g. different S-NSSAIs with the same Slice/Service Type but different Slice Differentiators (see sub-clause 5.15.2.1).

The network may serve a single UE with one or more Network Slice instances simultaneously via a 5G-AN and associated with at most eight different S-NSSAIs in total. The AMF instance serving the UE logically belongs to each of the Network Slice instances serving the UE, i.e. this AMF instance is common to the Network Slice instances serving a UE.

NOTE:	In this release of the specification there is no mechanism to restrict a UE subscribed to a set of Network Slices from simultaneously using only specific combinations of such subscribed Network Slices. .

The selection of the set of Network Slice instances for a UE is triggered by the first contacted AMF in a registration procedure normally by interacting with the NSSF, and can lead to a change of AMF. This is further described in clause 5.15.5.

A PDU Session belongs to one and only one specific Network Slice instance per PLMN. Different Network Slice instances do not share a PDU Session, though different slices may have slice-specific PDU Sessions using the same DNN.

During the handover procedure the source AMF selects a target AMF by interacting with the NRF as specified in clause 6.3.5.

NEXT change

#### 5.15.4.1	General

A UE can be configured by the HPLMN with slice configuration information.

The slice configuration information contains one or more Configured NSSAI(s). A Configured NSSAI may apply either to one PLMN or to all PLMNs that do not have a specific Configured NSSAI (e.g. this could be possible for NSSAIs containing only S-NSSAIs with standard values, see clause 5.15.2.1). There is at most one Configured NSSAI per PLMN.

The Configured NSSAI of a PLMN may include S-NSSAIs that have standard values or PLMN-specific values.

The Configured NSSAI for the Serving PLMN includes the S-NSSAI values which can be used in the Serving PLMN and may be associated with mapping of each S-NSSAI of the Configured NSSAI to the corresponding S-NSSAI values in the Configured NSSAI for the HPLMN.

The S-NSSAIs in the Configured NSSAI for the HPLMN, at the time when they are provided to the UE, match the Subscribed S-NSSAIs for the UE.

When providing a Requested NSSAI to the network upon registration, the UE in a given PLMN only includes and uses S-NSSAIs applying to this PLMN. The mapping of each S-NSSAI of the Requested NSSAI to the S-NSSAIs of the Configured NSSAI for the HPLMN may also be provided in roaming case. The S-NSSAIs in the Requested NSSAI are part of the Configured and/or Allowed NSSAIs applicable for this PLMN. Upon successful completion of a UE's Registration procedure, the UE obtains from the AMF an Allowed NSSAI, which includes one or more S-NSSAIs, possibly associated with their mapping to the S-NSSAIs in the  Configured NSSAI for the HPLMN. These S-NSSAIs are valid for the current Registration Area provided by the serving AMF the UE has registered with and can be used simultaneously by the UE (up to the maximum number of simultaneous Network Slices or PDU Sessions).

The UE might also obtain one or more rejected S-NSSAIs with cause and validity of rejection from the AMF. An S-NSSAI may be rejected:

-	for the PLMN; or

-	for the current Registration area.

While it remains RM-REGISTERED in the PLMN, the UE shall not re-attempt to register to an S-NSSAI rejected in the PLMN.

While it remains RM-REGISTERED in the PLMN, the UE shall not re-attempt to register to an S-NSSAI rejected in the current Registration Area until it moves out of the current Registration Area.

NOTE 1:	The details and more cases of S-NSSAI rejection are described in TS 24.501 [47].

The UE shall use only the S-NSSAI(s) in the Allowed NSSAI corresponding to a Network Slice for the subsequent procedures in the serving PLMN, as described in clause 5.15.5.

The UE stores (S)NSSAIs as follows:

-	When the UE is provisioned with a Configured NSSAI for a PLMN and optionally associated mapping of the Configured NSSAI to Configured NSSAI for the HPLMN, the Configured NSSAI and the mapping shall be stored in the UE until a new Configured NSSAI for this PLMN is provisioned in the UE:

-	When provisioned with a new Configured NSSAI for a PLMN and optionally associated mapping of the Configured NSSAI to Configured NSSAI for the HPLMN, the UE shall both replace any stored Configured NSSAI for this PLMN and the associated mapping with the new Configured NSSAI and the associated mapping, and delete any stored Allowed NSSAI and rejected S-NSSAI for this PLMN;

NOTE 2:	It is expected that the UE stores a received Configured NSSAI for a PLMN even when it is registered in other PLMNs. However, the number of Configured NSSAIs stored in the UE for PLMNs other than the HPLMN is up to UE implementation.

-	If received, the Allowed NSSAI for a PLMN and any associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN shall be stored in the UE. The UE should store the Allowed NSSAI also when the UE is turned off;

NOTE 3:	Whether the UE stores the Allowed NSSAI also when the UE is turned off is left to UE implementation.

-	When a new Allowed NSSAI for a PLMN is received, the UE shall replace any stored Allowed NSSAI for this PLMN with this new Allowed NSSAI and any associated mapping;

-	If received, a S-NSSAI permanently rejected in the PLMN shall be stored in the UE while RM-REGISTERED.

-	If received, a S-NSSAI rejected in the current Registration Area shall be stored in the UE while RM-REGISTERED until the UE moves out of the current Registration Area.

NOTE 4:	The storage aspects of rejected S-NSSAIs are described in TS 24.501 [47].

One or more S-NSSAIs in the Allowed NSSAI provided to the UE can have values, which are not part of the current UE's slice configuration information for the Serving PLMN. In this case, the Allowed NSSAI is associated with mapping of each S-NSSAI of the Allowed NSSAI to the S-NSSAI(s) of the Configured NSSAI for the HPLMN. This mapping allows the UE to associate applications to a S-NSSAI of the HPLMN as per NSSP of the URSP rules as defined in clause 6.6.2, TS 23.503 [45], and to the corresponding S-NSSAI from the Allowed NSSAI.

NEXT change

##### 5.15.5.2.1	Registration to a set of Network Slices

When a UE registers with a PLMN, if the UE for this PLMN has a Configured NSSAI or an Allowed NSSAI, the UE shall provide to the network in RRC and NAS layer a Requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE wishes to register, in addition to the 5G-S-TMSI if one was assigned to the UE.

The Requested NSSAI shall be one of:

-	the Configured-NSSAI, or a subset thereof as described below, if the UE has no Allowed NSSAI for the serving PLMN; or

-	the Allowed-NSSAI, or a subset thereof as described below, if the UE has an Allowed NSSAI for the serving PLMN; or

-	the Allowed-NSSAI, or a subset thereof as described below, plus one or more S-NSSAIs from the Configured-NSSAI for which no corresponding S-NSSAI is present in the Allowed NSSAI and that were not previously rejected in the PLMN by the network.

The subset of Configured-NSSAI provided in the Requested NSSAI consists of one or more S-NSSAI(s) in the Configured NSSAI applicable to this PLMN, not previously rejected in the PLMN by the network.

The UE shall include the Requested NSSAI at RRC Connection Establishment and in NAS messages. However, the UE shall not indicate any NSSAI in RRC Connection Establishment or Initial NAS message unless it has a Configured NSSAI or Allowed NSSAI for the corresponding PLMN. The RAN shall route the NAS signalling between this UE and an AMF selected using the Requested NSSAI obtained during RRC Connection Establishment. If the RAN is unable to select an AMF based on the Requested NSSAI, it routes the NAS signalling to an AMF from a set of default AMFs. In the NAS signalling the UE provides the mapping of each S-NSSAI of the Requested NSSAI to the S-NSSAIs of the Configured NSSAI for the HPLMN.

When a UE registers with a PLMN, if for this PLMN the UE has not included a Requested NSSAI, the RAN shall route all NAS signalling from/to this UE to/from a default AMF. When receiving from the UE a Requested NSSAI and a 5G-S-TMSI in RRC, if the RAN can reach an AMF corresponding to the 5G-S-TMSI, then RAN forwards the request to this AMF. Otherwise, the RAN selects a suitable AMF based on the Requested NSSAI provided by the UE and forwards the request to the selected AMF. If the RAN is not able to select an AMF based on the Requested NSSAI, then the request is sent to a default AMF.

When the AMF selected by the AN receives the UE Initial Registration request:

-	As part of the registration procedure described in TS 23.502 [3], clause 4.2.2.2.2, the AMF may query the UDM to retrieve UE subscription information including the Subscribed S-NSSAIs.

-	The AMF verifies whether the S-NSSAI(s) in the Requested NSSAI are permitted based on the Subscribed S-NSSAIs (to identify the Subscribed S-NSSAIs the AMF may use the mapping to the S-NSSAI(s) of the Configured NSSAI for the HPLMN provided by the UE, in the NAS message, for each S-NSSAI of the Requested NSSAI).

-	The AMF queries the NSSF (see (B) below for subsequent handling), except in the case when, based on configuration in this AMF, the AMF is allowed to determine whether it can serve the UE (see (A) below for subsequent handling).

NOTE 1:	The configuration in the AMF depends on operator's policy.

(A) When based on operator policy as described above, the AMF is allowed to determine whether it can serve the UE, and the following is performed:

-	AMF checks whether it can serve all the S-NSSAI(s) from the Requested NSSAI present in the Subscribed S-NSSAIs (potentially using configuration for mapping S-NSSAI values between HPLMN and Serving PLMN), or all the S-NSSAI(s) marked as default in the Subscribed S-NSSAIs in case no Requested NSSAI was provided (see clause 5.15.3).

-	If the AMF can sever the S-NSSAIs in the Requested NSSAI,  the AMF remains the serving AMF for the UE. The Allowed NSSAI is then composed of the list of S-NSSAI(s) in the Requested NSSAI permitted based on the Subscribed S-NSSAIs, or, if no Requested NSSAI was provided, all the S-NSSAI(s) marked as default in the Subscribed S-NSSAIs. It also determines the mapping if the S-NSSAI(s) included in the Allowed NSSAI needs to be mapped to Subscribed S-NSSAI(s) values. Then Step (C) is executed.

-	Else, the AMF queries the NSSF (see (B) below).

(B) When the AMF needs to query the NSSF, and the following is performed:

-	The AMF queries the NSSF, with Requested NSSAI, mapping of Requested NSSAI to Configured NSSAI for the HPLMN, the Subscribed S-NSSAIs (with an indication if marked as default S-NSSAI), PLMN ID of the SUPI and UE's current Tracking Area(s).

NOTE:	When more than one UE's Tracking Area is indicated, the UE is using more than one Access Type.

-	Based on this information, local configuration, and other locally available information including RAN capabilities in the current Tracking Area for the UE, the NSSF does the following:

-	It verifies whether the S-NSSAI(s) in the Requested NSSAI are permitted based on comparing the Subscribed S-NSSAIs and the mapping of Requested NSSAI to Configured NSSAI for the HPLMN.

-	It selects the Network Slice instance(s) to serve the UE. When multiple Network Slice instances in the UE's Tracking Areas are able to serve a given S-NSSAI, based on operator's configuration, the NSSF may select one of them to serve the UE, or the NSSF may defer the selection of the Network Slice instance until a NF/service within the Network Slice instance needs to be selected.

-	It determines the target AMF Set to be used to serve the UE, or, based on configuration, the list of candidate AMF(s), possibly after querying the NRF.

-	It determines the Allowed NSSAI, taking also into account the availability of the Network Slice instances as described in clause 5.15.8 that are able to serve the S-NSSAI(s) in the Allowed NSSAI in the current UE's Tracking Areas.

-	It also determines the mapping of each S-NSSAI of the Allowed NSSAI to the Subscribed S-SNSSAIs if necessary.

-	Based on operator configuration, the NSSF may determine the NRF(s) to be used to select NFs/services within the selected Network Slice instance(s).

-	Additional processing to determine the Allowed NSSAI in roaming scenarios and the mapping to the Subscribed S-NSSAIs, as described in clause 5.15.6.

-	The NSSF returns to the current AMF the Allowed NSSAI, the mapping if determined and the target AMF Set, or, based on configuration, the list of candidate AMF(s). The NSSF may return the NRF(s) to be used to select NFs/services within the selected Network Slice instance(s), and the NRF to be used to determine the list of candidate AMF(s) from the AMF Set. The NSSF may return NSI ID(s) to be associated to the Network Slice instance(s) corresponding to certain S-NSSAIs. NSSF may return the rejected S-NSSAI(s) as described in clause 5.15.4.1.

-	Depending on the available information and based on configuration, the AMF may query the appropriate NRF (e.g. locally pre-configured or provided by the NSSF) with the target AMF Set. The NRF returns a list of candidate AMFs.

-	If rerouting to a target serving AMF is necessary, the current AMF reroutes the Registration Request to a target serving AMF as described in clause 5.15.5.2.3.

-	Step (C) is executed

(C) The serving AMF shall determine a Registration Area such that all S-NSSAIs of the Allowed NSSAI are available in all Tracking Areas of the Registration Area (and also considering other aspects as described in clause  5.3.2.3) and then return to the UE the Allowed NSSAI and the mapping of the Allowed NSSAI to the Subscribed S-NSSAIs if provided. The AMF may return the rejected S-NSSAI(s) as described in clause 5.15.4.1.

Upon successful Registration, the UE is provided with a 5G-GUTI by the serving AMF. The UE shall include the related 5G-S-TMSI in any RRC Connection Establishment during subsequent initial accesses to enable the RAN to route the NAS signalling between the UE and the appropriate AMF.

If the UE receives an Allowed NSSAI from the serving AMF, it shall store this new Allowed NSSAI and the associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN, if any, and override any previously stored Allowed NSSAI for this PLMN, as described in clause 5.15.4.1.

When no Requested NSSAI was included or when an S-NSSAI was rejected in the PLMN, the AMF may update the UE slice configuration information for the PLMN as described in clause 5.15.4.2.

End of changes