3GPP TSG-SA WG2 Meeting #125	S2-180802

Gothenburg, Sweden; 22nd – 26th January 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 23.502 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the Inter-system mobility procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-01-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The Registration Request does not contain active flag setting, this should be the PDU Session(s) to be re-activated. As the AMF is not able to correlate the PDU Session IDs with the EBIs received from the MME in the EPS context, all PDU Session IDs received from the UE need to be provided to the SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 4.11.1.3.3, the active flag setting is changed to the “PDU Session(s) to be re-activated”. PDU Session(s) to be re-activated and EPS Bearer Identity of the default bearer are added as optional input parameters to the definition of UpdateSMContext service operation. - Minor other corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarify as the procerure description differs with the service operation definition |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.1.2.2.2, 4.11.1.3.3, 4.11.1.4.2, 4.11.2.1, 4.11.2.3, 5.2.8.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## *** Start of 1st change ***

4.11.1.2.2.2	Preparation phase

Figure 4.11.1.2.2.2-1 shows the preparation phase of the Single Registration-based Interworking from EPS to 5GS procedure.

Figure 4.11.1.2.2.2-1: EPS to 5GS handover using N26 interface, preparation phase

This procedure applies to the Non-Roaming (TS 23.501 [2] Figure 4.3.1-1), Home-routed roaming (TS 23.501 [2] Figure 4.3.2-1) and Local Breakout roaming Local Breakout (TS 23.501 [2] Figure 4.3.2-2) cases.

-	For non-roaming scenario, V-SMF, v-UPF and v-PCF+v-PCRF are not present

-	For home-routed roaming scenario, the SMF+PGW-C and UPF+PGW-U are in the HPLMN. v-PCF+v-PCRF are not present

-	For local breakout roaming scenario, V-SMF and v-UPF are not present. SMF+PGW-C and UPF+PGW-U are in the VPLMN.

In local-breakout roaming case, the v-PCF+v-PCRF forwards messages between the SMF+PGW-C and the h-PCF+h-PCRF.

1.	The source E-UTRAN decides that the UE should be handed over to the NG-RAN.

2.	The E-UTRAN sends a Handover Required (Target NG-RAN Node ID, Source to Target Transparent Container) message to the MME. Source E-UTRAN node proposes the need for data forwarding per E-RAB to the target NG-RAN using Source to Target Transparent Container.

3.	The MME selects the target AMF and sends a Forward Relocation Request (Target NG-RAN Node ID, Source to Target Transparent Container, EPS MM Context, EPS Bearer Context(s), Direct Forwarding Flag) message to the selected AMF.

The AMF converts the received EPS MM Context into the 5GS MM Context. This includes converting the EPS security context into a mapped 5G security context. The MME UE context includes IMSI, ME Identity, UE security context, UE Network Capability, and EPS Bearer context(s). An EPS Bearer context includes the SMF + PGW-C address and V-CN Tunnel Info at the UPF + PGW-U for uplink traffic, and APN.

NOTE:	If the AMF holds a native 5G security context for the UE, the AMF may activate this native 5G security context by initiating a NAS SMC upon completing the handover procedure.

The MME includes Direct Forwarding Flag to inform the target AMF of the applicability of indirect data forwarding.

Editor's note:	Whether 5GS Security Context can be derived from the EPS Security Context should be clarified by SA3 study.

4.	The AMF invokes the Nsmf_PDUSession_UpdateSMContext service operation (EPS Bearer Context(s), AMF ID) on the SMF identified by the PGW-C + SMF address received in step 3. The AMF ID is the UE's GUAMI which uniquely identifies the AMF serving the UE.

For home-routed roaming scenario, the AMF selects default V-SMFs per PDU Session.

The V-SMF selects the PGW-C+SMF using the H-SMF address as received from the AMF, and initates a Nsmf_PDUSession_Update service operation with the PGW-C+SMF.

Editor's note:	It is FFS whether and how to provide the SM context from MME to SMF via AMF during handover.

5.	If dynamic PCC is deployed, the SMF may initiate PDU Session Modification towards the h-PCF + h-PCRF to obtain the 5GS PCC Rules for the PDU Session. The h-PCF + h-PCRF does not apply the 5GS PCC Rules for the PDU Session.

6. The SMF+PGW-C modifies the PGW-U+UPF.

Editor's note:	The details of SMF+PGW-C and UPF interaction is FFS.

7.	The PGW-C+SMF (V-SMF in home-routed roaming scenario) sends a Nsmf_PDUSession_UpdateSMContext Response (PDU Session ID, N2 SM Information (PDU Session ID, QoS Rules, EPS Bearer Setup List, H-CN Tunnel-Info)) to the AMF. SMF includes mapping between QoS flows and EPS bearers as part of N2 SM Information container. NG-RAN can use the source to target transparent container and N2 SM Information container to determine which QoS flows have been proposed for forwarding and decide for which of those QoS flows it accepts the data forwarding or not. AMF stores an association of the PDU Session ID and the SMF ID.

In case the PDN Type of a PDN Connection in EPS is non-IP, and is locally associated in SMF to PDU Session Type Ethernet, the PDU Session Type in 5GS shall be set to Ethernet. In case the PDN type of a PDN Connection in EPS is non-IP, and is locally associated in UE and SMF to PDU Session Type Unstructured, the PDU Session Type in 5GS shall be set to Unstructured.

In case of PDU Session Type Ethernet, that was using PDN type non-IP in EPS, the SMF creates QoS rules based on the 5GS PCC Rules received from PCF.

8.	For home-routed roaming scenario only: The V-SMF selects a v-UPF and initiates an N4 Session Establishment procedure with the selected v-UPF. The V-SMF provides the v-UPF with packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session, including H-CN Tunnel Info. If CN Tunnel Info is allocated by the SMF, the V-CN Tunnel Info is provided to the v-UPF in this step.

The v-UPF acknowledges by sending an N4 Session Establishment Response message. If CN Tunnel Info is allocated by the UPF, the V-CN Tunnel info is provided to the V-SMF in this step.

9.	The AMF sends a Handover Request (Source to Target Transparent Container, N2 SM Information (PDU Session ID, QFI(s), QoS Profile(s), V-CN Tunnel Info), last used EPS PLMN ID, Handover Restriction List) message to the NG-RAN. If the last used EPS PLMN ID is not in the equivalent PLMNs list, the AMF adds the last used EPS PLMN ID into it and stores the last used EPS PLMN ID in UE's MM context.

10.	The NG-RAN sends a Handover Request Acknowledge (Target to Source Transparent Container, N2 SM response (PDU Session ID, list of accepted QFI(s) and AN Tunnel Info), N2 SM Information for PDU Forwarding (PDU Session ID, N3 Tunnel Info for PDU Forwarding)) message to the AMF. The NG-RAN includes one assigned TEID/TNL address per PDU Session (for which there is at least one QoS flow for which it has accepted the forwarding) within the SM Info container. It also includes the list of QoS flows for which it has accepted the forwarding. The NG-RAN stores the last used EPS PLMN ID in the UE context and sends to target NG-RAN during intra-NG-RAN handover procedure. The NG-RAN also uses the last used EPS PLMN ID to re-route the UE back to the last used EPS PLMN by selecting the suitable dedicated target frequency list (for idle mode mobility) or target cell (for connected mode mobility) according to operator's configuration.

11.	The AMF sends an Nsmf_PDUSession_UpdateSMContext Request (PDU Session ID, N2 SM response (list of accepted QFI(s) and AN Tunnel Info), (N2 SM Information for PDU Forwarding (PDU Session ID, N3 Tunnel Info for PDU Forwarding)) message to the SMF for updating N3 tunnel information. The SMF derives from this received list of QoS flows that should be mapped to the PDU Session and informs the UPF regarding this mapping.

12. SMF+PGW-C (V-SMF in home-routed roaming scenario) to AMF : Nsmf_PDUSession_UpdateSMContext Response (PDU Session ID, EPS Bearer Setup List).

This message is sent for each received Modify PDU Request message.

13.	SMF+PGW-C (V-SMF in home-routed roaming scenario) performs preparations for N2 Handover by indicating N3 UP address and Tunnel ID of NG-RAN to the UPF if N2 Handover is accepted by NG-RAN and by indicating the mapping between the TEID where the UPF receives data forwarded by the source eNB and the QFI + N3 Tunnel Info for PDU Forwarding where the UPF is to forward such data. If N2 Handover is not accepted by NG-RAN, SMF+PGW-C deallocates N3 UP address and Tunnel ID of the selected UPF.

The EPS Bearer Setup list is a list of EPS bearer Identifiers successfully handover to 5GC, which is generated based on the list of accepted QFI(s).

14.	The AMF sends the message Forward Relocation Response (Cause, Target to Source Transparent Container, Serving GW change indication, CN tunnel information for data forwarding, EPS Bearer Setup List, AMF Tunnel Endpoint Identifier for Control Plane, Addresses and TEIDs). The EPS Bearer Setup list is the combination of EPS Bearer Setup list from different SMF+PGW-C(s).

15.	If indirect data forwarding applies, the MME sends the Create Indirect Data Forwarding Tunnel Request/Response.

## *** Start of 2nd change ***

##### 4.11.1.3.3	EPS to 5GS Mobility Registration Procedure (Idle and Connected State) using N26 interface

Figure 4.11.1.3.3-1 describes the mobility procedure from EPS to 5GS when N26 is supported for idle and connected states.

Figure 4.11.1.3.3-1: EPS to 5GS mobility for single-registration mode with N26 interface

1.	The Registration procedure is triggered, e.g. the UE moves into 5G RAN coverage. Step 2 to 9 except step 5, 6 and 8 follow the Registration procedure in clause 4.2.2 with following enhancement.

2.	The UE sends Registration Request additionally including a TAU request message integrity protected using the EPS security context (for further security verification by the MME). The UE also includes a 5G-GUTI mapped from EPS GUTI as the old GUTI. The Registration type is set to "Mobility Registration Update" and the UE indicates in the Registration Request message that the UE is moving from EPC. For idle mode mobility, if the UE holds a native 5G-GUTI then the UE includes the GUAMI part of the native 5G-GUTI in RRC to enable the RAN to route the Registration Request to the same AMF (if available). The UE also includes the native 5G-GUTI as an additional GUTI in the Registration Request. The additional GUTI enables the AMF to retrieve the UE's MM context from the old AMF (if available). The UE includes at least the S-NSSAIs associated with the established PDN connections in the Requested NSSAI in RRC and NAS. The UE also includes the list of PDU Session IDs and related S-NSSAIs in the Registration request.

The UE integrity protects the Registration Request message using a 5G security context (if available).

3-4.	Steps 2-3 of clause 4.2.2.2.2 are performed.

Step 5, 6 and 8 are not performed when this procedure is part of EPS to 5GS handover.

5.	The target AMF derives the MME address and 4G GUTI from the old 5G-GUTI and sends Context Request to MME including EPS GUTI mapped from 5G-GUTI and the TAU request message according to TS 23.401 [13].

6.	The MME verifies the integrity protection of the TAU message and returns the UE context to the AMF according to TS 23.401 [13]. If the verification of the integrity protection fails, the MME returns an appropriate error cause. The AMF converts the received EPS MM Context into the 5GS MM Context. The UE context includes IMSI, ME Identity, UE security context, UE Network Capability, and EPS Bearer context(s). An EPS Bearer context includes the SMF + PGW-C address and APN.

7.	Steps 8-9 of clause 4.2.2.2.2 are optionally performed.

8.	The target AMF sends Context Acknowledge (Serving GW change indication) to MME according to TS 23.401 [13].

9.	Steps 11-12 of clause 4.2.2.2.2 are optionally performed.

Step 10, 12, 13 are performed only if the target AMF is different from the old AMF and the old AMF is in the same PLMN as the target AMF.

10.	If the UE' additional 5G-GUTI was included in the Registration Request, the target AMF shall invoke the Namf_Communication_UEContextTransfer to the old AMF identified by the additional 5G-GUTI, to request the UE's SUPI and MM Context. The old AMF associated to the additional 5G-GUTI also transfers the event subscriptions information by each consumer NF and the list of SM PDU Session ID / associated SMF ID, for the UE, to the target AMF.

11.	Steps 13-14c of clause 4.2.2.2.2 are performed, with the following difference: if MM context retrieved from the old AMF indicates non-3GPP access, then the target AMF indicates to the UDM that the target AMF identity to be registered in the UDM applies to both 3GPP and non-3GPP accesses.

12. The UDM invokes Nudm_UECM_DeregistrationNotification (see clause 5.2.3.2.2) to the old AMF.

13.	The old AMF unsubscribes with the UDM for subscription data using Nudm_SDM_Unsubscribe.

14.	Steps 16-20 of clause 4.2.2.2.2 are optionally performed (initiated by target AMF) with the following addition:

In the home-routed roaming case, the AMF selects V-SMFs based on the S-NSSAIs received from the UE.

AMF sends Nsmf_PDU_Session_UpdateSMContext (EPS Bearer Context(s) corresponding to PDU Connection) to the PGW-C+SMF and indicates  all the PDU Session(s) to be re-activated as received in the Registration request message. This step is performed for each PDU Connection and the corresponding PGW-C+SMF address/ID in the UE context the AMF received in Step 6. The AMF omits the PDU Session ID in the request.

The PGW-C+SMF finds the corresponding PDU Session based on the EPS Bearer Context(s).in the request. The PGW-C+SMF activates User Plane connections if the corresponding PDU Session is in the received PDU Session(s) to be re-activated. The PGW-C+SMF allocates its local resources and sends session modification request message to UPF(s). UPF(s) responds to the PGW-C+SMF(s) with session modification response. The PGW-C+SMF(s) updates its SM contexts and returns a Nsmf_PDU_Session_UpdateSMContextResponse message including N2 SM Context to the AMF depending on the need to establish User plane connection. The SMF responds with the PDU Session ID corresponding to the default EPS bearer ID in the request, and with the allocated EBI(s) information. The AMF stores an association of the PDU Session ID and the SMF ID, AMF stores the allocated EBI(s) associated to the PDU Session ID.

In case the PDN Type of a PDN Connection in EPS is non-IP, and it was originally established as Ethernet PDU Session when UE was camping in 5GS (known based on local context information that was set to PDU Session Type Ethernet in UE and SMF), the PDU Session Type in 5GS shall be set to Ethernet by the SMF and UE. In case the PDN type of a PDN Connection in EPS is non-IP, and is locally associated in UE and SMF to PDU Session Type Unstructured, the PDU Session Type in 5GS shall be set to Unstructured by the SMF and UE.

15.	After the HSS/UDM receives the Registration from AMF, it performs Cancel Location towards MME.

16.	The MME sends Cancel Location Ack to HSS/UDM.

Step 17 and 18 follow the steps 21 and 22 of Registration procedure in clause 4.2.2.2.2. The Registration Accept message shall include the updated 5G-GUTI that the UE will use to access 5GC via 3GPP or non-3GPP.

If a native 5G security for 3GPP access is available in the AMF (or has been retrieved from the old AMF), the AMF may continue to use this security context. Otherwise, the AMF shall either create the mapped security context based on the EPS security context obtained from the MME or initiate an authentication of the UE.

## *** Start of 3rd change ***

#### 4.11.2.3	EPS to 5GS Mobility

The following procedure is used by UEs in single-registration mode on idle mode mobility from 5GS to EPS.

This procedure is also used by UEs in dual-registration mode to perform registration in 5GS when the UE is also registered in EPC. The procedure is the General Registration procedure as captured in clause 4.2.2. Difference from that procedure are captured below.

The UE has one or more ongoing PDN connections including one or more EPS bearers. During the PDN connection establishment, the UE allocates the PDU Session ID and sends it to the PGW-C+SMF via PCO, as described in subclause 4.11.1.1.

Figure 4.11.2.3-1: Mobility procedure from EPS to 5GS without N26 interface

0.	The UE is attached in EPC.

1.	Step 1 in clause 4.2.2.2.2 (General Registration) with the following clarifications:

The UE indicates that it is moving from EPC. The UE in single registration mode provides the Registration type set to "mobility registration update", and a 5G-GUTI mapped from the 4G-GUTI (see clause 5.17.2.2: 5G-GUTI mapped from 4G-GUTI). The UE in dual registration mode provides the Registration type set to "initial registration", and a native 5G-GUTI or SUPI.

2.	Step 2 as in clause 4.2.2.2.

3.	Step 3 as in clause 4.2.2.2.2 (General Registration), with the following modifications:

If the Registration type is "mobility registration update" and the UE indicates that it is moving from EPC in Step 1, and the AMF is configured to support 5GS-EPS interworking procedure without N26 interface, the AMF treats this registration request as "initial Registration", and the AMF skips the PDU Session status synchronization.

NOTE 1:	The UE operating in single registration mode includes the PDU Session IDs corresponding to the PDN connections to the PDU Session status.

If the UE has provided a 5G-GUTI mapped from 4G-GUTI in Step 1 and the AMF is configured to support 5GS-EPS interworking procedure without N26 interface, the AMF does not perform Steps 4 and 5 in clause 4.2.2.2 (UE context transfer from the MME).

NOTE 2:	As the 5G-GUTI mapped from 4G-GUTI is unknown identity to the AMF, the AMF sends an Identity Request to the UE to request the SUPI. The UE responds with Identity Response (SUPI).

4.	Steps 6-13 as in clause 4.2.2.2.2 (General Registration).

5.	Step 14 as in clause 4.2.2.2.2 (General Registration), with the following modifications:

If the UE indicates that it is moving from EPC in Step 1 and AMF is configured to support 5GS-EPS interworking without N26 procedure, the AMF does not include the "initial Registration" indication to the UDM. The UDM does not send cancel location to the old MME.

NOTE 3:	If the UE does not maintain registration in EPC, upon reachability time-out, the MME can implicitly detach the UE and release the possible remaining PDN connections in EPC.

The subscription profile the AMF receives from HSS+UDM includes the DNN/APN and PGW-C+SMF ID for each PDN connection established in EPC.

6.	Steps 15-20 as in clause 4.2.2.2.2 (General Registration).

7.	Step 21 as in clause 4.2.2.2.2 (General Registration) with the following modifications:

The AMF includes a " Interworking without N26" indicator to the UE.

If the UE had provided PDU Session Status information in Step 1, the AMF sets the PDU Session Status to not synchronized.

8.	Step 22 as in clause 4.2.2.2.2 (General Registration)

9.	UE requested PDU Session Establishment procedure as in clause 4.3.2.2.1.

If the UE had setup PDN Connections in EPC which it wants to transfer to 5GS and maintain the same IP address/prefix, the UE performs the UE requested PDU Session Establishment Procedure as in clause 4.3.2.2 and sets the Request Type to "Existing PDU Session" or "Existing Emergency PDU Session" in Step 1 of the procedure. The UE provides a DNN and the PDU Session ID corresponding to the existing PDN connection it wants to transfer from EPS to 5GS.

UEs in single-registration mode performs this step for each PDN connection immediately after the Step 8. UEs in dual-registration mode may perform this step any time after Step 8. Also, UEs in dual-registration mode may perform this step only for a subset of PDU Sessions. The AMF selects the PGW-C+SMF for the Namf_PDUSession_CreateSMContext service based on the PDU Session ID received from the UE and the PGW-C+SMF ID in the subscription profile received from the HSS+UDM in Step 5 or when the HSS+UDM notifies the AMF for the new PGW-C+SMF ID in the updated subscription profile. AMF includes the PDU Session ID to the request sent to the PGW-C+SMF.

The PGW-C+SMF uses the PDU Session ID to determine the correct PDU Session.

As a result of the procedure, user plane is switched from EPS to 5GS.

10.	The PGW-C+SMF performs release of the resources in EPC for the PDN connections(s) transferred to 5GS by performing the PDN GW initiated bearer deactivation procedure as defined in clause 5.4.4.1 of TS 23.401 [13], except the steps 4-7.

## *** Start of 4th change ***

##### 5.2.8.2.6	Nsmf_PDUSession_Update SM Context service operation

Service operation name: Nsmf_PDUSession_Update SM Context.

Description: It allows to update the AMF-SMF association to support a PDU Session and/or to provide SMF with N1/N2 SM information received from the UE or from the AN.

Input, Required: SUPI, PDU Session ID.

Input, Optional: N1 SM container received from the UE, N2 SM information received from the AN (e.g. N3 addressing information, notification indicating that the QoS targets cannot be fulfilled for a QFI), Operation type (e.g. UP activate, UP deactivate, UP To Be Switched), Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding during HO from 5GS to EPS, UE location information, AN type, UE Time Zone, H-SMF identifier/address, EBI(s) to be revoked, PDU Session(s) to be re-activated, EPS Bearer Context(s) of the |PDN Connection.

Output, Required: Result Indication.

Output, Optional: PDU Session ID, Cause, released EBI list, allocated EBI information, N2 SM information (e.g. QFI, UE location information, notification indication indicating that the QoS targets cannot be fulfilled), N1 SM container to be transferred to the AN/UE.

See clause 4.3.3.2 and clause 4.3.3.3 for an example usage of this service operation.

See clause 4.9.1.2.2 for the usage of the "UP To Be Switched" Operation Type.

For the use of the "EBI(s) to be revoked" information, see clause 4.11.1.4.

If the consumer NF is AMF and the SMF determines that some EBIs are not needed, the SMF will put the EBIs back in the released EBI list.

If the consumer NF is AMF and inter-AMF mobility or Inter-system mobility happens, the SMF sends allocated EBI information to AMF.

If the NF consumer is AMF and AMF has PDU Session ID information, the PDU Session ID is mandatory for Input parameters. If the AMF does not have PDU Session ID, the PDU Session ID is not required for Input, and is required for Output.

## *** End of changes ***