SA WG2 Meeting #126 	S2-182409

Feb 26 – Mar 2, 2018, Montreal, Canada	(was S2-182297 was S2-181598)

Source:	CATT

Title:	Solution for privacy verification

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	FS_eLCS / Rel-16

Abstract of the contribution: This contribution proposes a solution for key issue 4.

Introduction

This P-CR proposes solution for Key Issue 4: reduce overhead for repetitive non-successful privacy verification.

In this solution, GMLC is responsible for privacy verification based on information from PPR and UE:

- PPR is a network function defined in 3GPP TS 23.271, it is proposed to reuse it to store the subscriber LCS Privacy Profile (SLPP).

- UE may also provide the privacy setting to the network when user changes the setting. This information is more dynamic.

GMLC performs privacy check based on the information above.

Proposal

It is proposed to add a solution for Key Issue 4.

* * * Start of Change * * * *

6.X	Solution #X: User Privacy Setting send via AMF

6.X.1	Introduction

This solution addresses Key Issue 4: Reduce overhead for repetitive non-successful privacy verification.

6.X.2	Functional Description

GMLC is responsible for privacy verification based on information from PPR and UE. The function of PPR and procedure between GMLC and PPR could be found in TS 23.271.

This solution mainly focuses on how UE provide privacy setting to GMLC. That is, once user has changed the Privacy Setting on UE, the UE updates network with such changes to network during Registration procedure. Such updating procedure involves AMF, PPR and GMLC. The PPR decides the UE privacy profile based on operator policy and UE provided Privacy Setting preference. The PPR notify the changed UE privacy profile to GMLC. When LCS request arrives, the GMLC performs the privacy verification.

6.X.3	Procedures

6.X.3.1	Privacy Setting Update

The UE may provide Privacy Setting to GMLC during Registration procedure, once the Privacy Setting has changed by user.

# Figure 6.X.3-1: Privacy Setting Update procedure

1.	UE sends Registration Request to AMF. The message includes the UE privacy setting preference, e.g. allowed service type.

2.	AMF sends Nppr_PVPreference Set to LMF including the UE privacy setting preference. PPR then may change the privacy setting based on operator policy.

3.	PPR sends Nppr_PVPreference_Set Response to AMF, which includes the PPR decided UE Privacy Setting.

4.	PPF sends Ngmlc_UserPrivacySetting_Set to GMLC. The message includes the UE ID and UE privacy setting profile.

5.	GMLC stores this UE privacy setting profile, and sends Ngmlc_UserPrivacySeting_Response to PPR..

6.	AMF sends Registration Accept to UE. The message includes the UE privacy setting accepted by network.

Editors’ Note: How this solution works in roaming scenario is FFS.

Note: Services provided by PPR and GMLC is FFS.

6.X.3.2	Privacy Verification

When GMLC receives LCS Service Request from External Client, it performs privacy verification based on the subscriber LCS Privacy Profile (SLPP) information from PPR first and then the User privacy setting profile received from the PPR. If any one of the privacy verification fails, GMLC shall terminate the request, and response to the External client with appropriate error code. Otherwise, the GMLC continue the location procedure.

6.X.4	Impacts on existing entities and interfaces

Impacted entities:

-	UE provides privacy setting to AMF.

-	AMF:

-	Receive privacy setting from UE

-	Send the UE provided privacy setting to PPR and receive updated setting from PPR.

-	PPR decides the privacy setting based on UE provided setting and operator policy, and provides it to AMF.

-	GMLC receives UE privacy setting profile from PPR and performs privacy check based on the setting.

Impacted interfaces:

-	N1, new interface between AMF and PPR, interface between PPR and GMLC are impacted.

6.X.5	Evaluation

Editor's Note: This clause provides an evaluation of the solution.

* * * End of Change * * * *