SA WG2 Meeting #126	S2-1825342

26 February -02 March, 2018, Montreal, Canada

Source:	Huawei, Hisilicon

Title:	Solution: UL-CL/BP Insertion

Document for:	Discussion and Approval

Agenda Item:	6.12

Work Item / Release:	FS_ETSUN / Rel-16

Abstract of the contribution: This contribution proposes a solution to insert UL-CL/BP in case the UL-CL/BP is controlled by a SMF different from the SMF that controls the PSA of the PDU Session.

- Discussion

In last meeting, a key issue was proposed and agreed:

How to insert an UPF supporting UL-CL/BP (IPv6 multi-homing) which is not controlled by the SMF that controls the PSA of the PDU Session. The UL-CL/BP to be inserted may be at the same or different regional / administrative areas

This contribution proposes a solution to solve the issue.

- Proposal

It is proposed to include the following text into the TR:

/*************************** Start of the first change ************************/

6.X	Solution # UL-CL/BP Insertion:

6.X.1	Overview

This solution addresses the following key issue:

- How to insert an UPF supporting UL-CL/BP (IPv6 multi-homing) which is not controlled by the SMF that controls the PSA of the PDU Session. The UL-CL/BP to be inserted may be at the same or different regional / administrative areas

6.X.2	Description of the solution

6.X.2.1	General

In this solution, the UPF that acts as UL-CL/BP is controlled by a local SMF.

The UL-CL/BP may be inserted during or after the PDU session establishment, e.g.:

- When UE moves to a location that supports to connect a local DN via UL-CL/BP; Or,

- When UE starts an application that has local deployment via the UL-CL/BP.

6.X.2.2 	UL-CL/BP in the same region as A-SMF

When the UE is within the same administrative domain as the PSA of the PDU Session, it is possible that some UPF are not controlled by the A-SMF, e.g. the UPF belonged to the enterprise and exclusively controlled by the SMF also belonged to the enterprise.

In that case when those UPF is to be selected, e.g. UL-CL/BP insertion, the Local SMF need be inserted. The architecture is as in Figure 6.x.2.2-1:

Figure 6.x.2.2-1 UL-CL/BP insertion when UE is in the same region as A-SMF

The local SMF is a network function that controls the UPF deployed close to the edge, e.g. the UPFs are deployed in an enterprise premise. The local SMF has the following function:

- UE IP address management in case of multi-homing (RA can be sent to UE via local SMF or A-SMF)

- Selection and control of UP function

- Configures traffic steering at UPF to route traffic to proper destination

- Charging data collection

The SMF that controls the PSA of the PDU session (i.e. A-SMF) determines whether a UL-CL/BP needs to be inserted and determines the DNAI the UL-CL/BP connects to. The SMF selects the local SMF that controls the UL-CL/BP based on DNAI and UE location information.

NOTE: it is assumed that Nxy interface is based on the N4 interface. However the relation between the Nxy and N4 interface is FFS. For example the enhancement on N4 interface to convey the charging data and IP address.

6.X.2.4	Procedures

6.X.2.4.1	Insertion of UL-CL/BP when it is in the same region as A-SMF

The procedure in figure 6.X.2.4.1-1 is to insert a UL-CL/BP for a PDU session when the UE is located in the same region as the SMF that controls the PSA. It is assumed that local PSA is collocated with UL-CL/BP. The solution also applies to the case where the local PSA is not collocated with UL-CL/BP.

Figure 6.X.2.4.1-1 Procedure to support UL-CL/BP insertion when UE is in the region of A-SMF

1. 	Before the procedure, a PDU session anchored in A-SMF has been established. The A-SMF may receive some triggers to insert a UL-CL/BP for the PDU session. The triggers the A-SMF receives includes:

- The UE moves to a new location based on the area of interest subscription (from AMF). Or

- Received trigger from PCF or from PSA that a new application has been detected (from PCF or PSA).

2. 	Based on the trigger, the SMF determines whether to insert UL-CL/BP. If yes, the SMF determines the DNAI that the UL-CL/BP will connect to.

If the SMF cannot find a UPF that supports the DNAI by itself, the SMF selects a Local SMF based on the DNAI and UE location information.

3. 	The A-SMF sends Nxy message to the Local SMF to request it to insert UL-CL/BP. The message includes UE location information, an indication of UL-CL or Multi-homing, AN tunnel information, PSA tunnel information, PDR to be installed in UL-CL/BP, and optionally the DNAI.

Based on the configuration the A-SMF determines the service area of this UL-CL/BP UPF. Based on that information, the A-SMF subscribe the notification when the UE leave this service area.

4. The Local SMF selects UL-CL/BP based on UE location, and optionally DNAI.

5. 	I-SMF sends N4 Session establishment request to the UL-CL/BP, including AN tunnel info, PSA tunnel info and PDR to be installed in UL-CL/BP.

6. 	Local SMF send Nxy message Ack to A-SMF, including the local UPF tunnel info (for both N3 and N9).

In case of Multi-homing, the Local SMF need allocate a local IPv6 prefix for the PDU Session. The Local SMF also includes local IPv6 prefix in this message, the A-SMF notifies the UE of the availability of the new IP prefix via IPV6 Router Advertisement.

7. 	A-SMF send N4 session update request to PSA, to update it with local UPF tunnel info (N9).

8. 	A-SMF send SM N2 request to AN via AMF to update AN with the local UPF tunnel info (N3).

6.X.2.3 	UL-CL/BP in the different region as A-SMF

Editor’s Note: It is FFS is the UL-CL/BP is in the different region as the A-SMF.

6.X.3	Impact of the solution to existing entities

SMF is enhanced to support local SMF selection function. A new local SMF is introduced to control the UL-CL/BP.

No impacts to other existing network functions are identified.

6.X.4	Evaluation of the solution

/*************************** End of the first change ************************/