3GPP TSG-SA2 Meeting #126	S2-181698

# Montreal, Canada, 26th Feb 2018 - 2nd Mar 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0101 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleaning up binding in BSF after PDU session termination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The binding created in BSF should be deleted at the end of the PDU sessions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the call flow of “Session management policy termination” to trigger the binding cleanup in BSF. After notifying AF about session termination the PCF should trigger the cleaning up binding information in BSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# * * * First change* * * *

### 4.16.6	Session Management Policy Termination

# Figure 4.16.6-1: Session Management Policy Termination

This procedure concerns both roaming and non-roaming scenarios.

In the non-roaming case the V-PCF is not involved. In the local breakout roaming case, the H-PCF is not involved. In the home routed roaming case, the V-PCF is not involved and the H-PCF interacts only with the H-SMF.

1.	The PCF may invoke the Npcf_SMPolicyControl_UpdateNotify service operation to request the release of a PDU Session.

2.	The SMF may invoke the Npcf_SMPolicyControl_Delete service operation to delete the PDU Session in the PCF. The SMF provides relevant information to the PCF.

3.	When receiving the request from step2, the PCF finds the PCC Rules that require an AF to be notified and removes PCC Rules for the PDU Session.

4.	The SMF removes all PCC Rules associated with the PDU Session.

5.	The PCF notifies the AF as explained in clause 7.3.1 steps 6-7 of TS 23.203 [24]. PCF should invoke Nbsf_Management_Deregister service operation to delete the binding created in BSF.

6.	If this is the last PDU Session for this subscriber requiring policy counter status reporting, the Final Spending Limit Report Retrieval as defined in clause 4.16.x.4 is initiated. If any existing PDU Sessions for this subscriber require policy counter status reporting, the Intermediate Spending Limit Report Retrieval as defined in clause 4.16.x.3 may be initiated to alter the list of subscribed policy counters.

7.	The PCF removes the information related to the terminated PDU Session and acknowledges to the SMF that the PCF handling of the PDU Session has terminated. This interaction is the response to the SMF request in step 2. The SMF deletes the subscription to SMF detected events for that PDU Session.

8.	The PCF unsubscribe the notification of the PDU Session related data modification from the UDR by invoking Nudr_UDM_Subscribe service if it has subscribed such notification.

* * * End of the changes* * * *