SA WG2 Meeting #126	S2-182223

26 February -02 March, 2018, Montreal, Canada

Source:	Huawei, Hisilicon

Title:	Solution: UL-CL/BP Insertion

Document for:	Discussion and Approval

Agenda Item:	6.12

Work Item / Release:	FS_ETSUN / Rel-16

Abstract of the contribution: This contribution proposes a solution to insert UL-CL/BP in case the UL-CL/BP is controlled by a SMF different from the SMF that controls the PSA of the PDU Session.

# Discussion

In last meeting, a key issue was proposed and agreed:

How to insert an UPF supporting UL-CL/BP (IPv6 multi-homing) which is not controlled by the SMF that controls the PSA of the PDU Session. The UL-CL/BP to be inserted may be at the same or different regional / administrative areas

This contribution proposes a solution to solve the issue.

# Proposal

It is proposed to include the following text into the TR:

/*************************** Start of the first change ************************/

## 6.X	Solution # UL-CL/BP Insertion:

### 6.X.1	Overview

This solution addresses the following key issue:

- How to insert an UPF supporting UL-CL/BP (IPv6 multi-homing) which is not controlled by the SMF that controls the PSA of the PDU Session. The UL-CL/BP to be inserted may be at the same or different regional / administrative areas

### 6.X.2	Description of the solution

#### 6.X.2.1	General

In this solution, the UPF that acts as UL-CL/BP is controlled by a local SMF.

The UL-CL/BP may be inserted during or after the PDU session establishment, e.g.:

- When UE moves to a location that supports to connect a local DN via UL-CL/BP; Or,

- When UE starts an application that has local deployment via the UL-CL/BP.

#### 6.X.2.2 	UL-CL/BP in the same region as A-SMF

When the UE is within the same administrative domain as the PSA of the PDU Session, the architecture is as in Figure 6.x.2.2-1:

Figure 6.x.2.2-1 UL-CL/BP insertion when UE is in the same region as A-SMF

In the Figure, the local SMF is a network function that controls the UPF deployed close to the edge, e.g. the UPFs are deployed in an enterprise premise. The local SMF has the following function:

- UE IP address management in case of multi-homing (RA can be sent to UE via local SMF or A-SMF)

- Selection and control of UP function

- Configures traffic steering at UPF to route traffic to proper destination

- Charging data collection

The SMF that controls the PSA of the PDU session (i.e. A-SMF) determines whether a UL-CL/BP needs to be inserted and determines the DNAI the UL-CL/BP connects to. The SMF selects the local SMF that controls the UL-CL/BP based on DNAI and UE location information.

#### 6.X.2.4	Procedures

##### 6.X.2.4.1	Insertion of UL-CL/BP when it is in the same region as A-SMF

The procedure in figure 6.X.2.4.1-1 is to insert a UL-CL/BP for a PDU session when the UE is located in the same region as the SMF that controls the PSA. It is assumed that local PSA is collocated with UL-CL/BP. The solution also applies to the case where the local PSA is not collocated with UL-CL/BP.

Figure 6.X.2.4.1-1 Procedure to support UL-CL/BP insertion when UE is in the region of A-SMF

1. 	Before the procedure, a PDU session anchored in A-SMF has been established. The A-SMF may receive some triggers to insert a UL-CL/BP for the PDU session. The triggers the A-SMF receives includes:

- The UE moves to a new location based on the area of interest subscription (from AMF). Or

- Received trigger from PCF or from PSA that a new application has been detected (from PCF or PSA).

2. 	Based on the trigger, the SMF determines whether to insert UL-CL/BP. If yes, the SMF determines the DNAI that the UL-CL/BP will connect to.

If the SMF cannot find a UPF that supports the DNAI by itself, the SMF selects a Local SMF based on the DNAI and UE location information.

3. 	The A-SMF sends Nxy message to the Local SMF to request it to insert UL-CL/BP. The message includes UE location information, an indication of UL-CL or Multi-homing, AN tunnel information, PSA tunnel information, PDR to be installed in UL-CL/BP, and optionally the DNAI.

4. The Local SMF selects UL-CL/BP based on UE location, and optionally DNAI.

5. 	I-SMF sends N4 Session establishment request to the UL-CL/BP, including AN tunnel info, PSA tunnel info and PDR to be installed in UL-CL/BP.

6. 	Local SMF send Nxy message Ack to A-SMF, including the local UPF tunnel info (for both N3 and N9).

In case of Multi-homing, the Local SMF need allocate a local IPv6 prefix for the PDU Session. The Local SMF also includes local IPv6 prefix in this message, the A-SMF notifies the UE of the availability of the new IP prefix via IPV6 Router Advertisement.

7. 	A-SMF send N4 session update request to PSA, to update it with local UPF tunnel info (N9).

8. 	A-SMF send SM N2 request to AN via AMF to update AN with the local UPF tunnel info (N3).

#### 6.X.2.3 	UL-CL/BP in the different region as A-SMF

Editor’s Note: It is FFS is the UL-CL/BP is in the different region as the A-SMF.

### 6.X.3	Impact of the solution to existing entities

SMF is enhanced to support local SMF selection function. A new local SMF is introduced to control the UL-CL/BP.

No impacts to other existing network functions are identified.

### 6.X.4	Evaluation of the solution

/*************************** End of the first change ************************/