SA WG2 Meeting #126 	S2-182299

Feb 26 – Mar 2, 2018, Montreal, Canada	(was S2-181714)

Source:	CATT

Title:	5GS LCS Functionality and Allocation to 5GS elements for Key Issue #1

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	FS_eLCS / Rel-16

Abstract of the contribution: This contribution proposes a solution for key issue 1.

Introduction

Highlights of this PCR:

- Based on the assumption that most existing LCS functional defined in TS23.271 will continue exists, this P-CR propose two options of allocation of LCS functional entities to 5GC elements, by using the service based architecture define in TS23.501 as base.

- In 5GC, the AMF, LMF, UDM/UDR replaces the MME, E-SMLC, HSS in EPC.

- PPR, PMD are shown as standalone entities, and Service Interface defined.

- Introduce the an new Internal LCS Consumer functional entity in 5GC, currently not clear which 5G elements need this functionality, listed here for further enhancement.

- Document Some open issues common to both option need further study: e.g.

  - Selection of an LMF, for MO-LR and MT-LR request;

  - Determination of an AMF, for MT-LR request;

  - Location Service continuity in UE mobility case: e.g. inter-AMF handover;

  - Services that each network elements provide;

Proposal

It is propose to add following solution for Key Issue #1. All text is new.

* * * Start of Change * * * *

6.X	Solution #X: 5GS LCS Functionality and Allocation to 5GS elements - Option 1 and Option 2.

6.X.1	Introduction

This solution addresses the key issue 1.

Base on analysis of LCS function in Section 6.2 of TS23.271 [6] for EPS, this solution introduce those existing LCS functions, i.e. the same functional groups and functional blocks into 5G.

Since elements of 5GC are different from that of EPS, this solution proposes a kind of allocation of LCS functional entities to 5GC elements, by following:

- In 5GC, the AMF, LMF, UDM/UDR replaces the MME, E-SMLC, HSS in EPC.

- The PPR may be standalone entities, or the PPR functionality may be integrated in H-GMLC.

- The PMD may be standalone entities, or the PMD functionality may be integrated in PPR.

- Standalone PPR and PMD entities will provide Service Interface, e.g. Nppr and Npmd.

- The Le reference point remains unchanged.

This solution uses the functionality of PPR, PMD, GLMC and LRF specified in TS23.271 [6] as base. Further enhancement will be developed and added during the study. If solutions to other key issues indentified some new functionality or enhancements, then this solution will be updated accordingly. E.g. introduce NEF for exposures purpose.

This solution need further study following issues:

- Selection of an LMF, for MO-LR and MT-LR request;

- Determination of an AMF, for MT-LR request;

- Location Service continuity in UE mobility case: e.g. inter-AMF handover;

- Services that each network elements provide;

6.X.2	Functional Description

6.X.2.1	Common Functionality

This section documents common functionality of the 5GS to support the commercial LCS.

Table 6.x shows a summary of the Functional Groups and Functional Blocks for Location services.

Table 6.x: Summary of Functional Groups and Functional Blocks for Location services

| Funct. Group | Functional component | Full name of Functional Block | Abbrev. |
| --- | --- | --- | --- |
| Loc. | Location Client | (External) Location Client Function | LCF |
| Client | Component | Internal Location Client Function | LCF -internal |
| LCS Server in PLMN | Client handling component | Location Client Control Function | LCCF |
|  |  | Location Client Authorization Function | LCAF |
|  |  | Location Client Co-ordinate Transformation Function | LCCTF |
|  |  | Location Client Zone Transformation Function | LCZTF |
|  | System handling component | Location System Control Function | LSCF |
|  |  | Location System Billing Function | LSBF |
|  |  | Location System Operations Function | LSOF |
|  |  | Location System Broadcast Function | LSBcF |
|  |  | Location System Co-ordinate Transformation Function | LSCTF |
|  |  | Location IMS – Interworking Function | LIMS-IWF |
|  | Subscriber Handling component | Location Subscriber Authorization Function | LSAF |
|  |  | Location Subscriber Translation Function | LSTF |
|  |  | Location Subscriber Privacy function | LSPF |
|  | Positioning component | Positioning Radio Control Function | PRCF |
|  |  | Positioning Calculation Function | PCF |
|  |  | Positioning Signal Measurement Function | PSMF |
|  |  | Positioning Radio Resource Management | PRRM |

In addition to those functionalities in above table, the AMF should support transport NRPPa messages defined in TS38.455 [xx] between UE and the LMF.

Note1: The RAN WG may develop new positioning component for NG-RAN. Thus, above table will be updated accordingly.

In following two sections, two options for function allocations are described, respectively.

Note2: Some function block in above table may be associated with each entity, e.g. LSOF. That does not mean each entity will provide the same service.

6.X.2.2	Option A: EPS like allocation

In this option,

- The AMF implement all LCS related functionalities in MME (see section 6.3.13 of TS23.271[6]).

- The LMF implement all LCS related functionalities in E-SMLC (see section 6.3.14 of TS23.271[6]).

Table 6.ay and figure 6.az show the distribution of LCS functional blocks to 5G network elements and general LCS Logical Architecture for 5G.

In this option, mapping LCS related procedure defined in TS23.271[6] to Service base procedure is needed.

Table 6.ay: Allocation of LCS functional entities to 5G elements

|  | UE | RAN | GMLC | AMF | LMF | UDM/UDR | PPR | PMD | Client |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location client functions |  |  |  |  |  |  |  |  |  |
| LCF | X |  |  | X |  |  |  |  | X |
| LCF Internal |  |  |  |  |  |  |  |  |  |
| Client handling functions |  |  |  |  |  |  |  |  |  |
| LCCTF |  |  | X |  |  |  |  |  |  |
| LCCF |  |  | X |  |  |  |  |  |  |
| LCAF |  |  | X |  |  |  |  |  |  |
| LCZTF |  |  | X |  |  |  |  |  |  |
| System handling functions |  |  |  |  |  |  |  |  |  |
| LSCF |  |  |  | X |  |  |  |  |  |
| LSBF |  |  | X | X |  |  |  |  |  |
| LSOF | X | X | X | X | X |  |  |  |  |
| LSBcF |  | X |  |  | X |  |  |  |  |
| LSCTF |  |  | X |  |  |  |  |  |  |
| LIMS-IWF |  |  | X (Note 1) |  |  |  |  |  |  |
| Subscriber handling functions |  |  |  |  |  |  |  |  |  |
| LSAF |  |  | X | X |  |  | X |  |  |
| LSPF |  |  | X | X |  | X | X |  |  |
| LSTF |  |  |  |  |  |  |  | X |  |
| Positioning functions |  |  |  |  |  |  |  |  |  |
| PRCF |  |  |  |  | X |  |  |  |  |
| PCF | X |  |  |  | X |  |  |  |  |
| PSMF | X | X |  |  |  |  |  |  |  |
| PRRM |  | X |  |  |  |  |  |  |  |
|  | UE | RAN | GMLC | AMF | LMF | UDM/UDR | PPR | PMD | Client |

# Figure 6.az: Generic LCS Logical Architecture for 5GC

Editors’ Note: The detail of services provided by NFs in this option is FFS.

6.X.2.3	Option B: LMF centric allocation

In this option,

- The AMF partially implement one LCS related functionalities i.e. LSOF (see section 5.4.2.3 of TS23.271 [6]).

- The LMF implement all LCS related functionalities in E-SMLC (see section 6.3.14 of TS23.271 [6]) and most of LCS related functionalities in MME.

Table 6.by and figure 6.bz show the distribution of LCS functional blocks to 5G network elements and general LCS Logical Architecture for 5G.

Compared to that of Option A, this option require LMF implement following functionality: LCF, LSCF, LSBF, LSAF and LSPF (those are originally implemented by MME), in addition to all LCS related functionalities in E-SMLC (see section 6.3.14 of TS23.271 [6]).

Table 6.ay: Allocation of LCS functional entities to 5G elements

|  | UE | RAN | GMLC | AMF | LMF | UDM/UDR | PPR | PMD | Client |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location client functions |  |  |  |  |  |  |  |  |  |
| LCF | X |  |  |  | X |  |  |  | X |
| LCF Internal |  |  |  |  |  |  |  |  |  |
| Client handling functions |  |  |  |  |  |  |  |  |  |
| LCCTF |  |  | X |  |  |  |  |  |  |
| LCCF |  |  | X |  |  |  |  |  |  |
| LCAF |  |  | X |  |  |  |  |  |  |
| LCZTF |  |  | X |  |  |  |  |  |  |
| System handling functions |  |  |  |  |  |  |  |  |  |
| LSCF |  |  |  |  | X |  |  |  |  |
| LSBF |  |  | X |  | X |  |  |  |  |
| LSOF | X | X | X | X | X |  |  |  |  |
| LSBcF |  | X |  |  | X |  |  |  |  |
| LSCTF |  |  | X |  |  |  |  |  |  |
| LIMS-IWF |  |  | X (Note 1) |  |  |  |  |  |  |
| Subscriber handling functions |  |  |  |  |  |  |  |  |  |
| LSAF |  |  | X |  | X |  | X |  |  |
| LSPF |  |  | X |  | X | X | X |  |  |
| LSTF |  |  |  |  |  |  |  | X |  |
| Positioning functions |  |  |  |  |  |  |  |  |  |
| PRCF |  |  |  |  | X |  |  |  |  |
| PCF | X |  |  |  | X |  |  |  |  |
| PSMF | X | X |  |  |  |  |  |  |  |
| PRRM |  | X |  |  |  |  |  |  |  |
|  | UE | RAN | GMLC | AMF | LMF | UDM/UDR | PPR | PMD | Client |

# Figure 6.bz: Generic LCS Logical Architecture for 5GC

Editors’ Note: The detail of services provided by NFs in this option is FFS.

6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

6.X.4	Impacts on existing entities and interfaces

Editor's note:	This clause describes impacts to existing entities and interfaces.

6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * End of Change * * * *