SA WG2 Meeting #126	S2-182314

Feb 26 – Mar 2, 2018, Montreal, Canada	(was S2-181715)

Source:	CATT

Title:	Updates to KI#12 Inter-RAT mobility support to/from NB-IoT

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	FS_5G_CIoT / Rel-16

Abstract of the contribution: This paper updates the KI#12: with more clear description of inter-RAT mobility, one open issues, etc.

1.	Discussion

In 5GS, three types of RAT, e.g. NR, NB-IOT, evolved WB-EUTRA could connect to 5GC. For IoT case, it is proposed to consider mobility between NR and E-UTRA as well.

In order to not mix up with EPC-5GC interworking related key issues, it is proposed to clarify this KI only focus on the intra 5G CN mobility scenarios.

With NR control plane work on-going within RAN WGs, how to handle NR related UE capability should also be considered especially those IoT related parameter, features developed by RAN WGs. The TS23.501 already defined how to handle UE NR radio capability, and this should be used as one baseline

During the meeting, following conclusion reached:

- The focus of this key issues is study connect mode inter-rat mobility to/from NB-IOT and NR or E-UTRA;

- The mobility between NR and E-UTRAN connected to 5GC is out of the scope of this key issues;

With above focus, one EN on GPRS/UTRAN could be deleted.

2.	Proposal

It is proposed to update KI#12 as following:

*** Start of changes ***

5.12	Key Issue 12: Inter-RAT mobility support to/from NB-IoT

5.12.1	Description

In Rel-14, inter-RAT mobility to/from NB-IoT without the need to detach/reattach was introduced with a CN only approach with three aspects:

-	Force TAU triggering UE via different TAs for different RAT types WB-E-UTRAN and NB-IoT

-	Maintaining two separate UE radio capabilities in the CN (MME) depending on whether they were received when the UE was camping on WB-E-UTRAN or NB-IoT.

-	PDN connection handling based on subscription.

In Rel-16, inter-RAT mobility to/from NB-IoT to other RAT is the focus of this key issue. Any mobility between NR and WB-EUTRA connect to 5GC is out of  tthe scope of this key issue.

5.12.2	Architectural requirements

The architectural requirements are:

-	Intra-PLMN connected mode inter-RAT mobility to and from NB-IoT should be supported in a radio, core network and battery efficient manner. The other RAT to be supported are NR, WB-E-UTRAN. The identified the inter-RAT mobility includes:

- Between NB-IoT connected to 5GC and WB-EUTRAN connected to 5GC;

- Between NB-IoT connected to 5GC and WB-EUTRAN connected to EPC; or

- Between NB-IoT connected to 5GC and WB-EUTRAN connected to 5GC.

-	The solution should enable data sent on different RATs to be correctly recorded on CDRs.

-	The HPLMN should be able to influence which DNNs/S-NSSAIs are maintained, reconnected or disconnected at inter-RAT idle mode mobility to/from NB-IoT.

5.12.3	Architectural baseline

The architecture baseline uses the EPC solution as baseline for the core network aspects.

-	It is assumed that the UE radio capabilities provided by the UE in NB-IoT are different from the UE radio capabilities provided by the UE in WB-E-UTRAN and NR.

-	Therefore two sets of capabilities are maintained by the AMF:

-	UE radio capabilities provided while UE is camping on NB-IoT are stored in AMF and provided to the RAN only when the UE is camping on NB-IoT.

-	UE radio capabilities provided while UE is camping on WB-E-UTRAN or NR are stored in AMF and provided to the RAN only when the UE is camping on WB-E-UTRAN or NR.

-	When the UE preforms inter-RAT idle mode mobility to/from NB-IoT, the SMF decides, based on policies received from UDM or based on local policy, whether to maintain, reconnect or release a PDU session based on the DNN/S-NSSAI information.

5.12.4	Open issues

Editor's Note: This clause lists the open issues for supporting key issue 12.

*** End of changes ***