3GPP TSG-SA WG2 Meeting #126	S2-182643

Montreal, Canada, 26 Feb – 03 March, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0192 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of interworking procedures without N26 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-02-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is ambiguity in the current description of these procedures in the section 5.17.2.3.1. Indeed, it is said: “Interworking procedures without N26 interface use the following two features: 1. When PDU Session are created in 5GC, the PGW-C+SMF updates its information along with DNN in the HSS+UDM. 2. The HSS+UDM provides the information about dynamically allocated PGW-C+SMF and APN/DNN information to the target CN network. To support mobility both for single and dual registration mode UEs, the following also are supported by the network: 3. When UE performs Initial Attach in EPC (with or without "Handover" indication in PDN CONNECTIVITY Request message) and indicates that it is moving from 5GC, the MME does not include "initial attach" indicator to the HSS+UDM. This results in HSS+UDM not cancelling the registration of AMF, if any. 4. When UE performs Initial Registration in 5GC and indicates that it is moving from EPC, the AMF does not include "initial attach" indicator to the HSS+UDM. This results in HSS+UDM not cancelling the registration of MME, if any. 5. When PDN connections are created in EPC, the MME stores the PGW-C+SMF and APN information in the HSS+UDM. NOTE 3: Items 4 and 5 are also supported in networks that support interworking with N26 procedures. This enables a VPLMN that does not deploy N26 interface to provide IP address continuity to roamed-in single-registration mode UEs from a HPLMN that only supports interworking with N26 procedures.” The section separates the features required for interworking procedures and for the ones required for mobility. However: - 1 and 2 are related to mobility more specifically. - 5 is related to mobility. It happens on 4G. It is similar to 1 which happens on 5G. - 3 and 4 are related to registration and interworking aspects more than to mobility aspects, but they are perquisite for mobility. In addition, we can have interworking without N26 without mobility (IP continuity) e.g. in case the UE sends TAU instead of attach. Given the following ambiguity, some rewarding is proposed in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | clarifications on the principles of interworking without N26 and the way they are fulfilled |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | misuderstanding of the principles |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR does not imply changes to the interworking principles. It just propose a rewording of the text for more clarity in the principles. The proposed changes should not have any impact on stage 3 ongoing work |  |  |  |  |  |  |  |  |

**** Start of 1st Change ****

#### 5.17.2.3	Interworking Procedures without N26 interface

##### 5.17.2.3.1	General

For interworking without the N26 interface, IP address continuity is provided to the UEs on inter-system mobility by storing and fetching PGW-C+SMF and corresponding APN/DDN information via the HSS+UDM. In such networks AMF also provide an indication that interworking without N26 is supported to UEs during initial Registration in 5GC or MME may optionally provide an indication that interworking without N26 is supported in the Attach procedure in EPC as defined in TS 23.401 [26].

NOTE 1:	The indication from MME that interworking without N26 is useful for UE supporting dual registration mode.

This indication is valid for the entire Registered PLMN and for PLMNs equivalent to the Registered PLMN. The same indication is provided to all UEs served by the same PLMN. UEs that operate in interworking without N26 may use this indication to decide whether to register early in the target system. UEs that operate in single-registration mode may use this indication as described in clause 5.17.2.3.2.

Interworking procedures without N26 interface use the following two features:

1.	When UE performs Initial Attach in EPC (with or without "Handover" indication in PDN CONNECTIVITY Request message) and indicates that it is moving from 5GC, the MME does not include "initial attach" indicator to the HSS+UDM. This results in HSS+UDM not cancelling the registration of AMF, if any.

2.	When UE performs Initial Registration in 5GC and indicates that it is moving from EPC, the AMF does not include "initial attach" indicator to the HSS+UDM. This results in HSS+UDM not cancelling the registration of MME, if any.

To support mobility both for single and dual registration mode UEs, the following also are supported by the network:

3.	When PDU Session are created in 5GC, the PGW-C+SMF updates its information along with DNN in the HSS+UDM.

4.	The HSS+UDM provides the information about dynamically allocated PGW-C+SMF and APN/DNN information to the target CN network.

5.	When PDN connections are created in EPC, the MME stores the PGW-C+SMF and APN information in the HSS+UDM.

NOTE 3:	Items 3, 4 and 5 are also supported in networks that support interworking with N26 procedures. This enables a VPLMN that does not deploy N26 interface to provide IP address continuity to roamed-in single-registration mode UEs from a HPLMN that only supports interworking with N26 procedures.

Networks that support 5GS-EPS interworking procedures without N26 interface do not need to provide the UEs with mapped target system parameters (e.g. QoS parameters, bearer IDs/QFI, PDU Session ID, etc.) of the target system when UE is in the source network.

When an AMF in such a network receives a request to allocate an EBI(s) for a QoS Flow(s) from a PGW-C+SMF, it may not provide the EBI(s).

NOTE 4:	A UE in a VPLMN that supports interworking without N26 may be provided with mapped QoS parameters from PGW-C+SMF in HPLMN for home-routed PDN connection, if the HPLMN supports interworking procedures with N26 interface.

A UE that operates in dual registration mode ignores any received mapped target system parameters (e.g. QoS parameters, bearer IDs/QFI, PDU Session ID, etc.).