SA WG2 Meeting #127bis	S2- 184856

28 May - 1 June 2018, Newport Beach, US	(revision of S2-18xxxx)

Source:	Ericsson

Title:	PCR 23.724: Local RRM Policy UE Differentiation for KI#13

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	Fs_CIOT_5G / Rel-16

Abstract of the contribution: Proposal for a solution to KI#13 introducing support for Local RRM Policy UE Differentiation

# Discussion

In context of recent RAN WG’s  work on enhancements for NB-IoT in NB_IOTenh2-Core, new mechanism enahnacing RAN to optimize the RAN resource usage based on a so called Local RRM Policy UE Differentiation Information has been introduced. It is proposed to introduce Local RRM Policy UE Differentiation Information as complement to solution #26 to address Key Issue#13.

# Proposal

It is proposed to add the solution below for Local RRM Policy UE Differentiation Information to the 5G CIoT TR 23.724.

***** First Change *****

# 6	Solutions

## 6.X	Solution #X: Local RRM Policy UE Differentiation Information in 5GS

### 6.X.1	Introduction

The proposal is to introduce support for Local RRM Policy UE Differentiation Information.

### 6.X.2	Functional Description

The Local RRM Policy Specific UE Differentiation Information is generated by the NG-RAN and contains the local RRM policy information that is specific to one or serveral different Local RRM Policy IDs. This local RRM policy may be used and maintained by any NG-RAN node with the same Local RRM Policy ID. It can be used and maintained over multiple IDLE-CONNECTED cycles.

The Local RRM Policy Specific UE Differentiation Information is provided to the MME at release of the UE associated NGAP signalling connection and stored in the AMF. When the AMF receives the Local RRM Policy Specific UE Differentiation Information, the AMF overwrites any one for that Local RRM Policy ID that was previously received, and does not change any stored information for any other Local RRM Policy IDs.

The Local RRM Policy Specific UE Differentiation Information is stored transparently in the AMF.

The Local RRM Policy Specific UE Differentiation Information for the full set of stored Local RRM Policy IDs is provided to the NG-RAN at establishment of the UE associated NGAP signalling connection.The Local RRM Policy Specific UE Differentiation Information is transferred at mobility between AMFs but not  to/from an MME.

### 6.X.3	Support of EPC interworking

Editor's Note:	This clause describes if and how EPC-5GC interworking is supported by this solution.

TBD

### 6.X.4	Procedures

Following NGAP messages and procedures using these messages will be enhanced to transfer Local RRM Policy UE Differentiation Information between relevant NFs:

- NGAP: DOWNLINK NAS TRANSPORT

- NGAP: INITIAL CONTEXT SETUP REQUEST

- NGAP: PATH SWITCH REQUEST ACKNOWLEDGE

- NGAP: HANDOVER REQUEST

- NGAP: UE CONTEXT RELEASE COMPLETE

- NGAP: HANDOVER REQUIRED

Furthermore, Local RRM Policy UE Differentiation Information is transferred between the AMFs at mobility registration procedure.

### 6.X.5	Impacts on existing entities and interfaces

The NG-RAN generates the Local RRM Policy UE Differentiation Information and supports transfer and reception of the Local RRM Policy UE Differentiation Information in the messages listed in section 6.X.4.

AMF supports reception, storage and transfer of the Local RRM Policy UE Differentiation Information.

### 6.X.6	Evaluation

Editor's Note: This clause provides an evaluation of the solution.

TBD

***** End of Changes *****