SA WG2 Meeting #127bis	S2-185178

28 May - 1 June 2018, Newport Beach, USA                         (revision of S2-183637)

Source:	Huawei, HiSilicon

Title:	Solution for Key Issue 7: NWDAF assisting Future Background Data Transfer

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes solution for Key Issue 7: NWDAF assisting Future Background Data Transfer.

1 Proposal

It is proposed solution for Key Issue 7: NWDAF assisting Future Background Data Transfer.

* * * * Start of Changes * * * *

6.X	Solution X: NWDAF assisting Future Background Data Transfer

6.X.1	Description

This solution is for Key Issue 7: NWDAF assisting Future Background Data Transfer.

6.X.1.1 Information to support Future Background Data Transfer

Editor's note: The potential and necessary information for the support of future background data transfer is FFS.

6.X.1.2 Procedure for Future Background Data Transfer

The Procedures for future background data transfer as specified in TS 23.502 [3], clause 4.16.7.2, can be updated as follows.

Figure 6.x.1.2.1-1: Updated procedure for Negotiation for future background data transfer

1-4. These steps are the same as step 1-4 in clause 4.16.7.2, TS 23.502 [3].

3A. The H-PCF may request background data transfer analytic information from the NWDAF associated with the information provided by the AF in step 1.

4A. With historical information, such as the 5GS NFs load info/capacity info, the selected background data transfer policy in use for either same or different AF(s), etc., the NWDAF offline derives background data transfer analytic information. The NWDAF could coordinate multiple H-PCFs for background data transfer requests coming from multiple AFs. The NWDAF provides the background data transfer analytic information to the H-PCF. The recommended background data transfer analytic information provided by the NWDAF to the PCF may be as follows:

- A list of recommended time window for the background data transfer;

- Optionally a list of recommended maximum aggregated bitrate for background data transfer.

5. The H-PCF determines one or more transfer policies, based on information provided by the NWDAF, information provided by the AF and other available information.

6-13. These steps are the same as step 6-13 in clause 4.16.7.2, TS 23.502 [3].

12A-13A. The H-PCF send the selected background data transfer policy to the NWDAF by invoking the Nnwdaf_BDTPolicyUpdate request service operation. Note that if the H-PCF determines only one transfer policy in step 5, the step 12A and 13A steps are performed after step 6.

6.X.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

6.X.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * End of Change * * *