3GPP TSG-SA2 Meeting #127bis	S2-185396

# May 28 - June 1, 2018, Newport Beach, California USA

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0442 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configured NSSAI update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_ph1 |  |  |  |  |  |  | Date: |  | 2018-05-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the agreement in SA2#126, the Configured NSSAI for a UE for a PLMN can only be updated when UE has registered in the PLMN: The changing of Configured NSSAI for PLMN can be caused by two different reasons: 1) UE's Subscribed S-NSSAIs are changed by including new S-NSSAI, which causes Configured NSSAI for all different PLMNs need to be updated. 2) The Network Slice deployment of a VPLMN is changed, which causes some network slices of HPLMN becoming supportable, hence the Configured NSSAI for this PLMN need to be updated. The above events may happen when: - the UE RM state is RM-DEREGISTERED. In this case, the Configured NSSAI for any PLMN on the UE cannot be updated on time. - the UE is registered in a PLMN different from the PLMNs whose Configured NSSAI need to be updated. In this case, the Configured NSSAI for those PLMNs cannot be updated on time. As the Requested NSSAI can be a subset of the Configured NSSAI for the PLMN, if the UE Configured NSSAI for a PLMN is not updated successfully on the UE, in some scenarios the network cannot be aware of the inconsistency just based on the received Requested NSSAI when UE registers. A mechanism for update Configured NSSAI of PLMN to the UE should be considered to cover all of the above cases. Some potential solutions have been discussed in S2-183344 and online in SA2# 127. In this CR, it is proposed to send the latest Configured NSSAI for the serving PLMN to the UE when UE performs Initiate Registration, or Mobility Registration from a different PLMN. Note: It seems the only usage of Configured NSSAI of VPLMN is for RAN to select an appropriate AMF during the initial registration to avoid potential AMF relocation. But this configuration on the UE causes complex update issue as highlighted above. If the group agrees to remove Configured NSSAI for VPLMN, the issue will be significantly simplified to how to update Configured NSSAI for HPLMN only. If that the author can update the contribution accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the AMF receives a registration request from the UE, the AMF will provide the UE with the Configured NSSAI for the Serving PLMN and/or the associated mapping with Configured NSSAI for the HPLMN if the registration type is “initial registration”, or if the registration type is “mobility registration update” and the old AMF is from another PLMN (based on 5G GUTI in the registration request). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Configured NSSAI in UE will not be updated for those PLMNs which UE doesn’t register to when the Subscribed S-NSSAI(s) changes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** 1st Change *****

### 5.15.4	UE NSSAI configuration and NSSAI storage aspects

#### 5.15.4.1	General

A UE can be configured by the HPLMN with Network Slice configuration information.

The Network Slice configuration information contains one or more Configured NSSAI(s). A Configured NSSAI may apply either to one PLMN or to all PLMNs that do not have a specific Configured NSSAI (e.g. this could be possible for NSSAIs containing only S-NSSAIs with standard values, see clause 5.15.2.1). There is at most one Configured NSSAI per PLMN.

The Configured NSSAI of a PLMN may include S-NSSAIs that have standard values or PLMN-specific values.

The Configured NSSAI for the Serving PLMN includes the S-NSSAI values which can be used in the Serving PLMN and may be associated with mapping of each S-NSSAI of the Configured NSSAI to one or more corresponding S-NSSAI values in the Configured NSSAI for the HPLMN.

The S-NSSAIs in the Configured NSSAI for the HPLMN, at the time when they are provided to the UE, shall match the Subscribed S-NSSAIs for the UE. When the Subscribed S-NSSAI(s) are updated (i.e. removing an old and/or adding a new) and it is applicable to the Serving PLMN the UE is registered in as described in clause 5.15.3 , the AMF may update the UE with the Configured NSSAI of the Serving PLMN and/or Allowed NSSAI and/or the associated mapping with Configured NSSAI for the HPLMN.

When providing a Requested NSSAI to the network upon registration, the UE in a given PLMN only includes and uses S-NSSAIs applying to this PLMN, possibly associated with mapping of each S-NSSAI of the Requested NSSAI to the S-NSSAIs of the Configured NSSAI for the HPLMN i.e. part of the Configured and/or Allowed NSSAIs applicable for this PLMN. Upon successful completion of a UE's Registration procedure over an Access Type, the UE obtains an Allowed NSSAI for this Access Type, which includes one or more S-NSSAIs, from the AMF, possibly associated with mapping of Allowed NSSAI to Configured NSSAI for the HPLMN. These S-NSSAIs are valid for the current Registration Area and Access Type provided by the AMF the UE has registered with and can be used simultaneously by the UE (up to the maximum number of simultaneous Network Slices or PDU Sessions).

The UE might also obtain one or more rejected S-NSSAIs with cause and validity of rejection from the AMF. An S-NSSAI may be rejected:

-	for the entire PLMN; or

-	for the current Registration Area.

While it remains RM-REGISTERED in the PLMN and regardless of the Access Type, the UE shall not re-attempt to register to an S-NSSAI rejected for the entire PLMN until this rejected S-NSSAI is deleted as specified below.

While it remains RM-REGISTERED in the PLMN, the UE shall not re-attempt to register to an S-NSSAI rejected in the current Registration Area until it moves out of the current Registration Area.

NOTE 1:	The details and more cases of S-NSSAI rejection are described in TS 24.501 [47].

The UE stores (S-)NSSAIs as follows:

-	When the UE is provisioned with a Configured NSSAI for a PLMN and/or the associated mapping of this Configured NSSAI to the Configured NSSAI for the HPLMN, the Configured NSSAI and/or if present, the associated mapping shall both be stored in the UE until a new Configured NSSAI for this PLMN and/or the associated mapping are provisioned in the UE:

-	When provisioned with a new Configured NSSAI for a PLMN and/or the new associated mapping of this Configured NSSAI to the Configured NSSAI for the HPLMN, the UE shall:

-	replace any stored (old) Configured NSSAI for this PLMN with the new Configured NSSAI for this PLMN; and

-	delete any stored associated mapping of this old Configured NSSAI for this PLMN to the Configured NSSAI for the HPLMN and, if present, store the associated mapping of this new Configured NSSAI to the Configured NSSAI for the HPLMN; and

-	delete any stored Allowed NSSAI and associated mapping as well as rejected S-NSSAI for this PLMN;

NOTE 2:	It is expected that the UE keeps storing a received Configured NSSAI for a PLMN and associated mapping to the Configured NSSAI for the HPLMN even when registering in another PLMN. However, the number of Configured NSSAI and associated mapping to be kept stored in the UE for PLMNs other than the HPLMN is up to UE implementation.

-	If received, the Allowed NSSAI for a PLMN and Access Type and any associated mapping of this Allowed NSSAI to the Configured NSSAI for the HPLMN shall be stored in the UE. The UE should store this Allowed NSSAI and any associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN also when the UE is turned off;

NOTE 3:	Whether the UE stores the Allowed NSSAI and any associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN also when the UE is turned off is left to UE implementation.

-	When a new Allowed NSSAI for a PLMN and any associated mapping of the Allowed NSSAI to the Configured NSSAI of the HPLMN are received over an Access Type, the UE shall:

-	replace any stored (old) Allowed NSSAI and any associated mapping for these PLMN and Access Type with this new Allowed NSSAI; and

-	delete any stored associated mapping of this old Allowed NSSAI for this PLMN to the Configured NSSAI for the HPLMN and, if present, store the associated mapping of this new Allowed NSSAI to the Configured NSSAI for the HPLMN;

-	If received, an S-NSSAI rejected for the entire PLMN shall be stored in the UE while RM-REGISTERED in this PLMN regardless of the Access Type or until it is deleted.

-	If received, an S-NSSAI rejected for the current Registration Area shall be stored in the UE while RM-REGISTERED until the UE moves out of the current Registration Area or until it is deleted.

NOTE 4:	The storage aspects of rejected S-NSSAIs are described in TS 24.501 [47].

One or more S-NSSAIs in an Allowed NSSAI provided to the UE can have values, which are not part of the UE's slice configuration information for the Serving PLMN. In this case, the Allowed NSSAI is associated with mapping information regarding how each S-NSSAI of the Allowed NSSAI corresponds to the S-NSSAI(s) of the Configured NSSAI for the HPLMN. This mapping information is sufficient to allow the UE to associate for a given application the S-NSSAI as per NSSP of the URSP rules, as defined in clause 6.6.2 of TS 23.503 [45], with the corresponding S-NSSAI from the Allowed NSSAI.

#### 5.15.4.2	Update of UE Network Slice configuration

At any time, the AMF may provide the UE with a new Configured NSSAI for the Serving PLMN, associated with mapping of the Configured NSSAI to the Configured NSSAI for the HPLMN as specified in clause 5.15.4.1.The Configured NSSAI for the Serving PLMN and the mapping information is either determined in the AMF (if based on configuration, the AMF is allowed to determine the Network Slice configuration for the whole PLMN) or by the NSSF. The AMF provides the new Configured NSSAI as specified in TS 23.502 [3], clause 4.2.4 UE Configuration Update procedure.

When the UE performs initiate registration, or mobility registration from another PLMN (i.e. the old AMF indicated by the 5G GUTI in the Registration Request is from a different PLMN), the AMF shall provide the UE with the Configured NSSAI for the Serving PLMN and/or the associated mapping of the Configured NSSAI for the Serving PLMN to the Configured NSSAI for the HPLMN.

If the HPLMN performs the configuration update of a UE registered in the HPLMN (e.g. due to a change in the Subscribed S-NSSAI(s)), this results in updates to the Configured NSSAI for the HPLMN. Updates to the Allowed NSSAI and/or, if present, to the associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN are also possible if the configuration update affects S-NSSAI(s) in the current Allowed NSSAI.

If the VPLMN performs the configuration update of a UE registered in the VPLMN (e.g. due to a change in the Subscribed S-NSSAI(s)), this results in updates to the Configured NSSAI for the Serving PLMN and/or to the associated mapping of the Configured NSSAI for the Serving PLMN to the Configured NSSAI for the HPLMN. Updates to the Allowed NSSAI and/or to the associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN are also possible if the configuration update affects S-NSSAI(s) in the current Allowed NSSAI.

A UE for which the Configured NSSAI for the Serving PLMN has been updated and has deleted the stored Allowed NSSAI as described in clause 5.15.4.1 shall initiate a Registration procedure to receive a new valid Allowed NSSAI (see clause 5.15.5.2.1).

The update of URSP rules (which include the NSSP), if necessary, is described in TS 23.503 [45].

***** End of Change *****