3GPP TSG-SA2 Meeting #127bis	S2-185545

# Newport Beach, United States, 28th May 2018 - 1st Jun 2018		revision of S2-185207

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0089 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS flow binding for URLLC services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For a delay critical QoS flow, the PDB and PER requirements should be satisfied for the traffic not exceeding the MDBV. And as the delay critical QoS flow is GBR resource type, the PDB and PER requirements should be satisfied for the traffic not exceeding the GFBR. Althgh the GFBR is a granteed bit rate while MDBV is a traffic volume, and the time scales for GFBR and MDBV are different, there’s relationship between the two parameters. For example, if the GFBR of a QoS flow is a very large value, the traffic of the QoS flow may always exceed the MDBV, in this case although the bit rate of the traffic is under GFBR, the PDB and PER requirements can’t be satisfied due to the MDBV limitation. For the above reason, the SMF should consider the UL/DL-guaranteed bitrate of the PCC rule for the QoS flow binding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the PCC rule contains a delay critical GBR 5QI value, the SMF considers the UL/DL-guaranteed bitrate of the PCC rule for the QoS flow binding. The SMF may bind PCC Rules with the same binding parameters to different QoS Flows to ensure the GFBR of each QoS Flow doesn’t conflict with the MDBV limit, so that it’s possible for the traffic not exceeding GFBR can meet the MDBV limit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the GBR of the PCC rule is not considered for the QoS flow binding, the GFBR of the QoS flow may be a large vale, the traffic of the QoS flow may always exceed the MDBV, so the PDB and PER requirements can’t be satisfied although it’s a GBR QoS flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************************************Start of 1st change*****************************************

6.1.3.2.4	QoS Flow binding

QoS Flow binding is the association of a PCC rule to a QoS Flow within a PDU Session. The binding is performed using the following binding parameters:

-	5QI;

-	ARP;

-	QNC (if available in the PCC rule);

-	Priority Level (if available in the PCC rule);

-	Averaging Window (if available in the PCC rule);

-	Maximum Data Burst Volume (if available in the PCC rule).

When the PCF provisions a PCC Rule, the SMF shall evaluate whether a QoS Flow with QoS parameters identical to the binding parameters exists unless the PCF requests to bind the PCC rule to the QoS Flow associated with the default QoS rule. If no such QoS Flow exists, the SMF derives the QoS parameters, using the parameters in the PCC Rule, for a new QoS Flow, binds the PCC Rule to the QoS Flow and then proceeds as described TS 23.501 [2] sub-clause 5.7. If a QoS Flow with QoS parameters identical to the binding parameters exists, the SMF updates the QoS Flow, so that the new PCC Rule is bound to this QoS Flow.

NOTE:	For PCC rules containing a delay critical GBR 5QI value, the SMF can bind PCC Rules with the same binding parameters to different QoS Flows to ensure that the GFBR of the QoS Flow can be achieved with the Maximum Data Burst Volume of the 5QI.

When the PCF request that a PCC rule is bound to the QoS Flow associated with the default QoS rule by including the Bind to QoS Flow associated with the default QoS rule Indication in a dynamic PCC rule, the SMF shall bind the dynamic PCC rule to the QoS Flow associated with the default QoS rule and keep the binding as long as this indication remains set. When the PCF removes the association of a PCC rule to the QoS Flow associated with the default QoS rule, a new binding may need to be created between this PCC rule and the QoS Flow as described above.

The binding is created between a PCC Rule and a QoS Flow. The association shall cause the downlink part of the service data flow to be directed to the QoS Flow in the association. In the UE, the QoS rule associated with the QoS Flow instructs the UE to direct the uplink part of the service data flow to the QoS Flow in the association.

Whenever the authorized QoS of a PCC rule changes, the existing bindings shall be re-evaluated. The re-evaluation may, for a service data flow, require a new binding with another QoS Flow.

NOTE:	A QoS change of the default 5QI/ARP values doesn't cause the QoS Flow rebinding for PCC rules previously bound to the QoS Flow associated with the default QoS rule set, with the Bind to QoS Flow associated with the default QoS rule Indication set.

When the PCF removes a PCC Rule, the SMF shall remove the association of the PCC Rule to the QoS Flow.

The SMF shall report to the PCF that the PCC Rules bound to a QoS Flow are removed when the corresponding QoS Flow is removed.

**************************************End of changes*******************************************