SA WG2 Meeting #127-bis	S2-186276

Newport Beach, U.S. , May 28 – Jun. 1, 2018	(updated revision of S2-185763)

Source:	Telecom Italia, ZTE

Title:	Definition and assumptions

Document for:	Approval

Agenda Item:	6.21

Work Item / Release:	FS_ENS/Rel-16

Abstract of the contribution: This contribution proposes the definition of Mutually Exclusive Access to Network Slices and some architectural assumptions.

# Proposal

It is proposed to capture in the TR 23.740 the definition of Mutually Exclusive Access to Network Slices and some architectural assumptions.

First change

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Mutually Exclusive Access to Network Slices: The access to Network Slices is considered to be mutually exclusive for a UE when their respective S-NSSAIs are both present in the UE's subscription and the UE is prevented from accessing both S-NSSAIs simultaneously.

Next change

# 4	Architectural Assumptions and Requirements

For Mutually Exclusive Access to Network Slices the following assumptions apply:

-	Support of the Mutually Exclusive Access to Network Slices in a PLMN shall not impact Rel-15 5G UEs’ behaviour.

-	The network operator shall be able to ensure that the UE is prevented to access Network Slices which are mutually exclusive for that UE.

End of changes