SA WG2 Meeting #127-bis	 S2-184824

28th May – 1st June 2018, Newport Beach, USA

Source:	Ericsson

Title:	Solution to KI#7: NWDAF assisting Future Background Data Transfer

Document for:	Discussion/Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA/Rel-16

Abstract of the contribution: This document proposes a solution to key issue#7 in 23.791 v 0.3.0.

# Discussion

This contribution proposes a solution to key issue #7 to study:

-	What types of analytics information for Future Background Data Transfer could be provided by NWDAF for the PCF to determine the policy of future background data transfer;

Note that “What information does NWDAF need as input to determine such analytics information” is not described in this solution.

# 2	Proposal

This solution follows the proposal in solution #1, described in S2-184822. And then in particular:

Proposal 1: Define new EventIDs to allow the PCF to subscribe/unsubscribe/be notified or request analytics for BDT.

Proposal 2: NWDAF provides Traffic patterns and NW performance patterns to PCF A request for one of these patterns is identified by a new EventId.

Proposal 3: The NWDAF notifies the PCF on analytics to enhance existing NF functionality for Background Data Transfer.

* * * First Change * * *

## 6.x	Solution x: Enhancing Future Background Data Transfer with Analytics

### 6.x.1	Description

This is a solution to Key Issue#7: NWDAF assisting Future Background Data Transfer. It follows the solution X for key issue#1 on how NWDAF provides analytics to a NF, in this case PCF. Note that “What information does NWDAF need as input to determine such analytics information” is not described in this solution yet.

#### 6.x.2	Procedure for future BDT enhanced with Analytics

Prerequisite: The PCF subscribes to or periodically request for NW performance analytics for an ASP(s) or for specific UE(s) that has a subscription with each ASP. The PCF also subscribes or periodically request for to Mobility Patterns(s) analytics for those UEs that has a subscription with each ASP.

Table 4. EventId to provide analytics to enhance BDT functionality

| Event ID | Event Filter | Description (examples of patterns) |
| --- | --- | --- |
| UE Behavior parameters Expected Moving Trajectory | List of UE identities, Time, Date | UE´s expected geographical movement (described in 23.502 clause 14.5.6.3). |
| NW performance Pattern Load in Network | NW area, Time, Date | Estimated load in a network area at a given time and date. |

Figure 6.1.1.1-1: Procedure for future BDT enhanced with Analytics

The existing procedure listed in 4.16.7.2 in TS 23.502 applies, except steps 5 that needs to be updated to explain that a Policy decision PCF takes analytics into account to generate a transfer policy. The analytics information is either subscribed to or periodically requested by the PCF for an ASP or for a number of UE and therefore have it available at the time the ASP requests for a transfer policy.

### 6.1.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

NWDAF: New EventId to provide NW performance patterns, such as estimated load per NW area at a given time and Mobility patterns.

PCF. Use the analytics information on NW performance pattern and Mobility patterns as input for a policy decision that result on transfer policy determination.

### 6.1.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * End of Changes * * *