3GPP TSG-SA2 Meeting #127-bis	S2-184827

# Newport Beach, USA, 28th May – 1st June 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR |  | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on multiple PDU Sessions of the same attributes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1, It is not clear whehter an UE can have PDU sessions of the same attributes (DNN, S-NSSAI, SSC Mode, etc.) via 3GPP access and Non-3GPP access respectively; 2, It is not clear whether an UE can have multiple PDU sessions of the same attributes on the same access type. Though an UE may not need to initiate a PDU session establishment that has the same attributes as one existing PDU session, the UE may end up having such multiple PDU sessions when the following happens: a), a PDU session is handed-over from Non-3GPP access to 3GPP access. That PDU session may have the same attributes as one existing PDU on the 3GPP access; b), NW triggered Service Request via Paging or Notification for Non-3GPP access may switch over a PDU session from Non-3GP access to 3GPP access. That PDU session may have the same attributes as one existing PDU on the 3GPP access |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Clarify that a UE may have multiple PDU sessions of the same attributes on 3GPP access and Non-3GPP access; 2, Clarify that a UE may have multiple PDU sessions of the same attributes on the same access type; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is ambiguity on wether a UE can support multiple PDU sessions of the same attributes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes ****

## 5.6	Session Management

### 5.6.1	Overview

The 5GC supports a PDU Connectivity Service i.e. a service that provides exchange of PDUs between a UE and a data network identified by a DNN. The PDU Connectivity Service is supported via PDU Sessions that are established upon request from the UE.

Subscription Information may include multiple DNNs and may contain a Default DNN. The UE is assigned to a default DNN if it does not provide a valid DNN in a PDU Session Establishment Request message sent to the network.

Each PDU Session supports a single PDU Session type i.e. supports the exchange of a single type of PDU requested by the UE at the establishment of the PDU Session. The following PDU Session types are defined: IPv4, IPv6, IPv4v6, Ethernet, Unstructured.

PDU Sessions are established (upon UE request), modified (upon UE and 5GC request) and released (upon UE and 5GC request) using NAS SM signalling exchanged over N1 between the UE and the SMF. Upon request from an Application Server, the 5GC is able to trigger a specific application in the UE. When receiving that trigger message, the UE shall pass it to the identified application in the UE. The identified application in the UE may establish a PDU Session to a specific DNN, see clause 4.4.5.

SMF may support PDU Sessions for LADN where the access to a DN is only available in a specific LADN service area. This is further defined in clause 5.6.5.

The SMF is responsible of checking whether the UE requests are compliant with the user subscription. For this purpose, it retrieves and requests to receive update notifications on SMF level subscription data from the UDM. Such data may indicate per DNN and, if applicable, per S-NSSAI:

-	The allowed PDU Session Types and the default PDU Session Type.

-	The allowed SSC modes and the default SSC mode.

-	QoS Information (refer to clause 5.7):  the subscribed Session-AMBR, Default 5QI and Default ARP.

-	The static IP address/prefix.

-	The subscribed User Plane Security Policy.

-	the Charging Characteristics to be associated with the PDU Session. Whether this information is provided by the UDM to a SMF in another PLMN (for PDU Sessions in LBO mode) is defined by operator policies in the UDM/UDR.

NOTE 1:	The content of the Charging Characteristics as well as the usage of the Charging Characteristics by the SMF are defined in TS 32.240 [41].

An UE that is registered over multiple accesses chooses over which access to establish a PDU Session. As defined in TS 23.503 [45], the HPLMN may send policies to the UE to guide the UE selection of the access over which to establish a PDU Session.

NOTE 2:	In this Release of the specification, at any given time, a PDU Session is routed over only a single access network.

An UE may request to move a PDU Session between 3GPP and Non 3GPP accesses. The decision to move PDU Sessions between 3GPP access and Non 3GPP access is made on a per PDU Session basis, i.e. the UE may, at a given time, have some PDU Sessions using 3GPP access while other PDU Sessions are using Non 3GPP access.

In a PDU Session Establishment Request message sent to the network, the UE shall provide a PDU Session Identifier. PDU Session ID is unique per UE and is the identifier used to uniquely identify one of an UE's PDU Sessions. PDU Session ID shall be stored in the UDM to support handover between 3GPP and non-3GPP access when different PLMNs are used for the two accesses. The UE may also provide:

-	A PDU Session Type.

-	S-NSSAI.

-	The DNN (Data Network Name).

-	The SSC mode (Service and Session Continuity mode defined in clause 5.6.9.2).

Table 5.6.1-1: Attributes of a PDU Session

| PDU Session attribute | May be modified later during the lifetime of the PDU Session | Notes |
| --- | --- | --- |
| S-NSSAI | No | (Note 2) |
| DNN (Data Network Name) | No | (Note 1)(Note 2) |
| PDU Session Type | No | (Note 1) |
| SSC mode | No | (Note 2) The semantics of Service and Session Continuity mode is defined in clause 5.6.9.2 |
| PDU Session Id | No |  |
| User Plane Security Enforcement information | No | (Note 3) |
| NOTE 1: If it is not provided by the UE, the network determines the parameter based on default information received in user subscription. Subscription to different DNN(s) and S-NSSAI(s) may correspond to different default SSC modes and different default PDU Session Types NOTE 2: S-NSSAI and DNN are used by AMF to select a SMF to handle a new session. Refer to clause 6.3.2. NOTE 3: User Plane Security Enforcement information is defined in clause 5.10.3. |  |  |

Subscription Information may include a wildcard DNN per subscribed S-NSSAI: when a wildcard DNN is associated with a subscribed S-NSSAI, the subscription allows, for this S-NSSAI, the UE to establish a PDU Session using any DNN value.

NOTE 3:	The SMF is made aware whether the DNN of a PDU Session being established corresponds to an explicitly subscribed DNN or corresponds to usage of a wildcard subscription. Thus the SMF can reject a PDU Session establishment if the DNN of the PDU Session is not part of explicitly subscribed DNN(s) and local policies in the SMF require UE to have a subscription to this DNN.

An UE may establish multiple PDU Sessions, to the same data network or to different data networks, via 3GPP and via and Non-3GPP access networks at the same time. An UE may have a PDU Session on the 3GPP access that has the same attributes (S-NSSAI, DNN, PDU Session Type and SSC mode, except PDU Session ID) as a PDU Session on the Non-3GPP access. An UE may maintain multiple PDU sessions of the same attributes (S-NSSAI, DNN, PDU Session Type and SSC mode, except PDU Session ID) on the same Access Type.

An UE may establish multiple PDU Sessions to the same Data Network and served by different UPF terminating N6.

A UE with multiple established PDU Sessions may be served by different SMF.

The SMF shall be registered and deregistered on a per PDU Session granularity in the UDM.

The user plane paths of different PDU Sessions (to the same or to different DNN) belonging to the same UE may be completely disjoint between the AN and the UPF interfacing with the DN.

NOTE 4:	User Plane resources for PDU Sessions of a UE, except for regulatory prioritized service like Emergency Services and MPS, can be deactivated by the SMF if the UE is only reachable for regulatory prioritized services.

NOTE 5:	The handling if the UE goes out of the SMF service area is not specified in this Release of the specification.

**** End of Changes ****